# Idaho

# Risk Evaluation Manual for Petroleum Releases

Idaho Department of Environmental Quality 1410 North Hilton

Boise, Idaho 83706

This page intentionally left blank for correct double-sided printing.

##### Table of Contents

Introduction	1

Organization of This Document	1

Section 1.	Overview of the Risk Evaluation Process	1

  - Steps in the Risk Evaluation Process	1

    - Site Discovery	1

    - Determination of Imminent Threat	3

    - Initial Site Characterization and Evaluation	4

    - Comparison with Screening Levels	4

    - Development and Validation of a Conceptual Site Model	4

    - Identify and Collect Data for Risk Evaluation	5

    - Site Specific Risk Evaluation	5

    - Comparison with Acceptable Target Risk Levels and Decision Making	5

    - Development and Implementation of a Corrective Action Plan	6

    - Evaluation of Progress Toward Cleanup Criteria and Modification of the

Corrective Action Plan if Necessary	6

    - Site Closure with a Corrective Action Plan	6

    - Certification by a Registered Professional	6

Section 2.	Site Discovery and Initial Steps	7

  - Determining Imminent Threats	7

  - Initial Site Characterization	7

    - Site Characterization Introduction	7

    - Site Description and Adjacent Land Use	7

    - Soil Source Characterization	8

    - Groundwater Source Characterization	8

    - Documentation of Initial Site Characterization	8

Section 3.	Screening Level Evaluation	9

  - Screening Levels	9

  - Comparison of Site Concentrations with Screening Levels	11

  - Screening Level Evaluation Report	11

Section 4.	Site-Specific Risk Evaluation	12

  - Conceptual Site Model	12

    - Site Characteristics	12

    - Contaminant Source Areas	14

    - Pathways and Transport Mechanisms	15

    - Land Use and Receptors	17

  - Data Requirements and Evaluation	20

    - Data Collection Planning	20

    - Quality Assurance/Quality Control Considerations	21

    - Documentation of the Data Collected	22

    - Data Collection Methods	22

    - Vadose Zone Soil Characteristics	23

    - Saturated Zone Characteristics	25

    - Indicators of Biodegradation	26

    - Distribution of Petroleum Chemicals of Interest	27

  - Exposure Assessment	31

    - Physical Setting	31

    - Potentially Exposed Populations/Receptors	33

    - Exposure Pathways	33

    - Exposure Point Concentrations	34

    - Fate and Transport and Exposure Models	37

    - Estimation of Chemical Intake	38

  - Toxicity Assessment	39

    - Chemical-Specific Toxicological Factors	39

    - Carcinogens with a Mutagenic Mode of Action	41

  - Risk Characterization	42

  - Risk-Based Target Levels	43

    - Developing Remediation Standards for Groundwater and Surface Water Protection 44

    - Determination of Applicable Risk Based Levels	47

  - Deliverables	48

    - Work Plan	48

    - Implementing the Work Plan	50

    - Risk Evaluation Report	50

Section 5.	Developing and Implementing Corrective Action Plans	50

  - Contents of a Corrective Action Plan	50

  - Implementing a Corrective Action Plan	51

Section 6.	Activity and Use Limitations	53

  - Environmental Covenants	53

  - Specific Application of Activity and Use Limitations at Petroleum Release Sites	54

  - Use of Activity and Use Limitations at Impacted Neighboring Properties	54

  - Non-Remedial Land Use Restrictions	54

Section 7.	References	55

  - References Cited	55

  - Additional Resources for Data Collection	57

  - Additional Resources for Risk Evaluation	57

Tables

Table 1. Comparison of Risk Evaluation Options	3

Table 2. Screening Level Concentrations for Soil, Groundwater, and Soil Vapor	10

Table 3. Example Table Describing Derivation of Exposure Point Concentrations	35

##### Figures

Figure 1. Idaho Petroleum Release Risk Evaluation Process Flowchart	2

Figure 2. Sample Graphical Display of a Conceptual Site Model	32

Figure 3. Schematic of Leachate Migration from the Soil Source to the Stream	46

##### Appendices

Appendix A: References to Technical Documents

:

Appendix B: Application of Natural Attenuation

Appendix C: Estimation of Exposure Point Concentrations in Soil and Ground Water Appendix D: Sample Table of Contents for a Risk Evaluation Report

Appendix E: Practical Quantitation Limits

Acronyms and Abbreviations

| AST | Above Ground Storage Tank |
| --- | --- |
| ASTM | American Society for Testing and Materials |
| AUL | Activity and Use Limitation |
| CAP | Corrective Action Plan |
| COI | Chemical(s) of Interest |
| DEQ | State of Idaho, Department of Environmental Quality |
| EPA EPC | U.S. Environmental Protection Agency Exposure Point Concentration |
| HI | Hazard Index |
| HQ THQ | Hazard Quotient Target Hazard Quotient |
| IDAPA | Idaho Administrative Procedures Act |
| IDWR | Idaho Department of Water Resources |

| LNAPL | Light Non-Aqueous Phase Liquid |
| --- | --- |
| MCL | Maximum Contaminant Level |
| NA | Natural Attenuation |
| NAPL | Non-Aqueous Phase Liquid |
| POC | Point of Compliance |
| POE | Point of Exposure |
| PST | Petroleum Storage Tank |
| QA/QC | Quality Assurance/Quality Control |
| QAPP | Quality Assurance Project Plan |
| RCRA | Resource Conservation and Recovery Act |
| REM | Risk Evaluation Manual |
| RE | Risk Evaluation |
| ROE | Route(s) of Exposure |
| RSL | EPA Regional Screening Level |
| CSM | Conceptual Site Model |
| STATECOMM | State Communications Center |

| UCL | Upper Confidence Limit |
| --- | --- |
| UECA | Uniform Environmental Covenants Act |
| USACE | United States Army Corps of Engineers |

# Introduction

In July 2025, the Idaho Legislature approved the Department of Environmental Quality (DEQ) Rules IDAPA 58.01.24, Rules for  Petroleum Release Corrective Action (the Rule). This rule requires that substantive revisions to the manual be done so through public participation. This document represents the final version agreed upon through the public participation process. The manual was developed to supplement the Rule in assisting users through the risk evaluation process. Although the manual is guidance and is not enforceable, it does expand upon requirements in the Rule that must be followed. Therefore, throughout the manual there will be references to “must” whereby those actions are specified in the rule and “should” or “recommended” whereby those actions are optional. Lastly, the manual does not relieve the user from complying with other applicable state or federal laws such as the Water Quality Standards (IDAPA 58.01.02), Ground Water Quality Rule (58.01.11), or Idaho Rules for Public Drinking Water Systems (IDAPA 58.01.08).

#### Organization and References

The document begins with a general description of the steps in the risk evaluation (RE) process, which is then followed by detailed implementation information for each step. DEQ follows the Environmental Protection Agency’s (EPA) risk evaluation process and online risk assessment application. EPA’s Risk Assessment website will provide the user with numerous documents, a user’s guide, tables of applicable screening levels, and access to the online site-specific Regional Screening Level (RSL) Calculator (https://www.epa.gov/risk). These include, but are not limited to:

- EPA’s Regional Screening Levels (RSLs) User’s Guide  (https://www.epa.gov/risk/regional-screening-levels-rsls-users-guide)

- EPA’s Vapor Intrusion Screening Level (VISL) Calculator User’s Guide (https://www.epa.gov/vaporintrusion/visl-users-guide)

- Risk Assessment Guidance for Superfund, Volume I: Human Health Evaluation Manual (Part A) – Interim Final

- Risk Assessment Guidance for Superfund, Volume I: Human Health Evaluation Manual (Part C, Risk Evaluation of Remedial Alternatives) – Interim

- Guidance for Data Usability in Risk Assessment (Part A) – Final

- OSWER Technical Guide for Assessing and Mitigating the Vapor Intrusion Pathway from Subsurface Vapor Sources to Indoor Air

- EPA Technical Guide for Addressing Petroleum Vapor Intrusion at Leaking Underground Storage Tank Sites.

Information in the body of this manual is supported by 5 appendices.

Where references to technical documents are made, the most current version of the document should be used.

# Section 1. Overview of the Risk Evaluation Process

#### Steps in the Risk Evaluation Process

The overall RE process for a site where a petroleum release is discovered and reported to DEQ is illustrated in Figure 1. The process consists of multiple steps, each of which is briefly described in the following pages. In some cases, when adequate data are available concerning a release, the owner or operator (as defined in IDAPA 58.01.24.010) of the release site may skip the screening level evaluation and proceed directly to the site-specific RE, a more detailed level of evaluation, without formally completing the intermediate steps. Additionally, in cases where corrective action is known to be required, the owner/operator may proceed with corrective action as long as they notify DEQ, comply with any conditions imposed by DEQ, and incorporate the corrective action and subsequent risk evaluations into a corrective action plan (CAP) submitted for DEQ’s approval (IDAPA 58.01.4.61.06d).

###### Site Discovery

The RE process begins with the discovery of a petroleum release site. A petroleum release site may be discovered and reported to DEQ under a variety of circumstances. These include, but are not limited to, citizen complaints, investigations conducted as a part of real estate transactions, environmental impacts observed in surface water bodies, and notification of accidents and spills. It is the owner/operator’s responsibility to ensure that DEQ is notified (IDAPA 58.01.24.60).

Risk Evaluation Manual for Petroleum Releases—August 2018

Figure 1. Idaho Petroleum Release Risk Evaluation Process Flowchart. This process allows for moving directly to a detailed risk evaluation without first completing a general screening level evaluation.

###### Determination of Imminent Threat

Upon site discovery, the owner/operator must carefully evaluate the available information to determine whether the site poses any imminent threat to human health or safety, or to the environment. Threats include, but are not limited to, impacts to water wells; vapors or odors in residential and commercial structures; concentrations approaching explosive levels; impacts to a surface waterbody; and impacts to human health and the environment. If any imminent threats are identified, the owner/operator must call 911, if applicable; take immediate steps to abate the threat; and notify DEQ or Idaho State Communications Center (STATECOMM) immediately (see Section 2).

Documentation of abatement activities and confirmation that imminent threats have been removed should be provided to DEQ. Further details of this step are provided in Section 2.

###### Initial Site Characterization and Evaluation

Upon completion of the emergency response action, if any, it is necessary to perform an initial site characterization and evaluation (ISCE). In some cases, when adequate data are available concerning a release, the owner/operator may proceed directly to the site-specific RE. Additionally, in cases where corrective action is known to be required, the owner/operator may proceed to corrective action as long as they notify DEQ, comply with any conditions imposed by DEQ, and incorporate the corrective action and subsequent risk evaluations into a CAP submitted for DEQ’s approval (IDAPA 58.01.24.61.06d). As a part of the ISCE, media-specific data (for soil, soil vapor, and groundwater media) must be collected to characterize the source. This step is intended to identify the maximum contaminant concentrations for the identified petroleum products in soil and groundwater. Since a site may be a candidate for closure after the completion of the screening level evaluation (see Section 3), it is very important that the data collected at this time identify the maximum media-specific concentrations. Further details of this step are provided in Section 2.

###### Comparison with Screening Levels

This step involves the comparison of the maximum media-specific concentration of each contaminant, identified in the ISCE, with EPA’s generic regional screening levelsThere is a screening level for each contaminant in each media (see section 3.1 for more details on screening levels). The screening level is the lowest of the media-specific (soil, soil vapor, and groundwater) risk-based concentrations for the applicable exposure pathways. Screening levels are considered criteria that, if not exceeded (that is, maximum contaminant concentrations are below the screening levels), will allow unrestricted (residential) use of the property. Since exposure to these low concentrations (below the screening levels) of contaminants does not pose a threat to human health, if the owner/operator meets these criteria, they will not be required to evaluate site-specific exposure pathways and develop a conceptual site model (CSM), and no land use restrictions will be needed on the property. Table 1 identifies the appropriate pathways to consider for each COI.

When DEQ and the owner/operator agree that a petroleum release has been adequately characterized, DEQ will not require any further action of the owner/operator related to the release if the maximum contaminant concentrations in soil, soil vapor, surface water, and groundwater do not exceed the screening levels, subject to other applicable requirements, if any, as referenced in IDAPA 58.01.24.001.02. If any of the maximum soil, groundwater, or soil vapor concentrations exceed the screening levels, the owner/operator may either:

      - adopt the screening levels as cleanup criteria and develop a CAP to achieve these levels, or

      - perform a site-specific RE. Further details of this step are provided in Sections 3 and 4.

###### Development and Validation of a Conceptual Site Model

If the screening levels are exceeded, and the owner/operator chooses to conduct a more detailed (site-specific) RE, it is first necessary to develop and validate a CSM. A CSM provides the framework for the overall management of the site and should help guide data collection and risk management activities while the RE is being performed. The key elements of the CSM are:

      - release scenario(s),

      - source description,

      - petroleum chemicals of interest (COI),

      - an exposure model that incorporates the appropriate receptors,

      - exposure pathways and routes of exposure (ROE),

      - the site stratigraphy and hydrogeology, and

      - the spatial and temporal distribution of the petroleum COI.

An important part of this step is to validate the CSM based on site-specific data. Further details of this step are provided in section 4.1.

###### Identify and Collect Data for Risk Evaluation

Depending on the site-specific conditions, the RE may require the collection of additional site- specific data. To prepare for conducting a RE, the owner/operator must review the CSM, identify data gaps, and collect additional data if necessary. This data will be used to calculate estimates of site-specific exposure point concentrations (see section 4.3.4 and Appendix C) and an estimate of the site-specific cumulative risk (see section 4.5) and, potentially, to develop remediation standards using EPA’s RSL and VISL calculators. Further details of this step are provided in section 4.2.

###### Site Specific Risk Evaluation

Conducting the RE requires developing site-specific estimates of exposure point concentrations for soil and groundwater, estimating site-specific cumulative risk, comparing these with acceptable target risk levels and, if necessary, developing media-specific  remediation standards. Remediation standards are target concentrations developed using EPA’s RSL and VISL calculatorswith a combination of conservative default and site-specific parameters. These concentrations depend on the receptor, media, pathway, and ROE and are developed for each petroleum COI and each media identified in the CSM. This step is discussed in section 4.6.

###### Comparison with Acceptable Target Risk Levels and Decision Making

Once the site-specific cumulative risk has been estimated, it is compared to acceptable target risk levels, as specified in the rule. If acceptable target risk levels are not exceeded, the site is eligible for closure, subject to other applicable requirements, if any, as referenced in IDAPA 58.01.24.001.02. If the site-specific cumulative risk is at or above the acceptable target risk levels, two options are available:

      - Refine the CSM and RE. It is possible that the site-specific cumulative risk is lower than initially estimated and may not pose an unacceptable risk, but this can only be determined with a more refined CSM and RE. This may involve the collection of additional data or the use of other models. Before collecting these additional data, it is recommended to develop a work plan that outlines additional data needs and the overall approach for the refined evaluation. The plan should be submitted to DEQ for review and approval before proceeding with data collection and the evaluation becausethe suggestions from DEQ may help with completing the required work more efficiently and eliminate the need for re-sampling (IDAPA 58.01.24.03.c.i). Details of this step, including the basic contents of a work plan, are included in section 4.7.

      - Calculate and adopt media-specific remediation standards for each petroleum COI and develop and implement a CAP to achieve these concentrations.

Upon completion of the RE, the owner/operator should provide recommendations to DEQ. In most cases, it is anticipated that DEQ and the owner/operator will work together to identify the best alternative.

###### Development and Implementation of a Corrective Action Plan

Based on the results of the RE process, the owner/operator proposes a set of remediation standards and prepares a CAP describing the measures that will be used to achieve the standards, ensuring the long-term protection of human health and the environment. The CAP is submitted to DEQ for approval. The CAP may include a combination of options for active remediation and passive remediation along with limitations on activities and land uses that were assumed in the RE step. Details of this step are included in Section 5.

###### Evaluation  and Modification of the Corrective Action Plan

The data collected during implementation of the CAP must be evaluated by the owner/operator and submitted to DEQ so that DEQ can determine whether the CAP is progressing as anticipated. Modifications to the CAP may be necessary in some circumstances. Circumstances that might require modifying the CAP include identification of significant deviations from the remedial strategy, unacceptable risk levels to receptors,

or if data indicate that site cleanup is not progressing at the rate anticipated. In these cases, appropriate modifications to the CAP should be considered and an amended CAP submitted to DEQ for approval.

###### Site Closure with a Corrective Action Plan

When the regulatory obligations specified in the DEQ-approved CAP have been achieved, the site is eligible for closure. Closure involves the owner/operator submitting documentation that all the work specified in the CAP has been completed and all cleanup criteria have been achieved accompanied with a request for site closure.

###### Certification by a Registered Professional

If work is performed that the State of Idaho requires a geologist or professional geologist or professional engineer (Idaho Code, Title 54, Chapters 12 and 28) to perform, the owner/operator shall employ the use of, and obtain certification from, a geologist or professional geologist or professional engineer registered in Idaho for that work. An example that requires a professional engineer is construction of public works. A professional land surveyor is required for land surveys. Examples that require a geologist or professional geologist review are well design and most assessment, remediation system design, or corrective action that involves the subsurface.

# Section 2. Site Discovery and Initial Steps

#### Determining Imminent Threats

Once an impacted site has been identified or a release has occurred, the first step is to determine whether contamination poses any imminent threat to human health or the environment. If so, abatement measures must be completed before any other activities. Examples of abatement measures include taking action to prevent further release into the environment, providing an alternate water supply if drinking water supplies are impacted, evacuating residents/commercial workers if they are or could be exposed to vapors at high concentrations, installing booms on surface waterbodies that have a sheen, or ventilating utility facilities where vapors are present. If there is an imminent threat, or uncertainty regarding potential threats, associated with a release, call 911, if applicable, and STATECOMM at 1-800-632-8000 or (208) 846-7610.

Releases that do not pose an imminent threat do not have to be reported to STATECOMM, but do have to be reported to DEQ. Owner/operators shall follow the Petroleum Release Reporting, Release Response and Initial Abatement Measures as provided in IDAPA 58.01.02.851 and 852.

#### Initial Site Characterization

###### Site Characterization Introduction

Within the RE process, initial site characterization is performed after any necessary emergency response actions are completed. The overall objective of the initial site characterization is to identify the maximum chemical concentrations on the site in each of the affected media. These maximum concentrations are then compared with screening level criteria to determine the need for any further action.

A brief description of the initial site characterization process is presented below.

###### Site Description and Adjacent Land Use

The owner/operator must conduct a thorough site reconnaissance and a historical review of site operations to identify source(s) of contamination on site. Sources may be identified based on knowledge of a known or documented release; location of certain structures that typically represent a contamination source such as petroleum storage tanks (PSTs), pipes, pumps, etc.; interviews with current and former site employees who may have knowledge of source areas.

In addition to identifying sources, the owner/operator must collect data related to historic, current, and future land use on and adjacent to the site. A chronology of relevant site activities is often useful.

Based on this information, the owner/operator must identify the type of petroleum that may have been released at the site. Data collected during initial site characterization must follow DEQ’s quality management program, quality assurance project plan (QAPP), and standard operating procedures (SOP) or a DEQ-approved QAPP (see section 4.2 and Appendix A).

###### Soil Source Characterization

The owner/operator must collect soil data representative of the release’s maximum concentration of petroleum COI. The exact number of samples, the analytical methods used, and the specific technology used to collect data will vary among sites (see Appendix A for QA/QC references). It is recommended that the owner/operator develop a work plan and have it reviewed and approved by DEQ before beginning the work.

The overall intent of initial site characterization is to identify maximum concentrations of petroleum COI. However, for sites that will likely require further characterization, it may be efficient and cost-effective to collect additional data at this stage of the evaluation. Additional data may be used to identify the nature and extent of contamination and potential for exposure. For example, if contamination is suspected to exist in both surficial and subsurface soil zones, samples representing maximum concentrations in both zones must be collected.

###### Groundwater Source Characterization

In the initial site characterization, if there is evidence of possible groundwater impact from the site release, the owner/operator must collect groundwater samples below or immediately adjacent to the source. For sites that will require further characterization, it may be more efficient and cost-effective to install at least three monitoring wells so the magnitude and the direction of groundwater flow can be established.

###### Documentation of Initial Site Characterization

The owner/operator must document the initial site characterization results in a report that is submitted to DEQ. The reportmust include the information outlined in IDAPA 58.01.02.852.03 and DEQ’s QAPP and SOP (see Appendix A).

# Section 3. Screening Level Evaluation

Data collected during the initial site characterization is typically evaluated using screening levels. This evaluation is intended to achieve the following objectives of the screening level evaluation:

      - Demonstrate that the site or portions of the site do not pose a threat to human health and hence does not require any further evaluation, and

      - Identify areas of the site that need further evaluation.

These objectives are achieved by comparing the maximum site concentrations with the screening level values.

#### Screening Levels

The screening levels for the petroleum COIs adopted in the Rule have been developed by EPA and can be found on EPA’s website (see generic tables, RSL Calculator, and VISL Calculator). The screening levels represent unrestricted use and are risk-based target concentrations developed with the following:

    - the assumption that receptors will be residential,

    - conservative input parameter values,

    - an acceptable target risk of 10-6, and

    - a target hazard quotient (THQ) of 0.1

The screening level is the lowest of the media-specific (soil, soil vapor, and groundwater) risk-based concentrations for the following exposure pathways:. The default screening levels are based on a THQ of 0.1 and assumes multiple COIs with the same toxic endpoint. Almost all petroleum sites will include multiple COIs with the same toxic endpoint. Each petroleum COI and its associated toxic endpoint is presented in Table 1. The rationale for using a THQ of 0.1 for screening is that when multiple contaminants of concern are present at a site or one or more are present in multiple exposure media, the total hazard index could exceed 1.0 if each were screened at the THQ of 1.0. If you are screening only one contaminant, a THQ of 1.0 may be used but DEQ should be consulted (also see section 4.5).

Specific exposure factors and models used to develop screening levels are presented in Appendices A and E. Screening levels are the lowest target concentrations for the following exposure pathways and ROE for soil, soil vapor, and groundwater.

Specifically, screening levels for soil are the lowest of the following concentrations:

      - Surficial and subsurface soil concentrations protective of exposure via groundwater ingestion at maximum contaminant levels (MCLs) or risk-based concentrations at the downgradient edge of the source (use the Summary Table, Protection of Groundwater SSLs (soil screening level), MCL-based SSL column if an MCL for that COI exists or Risk-based SSL column if an MCL does not exist but the RSL calculator can also be used) or

      - Subsurface soil concentrations protective of exposure via indoor inhalation of vapors emanating from soil for a residential scenario, (use the VISL Calculator) or

      - Surficial soil concentrations protective of combined ingestion, dermal contact, and outdoor inhalation exposures for a residential scenario (use the Summary Table, Resident Soil column).

Screening levels for groundwater are the lowest of the following concentrations:

      - MCLs for chemicals that have them or calculated values for ingestion of water in a residential scenario (Use the Summary Table; MCL column if an MCL for that COI exists, Tap Water column where no MCL exists. However, EPA’s tap water screening level is based on dermal, ingestion, and inhalation so the Ingestion column in the Resident Tap Water table can also be used. The RSL calculator can be used for all scenarios.), or

      - Groundwater concentrations protective of exposure via indoor inhalation of vapors emanating from groundwater for a residential scenario (use the VISL Calculator rather than the EPA Summary Table for Resident Air and Industrial Air).

Screening levels for soil vapor are protective of exposure via indoor inhalation of vapors emitted from contaminated soil or groundwater. There are screening levels for both sub-slab and near-source soil gas using EPA’s VISL Calculator.

Because of the methods and assumptions used in the development of the screening levels and the current limitations of laboratory analytical methods, the calculated screening levels may be lower than the practical quantitation limit reported by a laboratory for selected chemicals. In these situations, site-specific review by DEQ will be required based on the criteria provided in

Section 500 of the Rule and Appendix E. Additionally, screening levels for chemicals without an MCL are based on conservative assumptions about the release and transport of contaminants in the subsurface. With justification, many of these parameters can be changed by the user in the calculator’s site-specific mode.

Table 1 identifies the appropriate pathways to consider for each COI.

Table 1.  Soil, Groundwater, and Soil Vapor Chemicals of Interest, Applicable Pathways, and Their Associated Toxic Endpoint

| Chemical | Gasoline/ JP-4/ AVGas | Diesel/ Fuel Oil No. 2/ Kerosene/Jet Fuels (Jet A, JP-5, JP-8) | Fuel Oil No.4 | Carcinogen | Mutagen | Target Endpoint(s) Chronic Exposure | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Benzene | X | X |  | C | No | Hematologic System, reproductive and developmental | ATSDR |
| Toluene | X | X |  | NC | No | Central Nervous system | ATSDR |
| Ethylbenzene | X | X |  | PC | No | Kidney, ear (irreversible damage inner ear) | ATSDR |
| Xylenes (mixed) | X | X |  | NC | No | Nervous system, Lungs, skin | ATSDR |
| 1,2 Dibromoethane (EDB)1 | X1 |  |  | PC | No | Reproductive, lung, skin, Nervous system | ATSDR |
| 1,2 Dichloroethane (EDC)1 | X1 |  |  | NC | No | Nervous system, liver, kidneys | ATSDR |
| Methyl Tert-Butyl Ether (MTBE) | X |  |  | NC | No | Nervous system, liver, and kidneys | ATSDR |
| Acenaphthene2 |  | X | X | PC | No | Kidneys, liver | NCBI 2024 |
| Anthracene2 |  | X | X | NC | No | Kidney, breast | NCBI 2024 |
| Benzo(a)pyrene2 |  | X | X | PC | Yes | Lungs, liver, stomach, skin | ATSDR |
| Benzo(b)fluoranthene2 |  | X | X | PC | Yes | Skin, lungs | ATSDR |
| Benzo(k)fluoranthene2 |  | X | X | PC | Yes | Skin, lungs | ATSDR |
| Benz(a)anthracene |  | X | X | PC | Yes | Lung, stomach skin, liver, bone | ATSDR |
| Chrysene2 |  | X | X | PC | Yes | Lung, stomach, skin | ATSDR |
| Fluorene2 |  | X | X | NC | No | Lung, stomach, skin | ATSDR |
| Fluoranthene2 |  | X | X | NC | No | Lung, stomach, skin | ATSDR |
| Naphthalene | X | X | X | PC | No | Lungs, blood, eyes, liver, central nervous system, skin | EPA |
| Pyrene2 |  | X | X | NC | No | Kidney | EPA |
| 1 Leaded only 2 Vapor intrusion is not applicable because there is no inhalation toxicity information and/or the chemical is not sufficiently volatile and toxic to pose an inhalation risk from a soil or groundwater source. |  |  |  |  |  |  |  |

#### Comparison of Site Concentrations with Screening Levels

If the maximum site concentration does not exceed the screening level for any petroleum COI in any media, the site is eligible for closure subject to other regulatory obligations (IDAPA 58.01.24.200) and the owner/operator may request DEQ approval for site closure. If the maximum site concentration for any petroleum COI exceeds the screening level, the owner/operator must select one of the following options:

    - Option 1:	Adopt screening levels as remediation standards and develop a CAP (see Section 1).

    - Option 2:	Perform a more detailed, site-specific RE.

If option 1 is chosen, the owner/operator must submit a report that includes the information outlined in IDAPA 58.01.02.852.03 and DEQ’s QAPP and SOP (see Appendix A). The owner must then submit a CAP that includes the information outlined in IDAPA 58.01.24.xx and Section 5.

# Section 4. Site-Specific Risk Evaluation

A site-specific RE is needed only if the owner/operator chooses not to use the default screening levels as the remediation standards.

Conducting a site-specific RE requires the completion of several steps. These include developing a CSM, identifying the chemicals present in environmental media, assessing exposure and exposure pathways, assessing the toxicity of the petroleum COIs, characterizing human health and ecological risks. This

section describes each of these steps and also provides the suggested format for some of the deliverables (most of which are reports) that assist in developing and documenting the evaluation.

#### Conceptual Site Model

A CSM provides an overall understanding of the site and a CSM report can be a convenient format for compiling all the relevant data associated with a release. The CSM provides a framework for the entire project and can help identify specific data needs (the correct quality and quantity of data that will be needed). A CSM is necessary in the development of any data collection work plan and is also an important communication tool for regulators, owner/operators, and stakeholders.

In addition to basic site information (discussed in section 4.1.1), there are three key elements that must be included in the CSM. Each element will require certain types of data in order to validate the CSM. The amount of data required is typically based on site-specific considerations. The CSM is a dynamic entity that should be revisited and potentially refined any time additional data is collected for a site. The resulting CSM refinements must be incorporated, if necessary, into the RE.

The three key elements are:

- Sources: Contaminant source areas (nature, magnitude, and extent of release) and COI (see section 4.1.2).

- Exposure Pathways: Release, fate and transport mechanisms, exposure pathways, and ROE (see section 4.1.3).

- Receptors: Characteristics of current and reasonably future land uses and receptors (see section 4.1.4).

More detailed information on these elements is provided below and in the sections on data requirements (section 4.2) and exposure assessment (section 4.3). OSWER’s Technical Guide for Assessing and Mitigating the Vapor Intrusion Pathway from Subsurface Vapor Sources to Indoor Air (Appendix A), discusses developing a CSM for vapor intrusion.

###### Site Characteristics

The following basic site characteristics must be included in the RE; each is described in the following sections:

- Site history and map,

- Ground surface conditions,

- Location of utilities on and adjacent to the site, and

- Location and description of site structures.

      - Site History and Map

The CSM must include a comprehensive chronology of events including remediation projects, tank removal activity, reported releases, etc. The chronology of events must be clearly and accurately documented.

All maps must be made to scale, with a bar scale and a north arrow. As appropriate, multiple site maps must be prepared to show sample collection points and locations of various site structures. A detailed facility map showing the layout of USTs, above ground storage tanks (ASTs), pipes, loading and unloading areas, sumps, paved and unpaved areas, canopy buildings, etc. must be prepared. A second facility map must be prepared to show locations of all on-site monitoring wells, water use wells, soil borings, soil vapor extraction wells, and soil excavation areas. All on-site structures must be clearly identified.

      - Ground Surface Conditions

Before conducting an RE, the owner/operator and any other parties that will be involved should make a site visit and walk through. It is important to document areas of the site that are paved, unpaved, landscaped, or covered with buildings. It is also highly useful to note the type, extent, slope, and general condition of the ground surface, and current land uses.

      - Location of Utilities on and Adjacent to the Site

Due to the potential for preferential flow of contaminated groundwater and vapors into underground utility lines and conduits, a thorough documentation of the location of and potential impacts to underground utilities must be performed. Utilities may include phone lines, water lines, sanitary sewers, storm sewers, and natural gas lines. A combination of site observations, knowledge of buried utilities, and discussions with utility representatives and the site owner should reveal the utility locations.

At a minimum, the following activities must be performed:

- Locate all underground utility lines and conduits within the area of known or potential soil and groundwater impact, both on site and off site, where the release may have migrated, or may migrate in the future.

- Determine the direction of flow in the utilities (water, storm water, and sewage).

- Identify utility lines/conduits on a base map that illustrates the extent of soil and groundwater impacts.

- Determine depth of utility lines/conduits relative to the depth of groundwater. Seasonal fluctuations of groundwater levels must be carefully evaluated. As appropriate, a cross-sectional diagram should be provided illustrating the depth to groundwater and the locations and depths of the lines/conduits.

- Determine any past impacts to utilities and any pertinent complaints that may have been previously filed with DEQ.

      - Existing Structures

The location, condition, and construction characteristics of each structure on and adjacent to the site should be described.

###### Contaminant Source Areas

This element of the CSM must include a discussion of the nature, location, timing, and magnitude of petroleum products spilled at the site. It is important to understand the location and timing of past spills and releases in making decisions regarding assessment needs related to current releases. The petroleum COI must be determined. Any actions performed as a part of emergency response must be delineated and their impact on the source evaluated. The residual size of the source after any emergency or interim actions must be determined.

Knowledge about the nature, magnitude, and extent of the release is necessary to identify the petroleum source in soil and/or the groundwater at the site as well as to identify the petroleum COIs. The following information regarding a release, if available, is critical:

- Release location,

- Release quantity,

- Petroleum product released, and

- Interim corrective action measures performed.

Release-related information can be obtained by reviewing inventory records, interviewing current and past employees, and checking any spill incident reports filed with DEQ. Information related to site activities can also help identify source location and petroleum COIs.

      - Location of Release

The location of the release defines the source of the petroleum that is in the soil and potentially the groundwater. Release locations include, but are not limited to, corroded or damaged tanks, pipe bends and joints, dispensers, and loading and unloading areas.

The owner/operator must review operational history to determine the location of the source. The assessment should focus on the identified release area; however, the exact location and timing of the spill source area may not be known. In these cases, soil and groundwater sampling must be used to identify the extent (vertical and horizontal) of residual soil and/or groundwater source. The exact number and location of necessary samples shall be determined on a site-specific basis using professional judgment and in consult with DEQ. DEQ may be able to provide information that can help the owner/operator improve the overall efficiency of sample collection or the entire RE and eliminate or reduce the need for re-sampling. .

      - Quantity of Release

An estimate of the amount released may help evaluate the severity of the impact and extent of contamination, and may help in planning site characterization measures. Estimation of release quantities is typically based on inventory records.

      - Chemicals of Interest and Spatial and Temporal Trends

This section must include a discussion of chemicals detected in each media and their spatial and temporal distribution. Contour maps indicating concentrations of individual petroleum COIs may be useful. Graphs of petroleum COI groundwater concentrations in individual wells and concentration along the flow line (concentration vs. distance plots) may be useful as well. Based on data collected over time, it should be determined whether the chemical concentrations are stable, declining, or expanding.

Identification of specific petroleum product(s) spilled or released is important to identify the petroleum COIs. Evaluation of non-petroleum chemicals in addition to those in Subsection

800.01 (Table 1) may be required by DEQ when there is a reasonable basis based on site-specific information. A reasonable basis shall be demonstrated by DEQ when it can show documentation of releases or suspected releases of other non-petroleum chemicals. The environmental behavior (mobility, persistence, biodegradation, and inter-media transport) of the petroleum product and its adverse environmental and human health effects depend on chemical properties and their concentrations in the petroleum product.

If a release can be identified as a single petroleum product based on a documented release, free product analysis, or location of impact (e.g., tank bottom of a particular product tank), only the COIs for that petroleum product need to be analyzed. If the product released cannot be conclusively identified, it is recommended that monitoring samples be analyzed for all petroleum COIs associated with the products suspected to have been stored at the site. Additional assessment may be required to identify the source of the release.

If previously collected data did not include all suspected site petroleum COIs, additional sampling may be necessary for petroleum COIs before a RE can be performed.

###### Exposure Pathways and Transport Mechanisms

The objective for this portion of the CSM is to describe site characteristics and mechanisms which have influenced or will influence the release, fate, and transport of chemicals of interest and to determine which exposure pathways and ROE will be most likely to occur.

      - Site Stratigraphy and Hydrogeology

The owner/operator mustconduct a review of published literature and any investigations conducted on adjacent sites to determine regional hydrogeology, soil types, and aquifer characteristics. This evaluation is used to determine the type and depth of aquifers in the area and whether they are confined, semi-confined, or unconfined. Regional information will help the owner/operator in efficiently collecting site-specific soil and groundwater information. The CSM must include a detailed discussion of site-specific stratigraphy and hydrogeology.

Site stratigraphy can be determined based on boring logs and an adequate number of geologic cross sections.

The hydrogeologic discussion must include estimates of horizontal and vertical hydraulic gradients, seasonal variations in groundwater elevations and flow direction, and hydraulic conductivity of relevant water-bearing zones. How these characteristics may influence the fate and transport of chemicals to potential receptors should be discussed.

The survey should also locate potentially impacted surface waterbodies located within one-half (1/2) mile of the site. If a surface water body is identified, information including the type (perennial or intermittent), water flow rate, flow direction, depth of water, width or surface area of the water body, and water use must be provided. The location of the water body must be indicated on an area map.

      - Complete Exposure Pathways and Routes of Exposure

A complete exposure pathway involves a source of petroleum products, release and transport mechanisms, ROE, and potential receptors. The definition of complete exposure pathways starts with knowledge of the release, petroleum COIs, and site physical conditions (described above), then combines these with assumptions about land use and receptors. Exposure pathways and ROE are described and discussed in more detail in section 4.3.3.

The CSM must identify each of these components in its descriptions of exposure pathways. One example of a common exposure pathway would be leaching of petroleum chemicals from a gasoline release in soils (release) to the groundwater with subsequent transport (fate and transport) to a residential well where water is extracted for drinking water and ingested by residents (land use and receptors). Another would be volatilization of petroleum chemical vapors from a soil source with subsequent transport through the soil and into the air in an occupied structure where they are inhaled.

Another part of defining the complete or potentially complete exposure pathways and ROE in a CSM is identifying the locations where exposure to petroleum chemicals can occur and the media of concern at those locations. These known or potential exposure locations are commonly called exposure units. The exposure unit for each complete pathway and receptor must be described. The exposure unit may consist of a specific location such as a well or it may be an area, such as a parcel, over which receptors move. For large sites, it may be useful or necessary to subdivide the site into multiple exposure units. This may also be true at sites where impacts of the release have moved off-site.

Exposure pathways defined in the CSM are subsequently used in the exposure assessment phase of the RE, which is described in detail in section 4.3.

      - Protection of Groundwater

During the development of the CSM, it is assumed that groundwater may be used for drinking water. Consequently, if groundwater is not remediated to levels suitable for ingestion, the CAP must also include provisions in the final remedy for implementing limitations on activities and land uses. Such limitations are achieved through environmental covenants (activity and use limitations [AULs]) to prevent exposure via groundwater ingestion throughout the horizontal and vertical extent of the plume that has concentrations exceeding the RSL for groundwater ingestion.

The CSM, as it relates to the use of groundwater, must include characterization and evaluation of the following groundwater aspects.

- The current and historical use of the groundwater for drinking water or irrigation.

- The location and approved use of existing groundwater wells within a one-half (1/2) mile radius from the release point at the contaminated site.

- The potential for communication between the impacted groundwater and other groundwater-bearing zones or surface water.

- The location of delineated source water protection areas for public drinking water systems.

This information, along with other data collected for the evaluation, will be used to determine if groundwater ingestion is a complete exposure pathway in the CSM, and will guide the selection and use of appropriate remedial target concentrations, measures, timeframes, and compliance conditions.

A water well survey must be conducted to identify all water use wells within a one-half (1/2) mile radius. Information sources include the U.S. Geological Survey, the Idaho Department of Water Resources (IDWR), water system operators, and local residents. Well characteristics including depth, water use, and screened interval must be documented. It is necessary to identify any dewatering wells located within a 1,000-foot radius of the site (or greater if the groundwater plume is extensive).

Land use considerations include, but are not limited to:

- Current zoning and land use. Consultation with local planning and zoning officials regarding future land use planning direction and interpretation of planning and zoning policies.

- Land use development trends. Conversion of agricultural land to residential land uses requires development of drinking water sources. The proximity of these conversions to urban centers and available public drinking water supplies must be taken into account.

- Local ordinances that include agreements with IDWR governing well drilling or state- designated restrictions or specifications for constructing wells into groundwater in a locality for drinking water purposes.

- The existence of source water assessment delineations and source water protection plans and ordinances associated with public drinking water supplies.

- The availability of alternative water supplies.

###### Land Use and Receptors

This portion of the CSM describes the current and future assumptions about land use as they relate to identifying potential receptors. The identification of land uses includes both the site of the release as well as adjacent properties which may be impacted by the release.

      - Land Use

Evaluating current and reasonably future land uses at and adjacent to the release site is a critical component when determining potential exposure points, exposure pathways, receptors, and exposure factors during the RE process and when determining cleanup concentrations.

Residential land use generally requires lower target concentrations. Cleanup to residential standards will usually allow unrestricted land use. Whenever assumed land use is other than

residential and cleanup is not to residential standards, DEQ will require that an AUL be imposed on the property through the use of an environmental covenant.

Residential land uses include use by sensitive subpopulations (e.g., children, older adults, pregnant women). Examples of land uses with sensitive subpopulations include but are not limited to hospitals, nursing homes, schools, childcare centers, farms with houses, and any other areas/structures with population groups considered sensitive.

The owner/operator must submit to DEQ illustrated land use maps clearly identifying current land uses at the site and the adjacent properties. One map must clearly show the area within one-half (1/2) mile of the known or estimated extent of contamination. At sites where there is likelihood that the extent of impacts may be greater, due to the magnitude of the spill or other site-specific conditions, a land use map covering the entire impacted and potentially impacted area is necessary. A walking land use survey should be conducted for the area within a 500-foot radius of the source, with an emphasis on the down-gradient direction and potential off-site impacts. The survey should clearly identify the following: schools, hospitals, residences (apartments, single-family homes, and others), day care centers, nursing homes, and types of businesses. In addition to this survey, efforts should be made to identify structures with basements (which may enhance the transport of contamination) in areas in closer proximity to the source and where site conditions warrant it, such as where shallow groundwater or non- aqueous phase liquid (NAPL) is present. The map should also identify surface water bodies, parks, recreational areas, wildlife sanctuaries, wetlands, and agricultural areas.

Current land use refers to land use as it exists today and that can be readily determined by a site visit. A site visit should identify homes, playgrounds, parks, businesses, industries, or other land uses at, and in close proximity of, the release site. As appropriate, state or local zoning boards; the U.S. Census Bureau; zoning, topographic, land use, housing and other types of maps; and aerial photographs can provide information for determining land use.

Undeveloped land  is considered residential unless the owner/operator develops and implements an AUL as a part of a DEQ-approved CAP.

The exposures to be evaluated in a RE can depend upon the activities that could occur under future uses of land and groundwater at the site.

For example, consider property that is currently used as farmland. If the impacted farmland includes a residence, the current land use is residential. However, if the owner/operator provides information establishing clear plans to develop the residence into a nonresidential building in the near future, the future use may be evaluated as nonresidential but will still require that an AUL be imposed on the property.

While evaluating future land use(s) presents uncertainties, DEQ has identified certain factors that assist in this evaluation. These factors include, but are not limited to, local zoning ordinances; knowledge of current land use and changing land use patterns; zoning decisions; community master plans; interviews with current property owners; nonresidential appraisal

reports; proximity to wetlands, critical habitat, and other environmentally sensitive areas, such as source water protection areas; and the use of remedial action institutional controls at a site.

Site characterization will include a determination of the on-site and off-site areas of impact. These areas are considered in determining the pathway-specific exposure units for each type of receptor. Exposure units are discussed in more detail in section 4.3 and Appendix C.

      - Receptors

The objective of the RE is to quantify the adverse health effects to current and potential future receptors both on-site and off-site. For simplification, the following definitions are used:

- On-Site −The area located within the legal property boundaries within which the source of the release is located. This includes soil, groundwater, surface water, and air within those boundaries. Adjacent property purchased after the release will be considered off-site.

- Off-Site −The areas of concern located outside the boundaries of the property where the release source is located. This includes soil, groundwater, surface water, and air located outside the property boundaries.

Risk is evaluated for various receptors depending on the specific exposure route, and whether the petroleum COI is noncarcinogenic, carcinogenic, or carcinogenic with a mutagenic mode of action. EPA’s RSL Calculator User’s Guide explains the various human receptor types (resident, composite worker, outdoor worker, indoor worker, construction worker, recreator), exposure routes, exposure durations for the receptors, and how  some chemicals and exposure route risk calculations incorporate modifications based on exposure occurring at different life stages.

      - Ecological Risk Considerations

In addition to identifying potential impacts to human receptors the CSM should also attempt to identify potential exposure pathways by which sensitive habitats, such as wetlands, surface water bodies, or other ecologically significant environments near the site, may be impacted by the release where wildlife may be the potential receptors.

Threatened and endangered species that may be exposed to site-specific chemicals should be identified. Note that within the RE process, protection of surface waters and streams is independent of an ecological RE. As appropriate, a walking survey in the vicinity of the site may be necessary to identify ecological receptors.

Contact DEQ to obtain additional guidance on these issues.

#### Data Requirements and Evaluation

Once the CSM has been developed, an evaluation of data collection needs, data quality needs, and data collection methodology must be conducted. Depending on the amount and complexity of the proposed data collection effort, a work plan may be needed to guide the data collection process. The suggested contents of this work plan are discussed in section 4.7.

This section describes general data collection objectives and requirements for the RE process. This includes the general QA/QC considerations to help ensure the data collected is of sufficient quality and quantity to meet desired objectives, the categories of data necessary to meet these objectives, and data collection techniques.

Data collection objectives for RE include, but are not limited to:

- Accurately characterize the magnitude, nature, and extent of contamination, including the characterization of petroleum chemical concentrations for all impacted media, as appropriate,

- Allow the development and/or validation of an accurate CSM, and

- Develop estimates of risks posed by the release.

###### Data Collection Planning

Data collection planning involves a careful review of all available site information and data for its suitability in the RE. This process, in the context of the CSM, must identify any data gaps with the goal of determining the types, locations, quality, and numbers of samples or measurements needed.

Depending on what stage of the RE process is in for a given site, the relative importance and specific data requirements of each objective described above will vary. The goal of data collection is to efficiently reduce uncertainty in those areas that contribute the most to potential or actual risk and for which the greatest uncertainty exists.

For example, it may be determined through the CSM that vapor intrusion represents the largest potential source of risk at a site, but little is known about subsurface conditions which may impact exposure point concentrations and only soil or groundwater data are available. In order to more realistically model this exposure pathway, it may be most effective to better characterize subsurface stratigraphy and properties, document the conditions supporting biodegradation, and collect soil vapor data.

At many impacted sites, data collection efforts may have been conducted multiple times over an extended period of time. For example, interim corrective action measures may have been undertaken prior to planning for the RE. Soil and groundwater data collected prior to such activities may not be representative of current conditions and generally should not be used in RE. At such sites, collection of additional soil and groundwater concentration data may be needed after completion of interim corrective measures.

The exact number of samples required is a site-specific decision based on the balance between cost and representativeness and requires professional judgment and expertise. Numerous tools are available to assist in estimating the number of samples required for statistical analysis, but

none provide a definitive number and statistical analysis may not be practical in many cases. These tools include, but are not limited to, data quality objectives (DQO) and statistical and geo- statistical evaluation. Selected references to assist in developing and completing data collection efforts are listed in section 7.2.

###### Quality Assurance/Quality Control Considerations

The RE process relies on site-specific data to make decisions related to the magnitude of site risk, nature and extent of remedial activity, and site-closure. Thus, it is important that data be reliable, representative, complete, and of known quality.

In order to assure the data will be of appropriate quality, QA/QC activities must be applied to all environmental data collection activities throughout the RE process (site characterization, risk evaluation, remediation implementation, and monitoring).

While the level of QA/QC applied to data collection efforts will vary (depending on factors such as site complexity, size of the release, and the immediacy of the response) all the elements of QA/QC described in DEQ’s quality management program, QAPP, and SOP or DEQ-approved QAPP (see section 4.2 and Appendix A) must be followed. This will allow an adequate review of the quality of the data used in the analysis.

If a work plan is submitted to DEQ for approval, it should include a project-specific QAPP. The QAPP documents the QA/QC procedures applied to various aspects of the project to assure that the data obtained are of the type and quality required for decision-making.

EPA has developed guidance for the development of QAPPs

and

DEQ has developed an internal QAPP template that may be restructured for external use (see Appendix A).

The QAPP is implemented during the data collection process. Problems can be identified and corrected at this stage. The impact of field and laboratory techniques and sampling and analysis conditions on data quality are determined using field and laboratory QC samples and periodic audits. Oversight and corrective action can prevent improper procedures or techniques from continuing.

Data verification and validation should be performed to validate data quality and usability. Data verification and validation is particularly dependent on compliance with field and laboratory procedures for sample collection, identification, handling, preservation, chain of custody management, shipping, analysis, and reporting.

EPA and DEQ provide several guidance documents to assist in performing data verification and validation (see Appendix A).

###### Documentation of the Data Collected

Once data have been collected, a field investigation report must be prepared and follow DEQ’s quality management program, QAPP, and SOP or a DEQ-approved QAPP (see section 4.2 and Appendix A).

.

###### Data Collection Methods

The common categories of data that are the focus of data collection efforts for a RE include:

- Vadose zone soil characteristics,

- Saturated zone characteristics, and

- Distribution of petroleum COIs in soil, soil vapor, groundwater, and surface water bodies.

EPA has developed default values for many of the vadose and saturated zone characteristics that affect the fate and transport of petroleum COIs. These values are presented in EPA’s RSL Calculator User’s Guide and tables. These default values can be used or site-specific values can be established.

###### Vadose Zone Soil Characteristics

The vadose zone or unsaturated zone soil is the media through which petroleum COIs can migrate to the groundwater and vapors can move upward to the surface and into an enclosed space. Thus, characteristics of vadose zone soils have considerable impact on risk estimates and target concentration calculations. Relevant vadose zone characteristics include:

- Vadose zone thickness and depth to groundwater,

- Bulk density,

- Porosity,

- Water and air content, and

- Fractional organic carbon content.

In addition to parameters mentioned above, additional site-specific, exposure pathway-specific parameters that may be valuable to measure include:

- Vapor permeability, effective diffusion coefficient, and building characteristics where indoor inhalation is a potentially complete exposure pathway, and

- Infiltration rates for developing a detailed evaluation of the potential for leaching to groundwater.

      - Thickness of Vadose Zone and Depth to Groundwater

Vadose zone thickness is determined from boring logs. This thickness is the distance from the ground surface to the depth at which the water table is encountered, less capillary fringe thickness.

Depth to groundwater is used in estimating vapor emissions from groundwater. For indoor inhalation, depth to groundwater below the “floor” of an existing structure of concern or the most likely location of a future structure should be used.

For sites with considerable seasonal fluctuation in water table level, depending on the data available and the nature of the fluctuations, a yearly average depth or a time-weighted depth may be used. The vadose zone depth that is estimated should also form the basis for the selection of soil samples to be used in evaluation of subsurface soil vapor emissions to indoor air (see EPA’s Technical Guide for Addressing Petroleum Vapor Intrusion at Leaking Underground Storage Tank Sites, Appendix A). Shallower water table depths often result in groundwater target levels set at lower concentrations to be protective of inhalation exposure pathways.

      - Porosity

Porosity is the ratio of the volume of voids to the total undisturbed field volume of soil. Many laboratories use dry bulk density and specific gravity data to determine porosity using the following equation:

n = 1 - ρb/ ρs	(4-1)

where,

n	=	porosity (cubic centimeter per cubic centimeter [cc/cc]) ρb	=	dry bulk density (grams per cubic centimeter [gm/cc]) ρs	=	specific gravity or particle density (gm/cc)

Dry bulk density is the dry weight of a soil sample divided by the field volume of the soil sample. An accurate measurement of bulk density requires determining the dry weight and volume of an undisturbed sample. This method involves collecting a core of known volume, using a thin-walled sampler, such as a Shelby tube or similar coring device, to minimize disturbance of the sample, and transporting the core to a laboratory for analysis. This method is described in American Society for Testing and Materials (ASTM) Method D2937, Standard Test Method for Density of Soil in Place by the Drive-Cylinder Method.

The Standard Test Method for Specific Gravity of Soil, ASTM Method D854, may be used to determine specific gravity. If specific gravity is not assessed, then 2.65 gm/cc can be assumed as the particle density.

Consideration should be given to collecting multiple samples if multiple lithologies are present that might affect COI transport.

If site-specific values of porosity are not available, it should be estimated based on values provided in an appropriate literature source.

      - Volumetric Water Content/Moisture Content

Volumetric water content is the ratio of water volume to the total soil volume. The ASTM Method D2216, Standard Test Method for Laboratory Determination of Water [Moisture] Content of Soil and Rock by Mass is a gravimetric oven-drying method; however, the water content value used in most models is the volumetric water content.

Hence, it may be necessary to use the following equation to convert gravimetric water content to a volumetric basis:

|  |  | (4-2) |
| --- | --- | --- |
| where, |  |  |
| θwv | = | volumetric water content (cc water/cc soil) |
| θwg | = | gravimetric water content, typically reported by the laboratory |
|  |  | (gm of water/gm of soil) |
| b | = | dry bulk density (gm of dry soil/cc of soil) |
| l | = | density of water (gm/cc) |

Volumetric water content can also be measured in the field through the use of a variety of instruments such as a neutron probe, time domain reflectometer, or gypsum block sensors.

      - Fractional Organic Carbon Content in Soil

Fractional organic carbon content is the organic carbon weight in the soil divided by soil weight and is expressed either as a ratio or as a percent. The Walkley Black Method

is a chemical oxidation method (rapid dichromatic oxidation) while ASTM Method 2974 is a furnace method for determining fractional organic carbon content in soil.

Results are usually reported as percent organic carbon content. The reported value can be converted to a fraction by dividing by 100.

If measurements of total organic matter content are available, they should be divided by 1.724 to estimate the fractional organic carbon content. This adjusts for the portion of soil organic matter that is actually carbon. Typically, total organic carbon content is estimated using ASTM Method 2974. Organic carbon content should be determined using soil samples not impacted by the release.

###### Saturated Zone Characteristics

Petroleum COIs that reach the water table typically travel horizontally through the saturated zone. However, if a vertical gradient is present, chemicals may also move vertically in the direction of the gradient. Saturated zone characteristics that determine the travel time and direction for the petroleum COIs include:

- Horizontal hydraulic conductivity,

- Horizontal and vertical hydraulic gradients (magnitude and direction), and

- Saturated zone soil characteristics (fractional organic carbon content and porosity).

      - Hydraulic Conductivity

Hydraulic conductivity is the discharge of water per unit area per unit hydraulic gradient in a subsurface formation. Estimates of site-specific hydraulic conductivity can be obtained by conducting aquifer tests such as slug or pump tests. Data gathered during the tests are then analyzed using appropriate methods. Slug tests are easier to conduct than pump tests, generate no wastewater for treatment, and may be more appropriate for low-permeability formations. The primary disadvantages are the small aquifer volume that is explored, resulting in the need to conduct multiple tests across the site. Properly conducted pump tests will often provide better estimates of hydraulic conductivity. The simplest pump tests are single-well short-duration (2 to 4 hours) tests. The best pump tests employ a pumping well and multiple observation wells and are of longer duration (12 hours or more). These tests will identify boundary effects. Regardless of the type of test conducted, different methods of data analysis will often yield different estimates of hydraulic conductivity. ASTM Method D4043 provides guidance on the selection of aquifer test methods.

In the absence of these tests, estimates of hydraulic conductivity may be obtained from literature, with values corresponding to the type of soil in the saturated zone, using empirical equations based on the grain size distribution of the porous formation, or they may use specific capacity data from well logs of wells in the vicinity of the site that are representative of the aquifer being investigated. In either case, adequate references and justification for the value chosen must be provided.

      - Hydraulic Gradient

The magnitude and direction of the hydraulic gradient is estimated by comparing water levels measured in the monitoring wells. Typically, water level contour maps are prepared based on

measured data using a computer program or manual calculations along with professional judgment. Calculations done using automated procedures should be spot-checked with hand calculations. Three wells or more are needed to adequately estimate the direction of flow and magnitude of the gradient. When drawing the contour maps, care should be taken to ensure that the measurements used are from monitoring wells that are screened in the same interval or hydrologic unit.

For sites that have seasonal variation in hydraulic gradient, estimate the average hydraulic gradient for each season. Consideration should also be given to determining any vertical gradients. This requires a comparison of adjacent water levels in wells screened in different intervals.

In areas where the shallow aquifer has been impacted and a deeper aquifer is used for drinking water, the vertical gradient must be determined. When drilling deep wells, care should be taken to avoid cross contamination.

      - Saturated Zone Soil Characteristics

In addition to hydraulic conductivity, other important saturated zone soil characteristics include fractional organic carbon content, porosity, and bulk density. These parameters are required to quantify the movement of chemicals within the saturated zone. The laboratory methods to measure these parameters were discussed in section 4.2.5.

###### Indicators of Biodegradation

Biodegradation of petroleum chemicals occurs in all media (soil, soil vapor, groundwater). Depending on the media, different indicators (chemical concentrations, geochemical indicators, microorganisms, oxygen, and carbon dioxide) can be measured at a site to demonstrate the occurrence of biodegradation of these organic substances.

When evaluating the potential for biodegradation in groundwater, these indicators are commonly classified into three groups: primary, secondary, and tertiary lines of evidence. Data collected under each line of evidence can be evaluated qualitatively or quantitatively to determine the occurrence of biodegradation.

The primary line of evidence demonstrates spatial stability or a reduction in chemical concentrations at a site by evaluating measured concentrations within monitoring wells, groundwater velocity, rates of contaminant transport, and time of the release.

The secondary line of evidence refers to measurement of geochemical indicators including reduction-oxidization (REDOX) potential, dissolved oxygen, dissolved nitrates, manganese, ferrous iron, sulfate, and methane. These indicators should be measured in at least three wells located along the flow line, one of which should be located at a background or up-gradient location, one within the plume near the source, and one within the plume down-gradient from the source.

The tertiary line of evidence involves microbiological studies such as identification of types of subsurface microbial populations and microbe cell counts.

Commonly used methods to estimate biodegradation rates include mass balance analysis for expanding, stable, or shrinking plumes and plots of plume concentration vs. distance. Additional details on biodegradation for petroleum COIs in groundwater are provided in Appendix B.

When evaluating the potential for, or presence of, biodegradation in soil or soil vapor, the most common indicators used are soil vapor concentrations, oxygen, and carbon dioxide. These are commonly measured at multiple depths in the subsurface and related to a potential source. More information on evaluation of biodegradation of petroleum vapors in relation to the vapor intrusion exposure pathway is provided in EPA’s Technical Guide for Addressing Petroleum Vapor Intrusion at Leaking Underground Storage Tank Sites (Appendix A).

###### Distribution of Petroleum Chemicals of Interest

Petroleum COI may be distributed between soil, soil vapor, groundwater, surface water, and sediments present at a release location. Knowledge of contaminant distribution entails a determination of the spatial extent and magnitude of concentrations in each of these media, where they are present.

      - Distribution of Petroleum Chemicals of Interest in Soil

Adequate soil concentration data are necessary to estimate risk to receptors, compare representative concentrations for each complete exposure pathway to target levels, and define the soil source dimensions. Sufficient data must be collected to define horizontal and vertical extent of impacts. The vadose zone investigation(s) must be organized to:

- Identify the horizontal and vertical extent of soil impacts. Unless otherwise directed by DEQ, the extent of impact is defined as those areas where petroleum COI concentrations exceed screening level concentrations.

- Identify areas of maximum concentration of petroleum COIs.

- Collect samples adequate to estimate exposure point concentrations for the potentially complete exposure pathways that have been defined based on the CSM.

To determine the spatial extent of contamination, soil borings should be drilled starting from the known or suspected source area and drilling outwards until borings with sample concentrations at or below screening levels are reached in all directions. To determine the vertical extent of contamination, soil borings areextended to the base of petroleum impact and samples collected from surface and subsurface soil zones as explained in the following sections.

The soil analytical data should help identify the soil source area. If more than one source area is identified at a site, each source area is evaluated separately. Once the soil source(s) is identified, source dimensions can be estimated. These dimensions can be used in the RSL and VISL calculators, along with other input parameters, to estimate risk and remediation standards protective of indoor inhalation and to evaluate the soil to groundwater leaching exposure pathway. Depth to subsurface soil source (used to estimate the target concentrations) is the depth, in the source area, from the surface to the zone where concentrations are above quantification limits. Professional judgment should be used in choosing the representative depth.

        - Surficial Soil Sampling

The RE process distinguishes between surficial soil and subsurface soil zones. Surficial soil is defined as the soil zone from the ground surface to 1 foot below ground surface. Exposure pathways that may apply to the surficial soil zone include direct contact exposure via incidental ingestion, inhalation of vapors and particulates, dermal contact, and leaching of petroleum COIs to groundwater and surface water.

Evaluation of soil exposure pathways within the surficial soil zone requires collection of an adequate number of soil samples to estimate both maximum and representative concentrations of all potential petroleum COIs. Given the small vertical depth interval of this soil zone, close attention should be given to vertical variations in contamination. The acceptability of vertical depth-composite samples versus samples from discrete depth intervals is a site-specific decision. Criteria that influence this decision include the type of petroleum COIs, vertical extent of contamination, and areal extent of the contaminated zone. Sampling within the surficial soil zone is typically done from test pits using hand samplers such as trowels or corers.

The presence of impervious (paved) surfaces poses difficulties for sampling. In some cases, very permeable material may be located 2 inches below the pavement. Residues from the paved surface may also be present. When sampling beneath impervious surfaces, sampling should begin 2 inches or deeper below concrete or asphalt pavement. Cracked areas in impervious surfaces may represent conduits for chemical migration or leaching and should be evaluated during the selection of sampling locations.

        - Subsurface Soil Sampling

Soil below the surficial soil zone (more than one foot below ground surface) and extending to the water table is termed the subsurface soil zone. Exposure pathways evaluated for this zone include volatilization from soil to indoor air and leaching to groundwater and surface water. Most receptors will not have direct exposure to this soil. However, some construction workers may be involved in excavation activities below the surficial soil zone and subsurface soil removed during excavation may be deposited at the surface allowing greater exposure. This possibility must be considered during the development of the CSM.

To test for indoor inhalation of vapors from subsurface soils, collect soil vapor samples..  Analyzing soil for a vapor intrusion should be conducted in areas directly overlying residual soil contamination as well as contamination adjacent to existing structures to delineate the exposure areas so better decisions can be made regarding AULs. More information on evaluation of the vapor intrusion exposure pathway is provided in EPA’s Technical Guide for Addressing Petroleum Vapor Intrusion at Leaking Underground Storage Tank Sites (Appendix A).

To test for exposure to a construction or utility worker during excavation activities, soil samples must be collected from both surface and subsurface soil zones to depths where construction- related activities are likely to occur.

To test for leaching of petroleum COIs in soil to groundwater, it is critical to determine the thickness of the contaminated soil zone, the distance from the bottom of the contaminated zone to the water table, if any, and representative concentrations of petroleum COIs within the contaminated zone.

Soil sampling must be done in accordance with the following guidelines and procedures:

          - Samples must be collected from the source area(s). Samples must be collected to determine the full horizontal and vertical extent of soil contamination. Sampling must strive to characterize any horizontal or vertical stratigraphic variation at the site that could impact petroleum COI fate and transport. To fully characterize the vertical extent of contamination, borings should be extended to the water table and features such as the capillary fringe and any smear zone, if encountered, must be characterized. Vertical sampling intervals generally should not be greater than 5 feet.

          - Soil borings must be logged and samples for laboratory analyses collected in accordance with current industry practice.

          - All samples must be adequately preserved according to requirements of the laboratory analyses and analyzed within holding times required by each method. Sample analyses must be conducted in accordance with current, EPA Office of Solid Waste and Emergency Response SW-846 Methods, or other accepted methods.

          - Adequate QA/QC procedures must be used to ensure sample quality and integrity. Section 4.2.2 contains additional information on QA/QC considerations.

          - All sampling equipment must be decontaminated using current state of industry practice such as described in ASTM D-5088, Practice for Decontamination of Field Equipment Used At Nonradioactive Waste Sites.

          - Abandonment of boreholes that extend to the water table, which are considered wells by IDWR, must follow IDWR abandonment procedures (IDAPA 37.03.09.16). Appropriate methodology for abandoning other boreholes is described in detail in the Standard Guide for Decommissioning Ground Water Wells, Vadose Zone Monitoring Devices, Boreholes, and Other Devices for Environmental Activities, Standard Guide D5299.

        - Subsurface Soil Vapor Data

At sites where petroleum COIs in soil or groundwater are volatile and there is concern about potential indoor inhalation of vapors, it is necessary to assess soil vapor concentrations. For details about these samples, refer to EPA’s Technical Guide for Addressing Petroleum Vapor Intrusion at Leaking Underground Storage Tank Sites (Appendix A).

        - Logging of Soil Boreholes

Each soil boring must be logged to record depths correlating with changes in lithology (with lithologic descriptions), soil vapor (e.g., photo-ionization detector) analyses, occurrence of groundwater, total depth, visual and olfactory observations, and any other pertinent data.

When a monitoring well is installed, as-built diagrams with depth to groundwater and construction details with a continuous soil profile with detailed lithologic descriptions must be submitted for each well. Particular emphasis should be placed on characteristics that control chemical migration and distribution, such as zones of greater or lesser permeability, changes in lithology, correlation between soil vapor concentrations and different lithologic zones, obvious areas of soil discoloration, organic content, fractures, and other lithologic characteristics.

      - Distribution of Petroleum Chemicals of Interest in Groundwater

Adequate groundwater samples must be collected to delineate the extent of free product, dissolved contaminant plumes in all directions, maximum petroleum COI concentrations, and representative concentrations based on the CSM. This sampling should take into account knowledge of the soil source and the direction of groundwater flow.

        - Groundwater Sampling

If groundwater has been impacted, temporary sampling points may be used to screen levels of groundwater impacts and to assist in determining optimal locations of permanent monitoring wells. A sufficient number of monitoring wells must be installed, designed, and located properly to delineate the horizontal and the vertical extent of the groundwater plume, estimate the direction of groundwater flow, identify source areas, and to document petroleum COI migration and NAPL. The monitoring wells must be installed in accordance with current IDWR rules (IDAPA 37.03.09) and industry standards such as ASTM D 5092, Standard Practice for Design and Installation of Ground Water Monitoring Wells in Aquifers. EPA documents such as the Resource Conservation and Recovery Act (RCRA) technical guidance for groundwater monitoring are also useful references.

Well casing and screen materials must be properly selected. The screen interval length must be minimized to the extent possible but should be set at least 2 to 3 feet above the expected high water table and encompass the range of expected variation in water table depth. EPA recommends screen interval lengths of 10 to 15 feet except where specific monitoring objectives or site knowledge result in other requirements. Wells must be properly developed and gauged after installation. A site survey must be conducted to establish well casing elevations. Based on the groundwater elevations, groundwater flow direction and gradient must be determined and plotted on a map.

Groundwater samples must be collected in accordance with the following guidelines and procedures:

          - Monitoring wells must be purged of an adequate number of well volumes prior to collecting a sample Low-flow purging is recommended (see Appendix A).

          - Samples must be collected using EPA approved methods and equipment.

          - All samples must be adequately preserved according to the requirements of the laboratory analyses and analyzed within holding times required by each method.

          - Sample analyses must be conducted in accordance with EPA Office of Solid Waste and Emergency Response SW846 Methods, or other accepted methods.

          - Adequate QA/QC procedures must be used to ensure sample quality and integrity. See section 4.2.2 for additional information on QA/QC considerations. All sampling equipment must be decontaminated using current state of industry practice such as

described in ASTM D-5088, Practice for Decontamination of Field Equipment Used At Nonradioactive Waste Sites.

          - If the plume is not delineated in all directions, locations of new monitoring wells must be chosen based on groundwater flow direction and location of the soil source area.

      - Surface Water and Sediment Sampling

Appropriate samples must be collected when petroleum COI migration is known or suspected to have impacted surface water. Water samples must be collected from upstream and downstream of a groundwater discharge point. Sediment samples must be collected if the CSM indicates a potential for releases to surface water.

In places where a groundwater plume may discharge into a stream, it is necessary to estimate the discharge concentration into the stream. This can be achieved by installing one or more monitoring wells or temporary wells within the plume adjacent to the stream.

#### Exposure Assessment

The goals of the exposure assessment portion of the RE are to:

- Characterize the physical setting,

- Identify potentially exposed populations,

- Identify complete or potentially complete exposure pathways,

- Estimate exposure concentrations, and

- Estimate petroleum chemical intakes.

The exposure assessment uses much of the data collected in support of and assumptions contained in the CSM (described earlier in section 4.1). A graphical display of the CSM like the sample shown in Figure 2 is helpful for assessing exposure.

###### Physical Setting

Characterization of the physical setting for exposure assessment identifies those site factors which may influence the fate and transport of chemicals from the source to a receptor. These factors include climate, meteorology, geologic and hydrogeologic setting, soil types, biodegradation potential, and surface water hydrology. When these factors are considered in the context of the media from which petroleum COIs are released (surficial soils, subsurface soils, groundwater, surface water, etc.), the transport mechanisms for the petroleum COIs from each media (leaching, groundwater transport, volatilization, etc.), and the chemical concentrations and distribution of the petroleum COIs in each media, the potential for a chemical to reach a receptor can be assessed. A more thorough discussion of the data needs related to description of the physical setting can be found in section 4.1 (Conceptual Site Model).

OFF SITE

ON SITE

FUTURE LAND USE

OFF SITE

ON SITE

CURRENT LAND USE

Risk Evaluation Manual for Petroleum Releases—August 2018

IMPACTED

TRANSPORT

EXPOSURE ROUTES

POTENTIAL RECEPTORS

GROUNDwATER

VOLATILIZATION

CONSTRUCTION ACTIVITY- WIND EROSION-DISPERSION- VOLATILIZATION

VOLATILIZATION

SUBSURFACE SOIL

SURFICIAL SOIL

WIND EROSION-DISPERSION- VOLATILIZATION

|  | Resident | Indoor Worker | Construction Worker |  | Resident | Indoor Worker | Construction Worker |  | Resident | Indoor Worker | Construction Worker |  | Resident | Indoor Worker | Construction Worker |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inhalation of Vapors/Particulates- Dermal Contact-Ingestion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Leaching to Groundwater |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Indoor Inhalation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inhalation of Vapors/Particulates- Dermal Contact and Ingestion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Leaching to Groundwater |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Indoor Inhalation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ingestion |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

YES NO

| FREE PRODUCT? |  |  |
| --- | --- | --- |
| UTILITIES THREATENED? |  |  |
| POTENTIAL SURFACE WATER IMPACT? |  |  |
| ECOLOGICAL RECEPTORS? |  |  |

Figure 2. Sample Graphical Display of a Conceptual Site Model

###### Potentially Exposed Populations/Receptors

Potential exposure populations/receptors must be identified from the information contained in section 4.1 (Conceptual Site Model).

###### Exposure Pathways

A receptor comes in contact with petroleum COIs through a complete exposure pathway. For an exposure pathway to be complete, all of the following must be present:

- a source of petroleum,

- a mechanism by which the petroleum is released,

- a transport medium through which petroleum travels from the point of release to the receptor location,

- a ROE by which the petroleum chemical enters the receptor’s body (ingestion, inhalation, or dermal contact) and

- a potential receptor.

If the migration of petroleum chemicals to a receptor or contact by a receptor is not possible (e.g., due to formal engineering controls such as a paved site that will prevent human contact with petroleum-contaminated soil) under current and future land use conditions, the site-specific petroleum COI concentrations may not pose a risk but it may require that limitations be placed on activities and land uses that are allowed on the property.

DEQ has identified the most commonly encountered exposure pathways for various environmental media for which an evaluation must be conducted. These exposure pathways are discussed below. At sites where receptors, exposure pathways, or ROE other than those discussed below are important, the owner/operator must identify them and discuss their quantitative evaluation with DEQ. In some cases it may be determined that one or more of the exposure pathways are incomplete and therefore do not need to be quantitatively evaluated. Adequate justification for exclusion of these exposure pathways must be provided to and approved by DEQ.

      - Pathways for Surficial Soils (0–1 foot below ground surface)

Surficial soils are defined as soils extending from the surface to 1 foot below ground surface. For construction worker exposure, however, the applicable depth is the depth of excavation. The exposure pathways associated with impacted surficial soils include:

- Groundwater protection (leaching of petroleum chemicals from soil to groundwater with subsequent potential ingestion of groundwater),

- Surface water protection (leaching of petroleum chemicals from soil to groundwater with subsequent migration to a surface waterbody), and

- Ingestion of soil, outdoor inhalation of vapors and particulate emissions from soil, and dermal contact with soil.

      - Exposure Pathways for Subsurface Soils (1 foot below ground surface to the water table)

Subsurface soils are defined as soils extending from 1 foot below the ground surface to the water table. The exposure pathways associated with subsurface soils include:

- Indoor inhalation of vapor emissions from soil,

- Groundwater protection (leaching of petroleum chemicals from soil to groundwater with subsequent potential ingestion of groundwater), and

- Surface water protection (leaching of petroleum chemicals from soil to groundwater with subsequent migration to a surface waterbody).

      - Exposure Pathways for Groundwater

Potentially complete exposure pathways for impacted groundwater include:

- Indoor inhalation of vapor emissions from groundwater, and

- Current and/or future ingestion of water on or off site.

EPA’s Technical Guide for Addressing Petroleum Vapor Intrusion at Leaking Underground Storage Tank Sites (Appendix A) describes in detail the process used to evaluate potential exposure via indoor inhalation of vapors from soil and groundwater.

      - Exposure Pathways for Surface Water and Sediments

Depending on the beneficial use designation of impacted surface waters, complete expousre pathways for surface water include:

- Intentional ingestion of surface water and ingestion of fish when surface water is used as a drinking water supply, and

- Ingestion of fish from surface waters designated for recreational use.

      - Other Exposure Pathways

Other complete or potentially complete exposure pathways, such as ingestion of produce grown in impacted soils, ingestion of fish, contact with contaminated sediments, or use of groundwater for irrigation purposes, should be evaluated on a case-by-case basis. The owner/operator should contact DEQ for further guidance.

###### Exposure Point Concentrations

Exposure point concentrations are the average petroleum chemical concentrations to which receptors could be exposed over a specified duration within a specified geographical area. The geographical area about which a receptor moves and within which a receptor contacts contaminated media during the specified exposure duration is termed an exposure unit. The exposure unit of all receptors must be considered and described. The exposure unit, or spatial area over which a given receptor is likely to be exposed, must be established for on-site receptors as well as any off-site impacted or potentially impacted receptors and for each exposure pathway or ROE. The same site may have different exposure units for current and future use scenarios.

A detailed discussion of the estimation of exposure point concentrations, the use of exposure units, and various methods available to estimate the exposure point concentrations is provided in Appendix C. This appendix describes the sources of uncertainty which affect the estimation of exposure point concentrations, strategies to reduce this uncertainty, and methodologies to evaluate data quality. It also provides guidance on the selection of appropriate data for various ROE and appropriate statistical methods for analysis of available data.

The RE report should clearly identify specific data and methods used to estimate the exposure point concentrations, and provide a rationale for the method and data used. The following information should be provided in a table:

- Media

- ROE/exposure pathway

- Receptor

- Data used

- Method of estimation

Table 3 shows how such a table might be constructed with examples of the type of information that would be supplied.

Table 3. Example Table Describing Derivation of Exposure Point Concentrations

| Media | Route of Exposure | Receptor | Data Obtained From | Method |
| --- | --- | --- | --- | --- |
| Surficial Soil | Direct Contact | Construction Worker | Unpaved Area | Average |
| Subsurface Soil | Indoor Inhalation | Commercial Worker | Building Footprint | Maximum |
| Groundwater | Ingestion | Residential | Source Area | Upper Confidence Limit |
| Soil Vapor | Indoor Inhalation | Commercial Worker | Sub-slab and near soil gas samples | Maximum |

As presented in Table 3, an exposure point concentration is estimated for each complete ROE. Various methods available to estimate the exposure point concentrations are discussed in Appendix C. Use of the maximum concentration as the exposure point concentration is most conservative and also the easiest to calculate when compared with other ways of calculating exposure point concentration (average, area-weighted average, upper limit of the confidence interval around the mean). Thus, if the risk that is calculated using the maximum concentration is acceptable, considerable computational effort can be avoided.

      - Soil

Exposure point soil concentrations used to evaluate protection of the groundwater exposure pathway should be calculated based on soil data collected within the source area only.

Soil data from the most recent comprehensive investigation  must be used. The use of older soil data , while discouraged, may be

acceptable if site conditions that would result in an increase in petroleum COI concentrations (such as a new release) have not changed. If a new release has or may have occurred, additional soil assessment activities that adequately characterize current conditions must be performed. New data collection efforts may be documented in a work plan and submitted for approval by DEQ.

      - Groundwater

Based on the CSM, several exposure point groundwater concentrations at a site may have to be estimated. These could include exposure point concentrations for the source area, at compliance locations, for protection of indoor inhalation on-site and off-site, and for off-site areas.

Estimation of these concentrations will often require the use of fate and transport models, discussed in section 4.3.5.

In order to account for site-specific conditions such as source characteristics in relation to the depth to groundwater and the age of the release, it is typically necessary to perform multiple sampling events over time to ensure that the potential for the release to impact groundwater has been accounted for and that groundwater concentrations are representative. However, if these factors can be accounted for, it may be possible to perform an RE using data from one sampling event. If temporal data indicate significant variability, additional sampling events may be required prior to conducting the RE. Subsequent to RE, DEQ may require additional monitoring data to be collected. If groundwater data less than 12 months old is unavailable, current data will be necessary for the RE. The owner/operator may elect to document data collection objectives in a work plan submitted to DEQ for review and approval.

      - Point of Exposure and Point of Compliance

The point of exposure (POE) is the location where a receptor has or could come in contact with petroleum COIs under current and future conditions. A separate POE is associated with each complete exposure pathway-receptor combination identified in the CSM. For direct exposure pathways, the POE is located at the source of the petroleum COIs. For example, for the ingestion of surface soil, the POE is at the same location as the soil source. For indirect exposure pathways, the POE and the source of petroleum COIs are physically separate. For example, for the case of indoor inhalation of vapors from soil, the POE is inside the building (the breathing space) whereas the source is the soil below and adjacent to the building. The POE location for the protection of the groundwater for groundwater ingestion is discussed in section 4.3.3.3.

A point of compliance (POC) is a location where concentrations are measured to determine if compliance with cleanup criteriahas been achieved. Concentration measurements at the POC may be in any media (e.g., soil, groundwater, soil vapor, etc.). The location of a POC may be identical to the POE or may be located between the source and the POE. In the latter case, the target concentrations at the POC are back-calculated to ensure that the concentrations at the POE do not exceed the target concentration at the POE. For example, for the protection of the groundwater exposure pathway, the POC well may serve as a sentry well for protection of the POE. The calculated target levels for the POC are then compared to measured concentrations. POC locations may be predetermined based on program-specific requirements. Most sites, particularly those involving groundwater impacts, will have multiple POC locations.

      - Estimating Point of Compliance Well Concentrations

As a part of the RE process, it is necessary to designate and monitor appropriate POCs. The POCs, located on-site and/or off-site, are used to provide additional assurance that approved concentrations at a selected POE are not exceeded. Monitoring of POC locations is required, and data obtained are compared with approved remediation standards. Monitoring of POC locations must be continued until the concentrations stabilize below approved levels. Concentrations at the POC may also be used to determine the need for additional remedial activities.

###### Fate and Transport and Exposure Models

Different types of models or equations, uptake equations, risk equations, and fate and transport models, are required to calculate chemical intake, risk, exposure point concentrations, and target concentrations. Fate and transport models simulate the transfer of chemicals from one media (such as soil) to another (air). They also are used to simulate the movement of chemicals within a given media, such as the transport of chemicals in groundwater from a source area to a POE. They allow the estimation of concentrations at points distant from the source. EPA’s default exposure parameters are on their website in both the Generic Tables section and in the RSL Calculator User’s Guide. Alternative models may be used if justified.

      - Fate and Transport Parameters

Fate and transport parameters are necessary in the models used to estimate exposure point concentrations and compliance point concentrations for indirect exposure pathways such as vapor intrusion and protection of groundwater. These parameters characterize the physical site properties such as the dilution attenuation factor, soil porosity, and infiltration rate. For RE, a combination of site-specific and default values for these parameters is typically used. However, the value of each parameter used, whether site-specific or default, must be justified based on site-specific conditions. Where fate and transport models other than those selected by EPA are used, the specific fate and transport parameters required to calculate exposure point concentrations and risk may vary and have specific data collection requirements.

      - Physical and Chemical Properties of the Petroleum Chemicals of Interest

The development of target levels requires the selection of values for the physical and chemical properties of petroleum COIs. Values of these parameters are listed on EPA’s website in both the Generic Tables section and in the User’s Guide. To use different values, the owner/operator must provide sufficient justification to DEQ. The use of different values will be allowed only with prior DEQ approval.

###### Estimation of Chemical Intake

Risk evaluation requires quantifying the magnitude, frequency, and duration of exposure for the receptor populations and exposure pathways selected for analysis. Oral exposures are quantified on a dose per unit body weight basis while inhalation exposure is based on concentration in air.

      - Exposure Factors

In order to determine receptor and exposure pathway-specific intake estimates, it is necessary to select values for a number of exposure parameters in the equations used to calculate intake. When determining these values, it is often necessary to make assumptions. EPA’s default exposure parameters are on their website in both the Generic Tables section and in the RSL Calculator User’s Guide. Alternative models may be used if justified.

Exposure factors describe the physiological and behavioral characteristics of the receptor. These factors include the following:

- Water ingestion rate,

- Body weight,

- Exposure duration for each ROE,

- Exposure frequency,

- Soil ingestion rate,

- Exposure times for indoor/outdoor inhalation,

- Dermal relative absorption factor,

- Skin surface area for dermal contact with soil,

- Soil-skin adherence factor, and

- Oral relative absorption factor.

#### Toxicity Assessment

###### Chemical-Specific Toxicological Factors

The toxicity of chemicals with carcinogenic adverse health effects is quantified using cancer slope factors (CSF) for oral and dermal ROE, or inhalation unit risk (IUR) for the inhalation route. A CSF is an upper-bound estimate of the probability of a response (developing cancer) per unit intake of a chemical over a lifetime. The IUR is the upper-bound excess cancer risk estimated to result from continuous exposure to a chemical at a concentration of 1 µg/m3 in air.

For chemicals that cause noncarcinogenic health effects, toxicity is typically quantified by reference doses (RfD) for oral and dermal ROE, and reference concentrations (RfC) for the inhalation ROE. The RfD is an estimate of a daily oral exposure to the human population (including sensitive subgroups) that is likely to be without risk of adverse health effects during a lifetime. Since RfDs are based on oral exposure, they are modified for use in dermal exposure assessment to take account of differences between gastrointestinal and dermal absorption. The RfC is an estimate of a continuous inhalation exposure to the human population (including sensitive subgroups) that is likely to be without an appreciable risk of adverse health effects over a lifetime.

The primary source of information for toxicity factors for the petroleum COI is the EPA RSL Generic Tables.

.

###### Carcinogens with a Mutagenic Mode of Action

Current EPA guidance for risk assessment of carcinogens addresses the potential for increased susceptibility to cancer from exposure to carcinogens that occurs early in life (EPA Guidelines for Carcinogen Risk Assessment, 2005) . When information is available to establish the mode of action of a chemical for early-life and adult exposures, it may be appropriate to develop separate risk estimates for childhood exposure and exposure occurring later in life. When the mode of action cannot be established, risk estimates are based on a lifetime daily average exposure without adjustment. Currently, there is sufficient information to develop separate risk estimates only for chemicals having a mutagenic mode of action.

Specifically, when data indicate a mutagenic mode of action for a chemical, meaning that the chemical interacts directly with DNA, available studies indicate higher cancer risk for a given exposure when it occurs early in life compared to a similar exposure in adulthood. Among the petroleum chemicals of interest, the following carcinogenic chemicals are considered mutagens:

- Benz(a)anthracene

- Benzo(a)pyrene

- Benzo(b)fluoranthene

- Benzo(k)fluoranthene

- Chrysene

EPA’s RSL Calculator User’s Guide and Frequent Questions addresses how adjustments are made to risk estimates for resident soil, resident tap water, resident air, recreator soil, and recreator surface water.

The effect of the adjustments will be a somewhat higher estimate of risk for a given exposure (and thus somewhat lower screening levels) compared to risk estimates without age-specific adjustment. Calculated risk estimates for residential exposure will incorporate these adjustments, but nonresidential risk estimates will not, as they are based on adult exposure.

#### Risk Characterization

The process of risk characterization consists of combining the intake estimates developed through the exposure assessment and toxicity information for the petroleum COIs and calculating carcinogenic risk and noncarcinogenic hazard. Calculations are made for each exposure pathway-receptor combination and are summed to calculate a cumulative risk and a hazard index (HI) for each receptor. These risk results are then presented and discussed in a risk summary and are compared to acceptable target risk levels.

Risk and hazard calculations are completed using a standard set of risk equations provided by the EPA in their RSL Calculator User’s Guide). Estimates made using these equations should be presented using one significant figure, although all digits should be retained for the supporting calculations, such as when summing risks from multiple petroleum chemicals or exposure pathways, in order to minimize rounding errors. For example, calculated cancer risks would be presented as 1 x 10-4 and HQs or HIs would be expressed as 2, 0.7, or 0.03.

This risk-based decision-making process specifies the acceptable risk levels for both carcinogenic and noncarcinogenic health effects. For carcinogenic effects, risk is quantified using the individual excess lifetime cancer risk (IELCR) that represents an increase over background in the probability of an individual developing cancer due to exposure to specific petroleum COI through a specific exposure pathway. Since a receptor may be exposed to multiple petroleum chemicals through multiple ROEs, the acceptable risk level should account for the effect of simultaneous exposure to multiple petroleum chemicals and multiple ROEs.

Overall site carcinogenic risk is calculated as the sum of the individual chemical risk estimates. The probability of a receptor getting cancer is thus the sum of the probabilities of getting cancer from exposure to each petroleum chemical through each exposure route. In this calculation there is an assumption of independence of action of the individual chemicals. If the carcinogenic actions of different petroleum chemicals are not independent there is the possibility of synergistic interaction in which case risk may be underestimated. However, information is generally not available to address this possibility.

For noncarcinogenic effects, risk is quantified using a HQ that represents the ratio of the estimated dose of a petroleum chemical for a ROE to the reference or allowable dose. The hazard quotient is calculated in the RSL calculator site-specific mode by the following formula:

HQ = (C × THQ) / RSL

where:

HQ =  unitless ratio of exposure concentration to reference concentration where a value less than or equal to unity indicates the exposure is not likely to result in adverse health effects
C = Concentration entered by the user 
THQ = Target Hazard Quotient provided by the user
RSL = Regional Screening Level, determined by the values entered by the user

When a

receptor is exposed to multiple chemicals and multiple ROEs, individual HQs are added together to estimate the HI.

For site-specific REs, the following target risk criteria must be satisfied at the POE:

Acceptable Target Risk Level: For combined exposure to all carcinogens and ROE, IELCR must be less than or equal to 1x10-5 for a receptor at a reasonable maximum exposure.

Acceptable HI: The summation of HQs for all petroleum chemicals that have noncarcinogenic health effects and ROE must be less than or equal to 1.

The screening level is the lowest of the media-specific (soil, soil vapor, and groundwater) risk-based concentrations for the following exposure pathways: The default screening levels are based on a THQ of 0.1 and assumes multiple COIs with the same toxic endpoint. Almost all petroleum sites will include multiple COIs with the same toxic endpoint. Each petroleum COI and its associated toxic endpoint is presented in Table 1. The rationale for using a THQ of 0.1 for screening is that when multiple contaminants of concern are present at a site or one or more are present in multiple exposure media, the total hazard index could exceed 1.0 if each were screened at the THQ of 1.0. If you are screening only one contaminant, a THQ of 1.0 may be used but DEQ should be consulted.

The target risk level and HI must be met for each current and potential future receptor at the site. If the initial calculated HI exceeds 1 further evaluation, including analysis of mode of action and toxic endpoint (see Table 1) for each petroleum chemical, can be completed. If this analysis indicates sufficient independence of target organs and mode of action for different chemicals, it may be acceptable to present separate HQs, and/or one or more HI for subsets of the chemicals. This kind of analysis should be performed by a toxicologist.

In addition to the target risk levels, DEQ requires that MCLs, or comparable risk-based values for groundwater ingestion, be met at the POE when the groundwater ingestion pathway is complete. If the groundwater ingestion pathway is incomplete, the leaching to groundwater and/or groundwater vapor intrusion pathways may still be complete.  Similarly, for impacts to surface water bodies, calculated target concentrations must be met.

The risk summary should include an analysis and discussion of the sources of uncertainty so that the risk estimates can be placed in perspective. The uncertainty analysis can be qualitative or quantitative in nature. It is more important during the analysis to identify the key site-related parameters and assumptions in the CSM that contribute the most to the uncertainty than to precisely quantify the degree of uncertainty. It is typically the case that there is insufficient information available at a given site to be able to reliably quantify the uncertainty in a given risk estimate, such as through conducting a probabilistic risk assessment or sensitivity analysis. The typical sources of uncertainty that are included in the analysis include the petroleum COIs selected and their estimated site concentrations, the toxicity values used, the fate and transport models used to estimate exposure point concentrations, the exposure parameter values used in the exposure assessment, and the summing of risks from multiple exposure pathways or petroleum chemicals.

#### Risk-Based Target Levels

As described in the Risk Characterization section 4.5, if the calculated risk or hazard for all petroleum COIs and complete ROE for identified potential receptors exceeds the Acceptable Target Risk or Hazard Levels and corrective action is required, then remediation standards to achieve these targets should be calculated using EPA’s RSL and VISL calculators by gathering site-specific data and updating the default parameters, if appropriate. The RSL and VISL calculators will calculate a risk-based screening level that can be used as the remediation standards. Ingestion of water is not included in this calculation (see section 4.6.1.1). EPA’s calculators will use chemical-specific toxicological factors, receptor-specific exposure factors, fate and transport parameters, physical and chemical properties of the petroleum COIs, and mathematical models to calculate remediation standards but other models may be used with DEQ approval.

Target concentrations should apportion the acceptable target cumulative risk and HI equally among the different chemical-exposure pathway combinations such that those petroleum chemicals which contribute the greatest proportion of the total risk have the most stringent remediation standards. The primary goal is to develop target concentrations in all media such that the acceptable target risk levels and hazard levels are met.

If a petroleum COI has toxicity that is based on both carcinogenic and noncarcinogenic effects, the applicable remediation standards for that chemical should be the lower of the two calculated allowable concentrations.

###### Developing Remediation Standards Concentrations for Groundwater and Surface Water Protection

For groundwater and surface water protection, target concentrations are calculated using somewhat different procedures, described below.

      - Groundwater Protection

The remediation standards for groundwater ingestion are equivalent to the federal MCLs or a risk-based calculated equivalent. MCLs are health-protective target concentrations promulgated by the EPA and adopted by the state of Idaho for the protection of drinking water and specified groundwater resources. For any petroleum COI that has an MCL, the remediation standards for groundwater ingestion is equal to the MCL. For any petroleum COI that does not have an MCL, the risk-based equivalent level is calculated as residential tap water (see section 3.1).

The remediation standards will apply at the POE. The POE will be the down-gradient property boundary, as it existed when the release occurred, or the nearest down-gradient location where a well exists or could be reasonably placed. Depending on site-specific conditions it is also possible to have multiple points of exposure.

      - Surface Water Protection

Potential impacts to streams and other surface waterbodies from a petroleum release must be evaluated and surface water quality must be protected as per IDAPA 58.01.02 (Water Quality Standards and Wastewater Treatment Requirements). The primary receptors and ROE for potentially impacted surface waters evaluated in the RE process are described in

section 4.6.1.2.1. Other ROE, such as contact with contaminated sediments or overland flow discharge, are evaluated on a case-by-case basis.

This section describes the evaluation of potential impacts to surface water via discharge of impacted groundwater to a surface waterbody. A schematic illustrating this exposure pathway is shown in Figure 3. This figure shows the relevant compliance and exposure locations and associated concentrations. Within the RE process, protection of surface waterbodies requires the owner/operator to determine or calculate the applicable surface water standards at the point where groundwater discharges into a surface waterbody (Cswpoe). Once the appropriate surface water standard is determined, compliance with the standard may be achieved in a number of ways. These include measuring surface water concentrations at the point of groundwater discharge, measuring groundwater concentrations at the point of discharge into the surface waterbody, or determining appropriate alternate concentrations in other media and at POC locations. Selection of alternate locations may be most appropriate for those sites where contamination has not yet reached a surface waterbody. Alternate concentrations (or remediation standards) and POC locations can include:

- Source area soils (Csoil), or

- Compliance points in groundwater at different distances between the surface water and the source (Ccw) other than the point where groundwater discharges into the surface waterbody.

Qsw , Csu

Groundwater Plume

Csoil

Cswpoe

Groundwater Source (Cgws)

Qgw

Lp

Direction of Groundwater Flow

Xs

Surface Water

PLAN VIEW

Ground Surface

Csoil

Ccw

Soil Source

Vadose Zone

Stream

Water Table

Qgw

GW Discharge

D

p

Direction of Groundwater Flow

Cgws

Groundwater Plume

CROSS-SECTIONAL VIEW

Cswpoe

Explanation of Symbols

Qsw =	Stream flow upstream of the point of groundwater discharge Csu =	Concentration upstream of the groundwater discharge

Qgw =	Impacted groundwater discharge into the stream

Csw =	Allowable downstream concentration after uniform mixing

Cswpoe =	 Allowable concentration at the point of groundwater discharge to the stream Cgws =	 Allowable concentration in the groundwater at the edge of the soil source Csoil =	Allowable soil concentration at the source protective of the stream

Ccw =	Allowable soil concentration in groundwater at different distances between the stream and the source

Lp =	Width of groundwater plume discharging to the stream

Dp =	Thickness of groundwater plume discharging to the stream

Xs =	Distance from the downgradient edge of the groundwater source to the stream

Figure 3. Schematic of Leachate Migration from the Soil Source to the Stream

        - Surface Water Quality Standards

The allowable concentration at the point of groundwater discharge into the surface water (Cswpoe), or the surface water quality standard, depends on the beneficial use designations of the surface waterbody as per IDAPA 58.01.02.100 and criteria assigned to protect those beneficial uses (IDAPA 58.01.02.200-250).

Beneficial uses include:

          - Aquatic life: Cold water, salmonid spawning, seasonal cold water, warm water, or modified.

          - Recreation: Primary contact or secondary contact.

          - Water supply: Domestic, agricultural, or industrial.

Each beneficial use has associated numerical and narrative criteria. Numerical criteria are specified values that are not to be exceeded. For narrative criteria involving toxic substances, concentrations are not specified, but must be low enough to ensure that designated beneficial uses are not impaired. When necessary, development of numeric criteria for toxic substances is governed by IDAPA 58.01.02.210.05.

The allowable concentrations for certain toxic substances associated with these beneficial uses are tabulated in IDAPA 58.01.02.210.01.b .

###### Determination of Applicable Risk Based Levels

The calculated allowable remediation standards for protection of groundwater and/or surface water and those calculated for other potentially complete exposure pathways are compared to determine which media-specific concentrations will guide the cleanup. Typically, the lower of these concentrations will be selected.

Using groundwater as an example, target POE concentrations will be based on the most limiting concentrations determined from evaluation of the complete or potentially complete exposure pathways in addition to the groundwater ingestion pathway. These exposure pathways may include indoor inhalation of vapors volatilized from groundwater, groundwater impacts to surface water, impacts to deeper groundwater systems with the potential for use as drinking water supplies, and also incidental ingestion of and dermal exposure from irrigation water. Also included is potential impairment of other beneficial uses of groundwater such as for agricultural or industrial water supplies.

The applicable POE will be based on the location of the plume (on-site vs. off-site) and which exposure pathway(s) and receptors are limiting with respect to risk and allowable groundwater concentration. In many cases, particularly if a plume has migrated off-site, there may be multiple POE and exposure pathways that will need to be considered.

For example, a petroleum-sourced volatile organic compound plume may have migrated off-site into a residential area and have the potential to discharge to a down-gradient stream.

Source area groundwater concentrations on-site must be controlled such that concentrations at the POE where groundwater discharges to the stream meet applicable criteria. In addition, unless vapor intrusion has been separately addressed, such as through the use of soil vapor, residential

vapor intrusion criteria in groundwater would need to be met at the down-gradient boundary of the source property and commercial criteria for on-site POE.

#### Deliverables

The primary deliverables associated with conducting a site-specific RE typically include an optional work plan for the collection and evaluation of data, a data collection report, and the required RE report. In many cases, the results of a data collection effort are combined with the RE report. Data collected must follow DEQ’s quality management program, QAPP, and SOP or a DEQ-approved QAPP (see section 4.2 and Appendix A). If the results of the RE report indicate the need for corrective action, a CAP must also be developed and submitted. The contents of CAPs are discussed in Section 5. This section describes the contents of a work plan.

###### Work Plan

A work plan may be desirable where extensive data collection activities or significant departures from default exposure assumptions or modeling are being proposed.

As appropriate, a work plan to fill identified data gaps may be prepared and submitted to DEQ for review. The amount of detail to be included in the work plan will vary among sites. At sites where a considerable amount of data has already been collected, the work plan may be a brief letter indicating activities to be performed to fill in the data gaps. For a complex or large site, a very detailed work plan, including the CSM, data collection methodology, analysis methods, a data QAPP, and a health and safety plan may need to be developed.

A RE work plan should address each item discussed in sections 4.7.1.1 through 4.7.1.11 as appropriate to the purpose and goals of the RE.

      - Purpose and Goals

The purpose and goals of the work plan should be clearly stated. If the purpose is to fill data gaps, then a listing and description of the data gaps should be provided along with an explanation as to why they constitute data gaps and how the work proposed will fill those gaps.

If the purpose of the work plan is to propose use of alternative models, exposure factors, physical and chemical properties, toxicity factors, or methods of calculating remedial target concentrations, then a rationale should be provided as to why the alternatives proposed are needed. A short description of the scope and nature of the work proposed should be provided.

      - Site Background

This portion of the work plan may refer to documents previously submitted to DEQ; it is not necessary to repeat the entire site background description. Reference should be made to document(s) that contain a comprehensive chronology of site investigations.

      - Conceptual Site Model

A concise summary of the existing CSM should be provided. In those cases where the goal of the work plan is to revise a previously completed RE, then an CSM already exists that can be referenced rather than repeated. If validation or revision of a previous CSM is necessary, those

aspects of the current CSM that are the target of the work plan should be described. Refer to section 4.1 for the suggested content of the CSM.

      - Exposure Factors

If alternate exposure factors values are proposed in the work plan these factors must be justified and acceptable to DEQ. The default exposure factor values can be obtained from the RSL and VISL calculators and tables.

      - Physical and Chemical Properties

Owner/operators should use the default physical and chemical properties for the petroleum COIs .

      - Toxicity

Toxicity values can be obtained from the RSL and VISL calculators and tables.

      - Fate and Transport Models

If alternative models are being proposed, the work plan should include reasons for using alternative models; it should demonstrate that the proposed models will better simulate site- specific conditions, and that the sufficient site-specific data are available or will be collected to justify the use of the proposed model. Alternative models must be approved by DEQ before they are used.

      - Fate and Transport Parameters

Data collection may be proposed to collect site-specific values in lieu of using DEQ default values for fate and transport parameters  used in models for the RE. The parameters for which values are to be obtained should be specified and the methodology used to obtain those values described.

DEQ accepts the use of chemical-specific biological decay rates or attenuation factors in the fate and transport models. Use of decay rates in a RE must use values supported by site-specific information. Refer to Appendix H for data necessary to demonstrate occurrence of natural attenuation and methods to calculate decay rates. Examples of this type of information include, but are not limited to:

- Consistent stable or decreasing concentration (above the RSLs) trends in properly located and constructed monitoring wells, and

- Measurements of natural attenuation parameters that provide evidence of biodegradation.

      - Calculating Remediation Standards Concentrations

The process of calculating target concentrations (remediation standards) is described in section 4.6. If a method of apportioning risk and calculating remediation standards different than the DEQ

default method is desired, the proposed methodology and rationale for the choice should be described.

      - Methodology and Quality Control

Work plans in support of data collection efforts should describe the methodology that will be used to obtain the data and provide a clear description of the level of data quality required to meet the stated objectives, and how this data quality will be achieved and ensured.

      - Schedule and Deliverables

The optional work plan should include an overall project schedule and deliverables that will be submitted to DEQ. The schedule should include any agency meetings necessary during work plan implementation.

###### Implementing the Work Plan

Upon receipt of a work plan approval, the owner/operator should implement the work plan according to the schedule in the work plan. In case there are delays, it is the owner/operator’s duty to inform DEQ of the delay and revised schedule. If there were deviations in the work performed from that specified in the approved work plan, DEQ should be notified of the deviations and their impact on the goals and purposes of the work plan.

Upon completion of work specified in the work plan, the owner/operator should document the results and submit them to DEQ. If the purpose of the work plan included data collection, the data collection results can be summarized in a separate data collection report or incorporated into a revised RE report.

###### Risk Evaluation Report

The RE for a site must be clearly and concisely documented and submitted to DEQ for review. An example outline of a RE report is presented in Appendix D but, at a minimum, must include the items required in the Rule (58.01.24.200) such as a screening level evaluation and/or site-specific evaluation, and results of the risk evaluation. Within 30 days of receiving a CAP, DEQ must provide an approval, an approval with modifications, or a rejection (specifying reasons for the rejection) in writing. Provisions are made in the Rule (Section 200.04) for extending this timeframe.

# Section 5. Developing and Implementing Corrective Action Plans

Development and implementation of a CAP may be necessary if, after the screening level evaluation or site-specific RE, concentrations exceed screening levels or an unacceptable risk is identified. The owner/operator must then develop cleanup criteria (also referred to as remediation standards) and create a CAP to achieve those standards or to ensure that the remaining risk is acceptable. The risk management strategy described in the CAP depends on the results of the RE (which exposure pathways, petroleum chemicals, and media are responsible for the unacceptable risk) and other circumstances unique to the site.

#### Contents of a Corrective Action Plan

As required in the Rule, the CAP must include:

    - A description of remediation standards selected, points of exposure, and points of compliance,

    - A description of the remedial strategy and actions to be taken to achieve the remediation standards,

    - Current and future land use and use of groundwater and surface water both on-site and off-site,

    - AULs that will be required as part of the remedial strategy and the associated environmental covenants,

    - Estimated timeline for completion of remedial actions,

    - The monitoring plan to gauge effectiveness of the remedial strategy, and

    - A description of practical quantitation limits and background concentrations as they apply to the remediation.

Additional elements that should also be a part of the plan include:

    - A brief site description,

    - A brief description of the RE approved by DEQ, including assumptions used for receptors and land use, and a list of approved remediation standards for specific exposure pathways, media, chemicals, and specific areas of the site (if applicable),

    - A clear description of reasons a CAP is needed,

    - An identification of the area(s) to be managed under the CAP,

    - A description of the strategy selected to achieve cleanup goals, including a comparison and evaluation of remedial alternatives considered in development of the overall strategy. A rationale for the strategy selected should be presented; any data collected in support of or to determine the feasibility of a particular remedial measure such as pilot testing, should be presented,

    - A description of what QA/QC measures will be implemented to ensure that data collected in support of remedial decision making is of adequate quality,

    - A description of project closure requirements, and

    - A reporting schedule to document progress of the corrective action.

#### Implementing a Corrective Action Plan

The CAP should not be implemented until approved by DEQ. Within 30 days of receiving a CAP, DEQ must provide an approval, an approval with modifications, or a rejection (specifying reasons for the rejection) in writing. Provisions are made in the Rule (Section 200.04) for extending this timeframe.

While DEQ is reviewing the CAP, implementation of interim corrective action measures may take place. However, as provided in IDAPA 58.01.02.852.06.c, owner/operators may, in the interest of minimizing environmental contamination and promoting more effective cleanup, begin cleanup of soil, surface water, and groundwater before the CAP is approved provided that they fulfill certain conditions including:

    - Notifying DEQ of their intention to begin cleanup;

    - Compliance with any conditions imposed by DEQ, including halting cleanup or mitigating adverse consequences from cleanup activities; and

    - Incorporation of these self-initiated cleanup measures in the CAP that is submitted to DEQ for approval.

Typical interim corrective actions include excavating and disposing contaminated soil, removing free product, extracting soil vapor, and pumping and treating groundwater.

Upon approval of the CAP, the owner/operator must implement the plan according to the proposed schedule. All performance data should be submitted to DEQ in a timely manner. This data should be carefully evaluated by both the owner/operator and DEQ to determine if the CAP is progressing as anticipated. Major deviations in schedule or plan implementation should be communicated to DEQ for review and approval well in advance of proposed implementation, along with recommended modifications if necessary. The specific modifications that may be needed will vary from site to site.

When the CAP has been successfully implemented, the owner/operator should document these conditions in a completion report along with a petition to DEQ for site closure.

# Section 6. Activity and Use Limitations

An AUL is a restriction or obligation, with respect to real property, that is created by an Environmental Covenant pursuant to the Uniform Environmental Covenants Act (UECA), Chapter 30, Title 55, Idaho Code.

The purpose of AULs is to reduce the potential for exposure to contaminants at a release site. AULs are also commonly referred to as “institutional controls.” AULs are typically proposed to DEQ for approval through the RE process. Non-remedial land use restrictions (e.g., local zoning ordinances) may reinforce the sustainability of AULs; however, they are not considered sufficient by themselves. These types of non-remedial land use restrictions, such as zoning ordinances, may change or be removed without DEQ approval and therefore are not approved by DEQ as part of a CAP. However, non-remedial land use restrictions may be relevant to evaluating current and reasonably likely future use and exposure scenarios at a site.

#### Environmental Covenants

Owner/operators may propose an Environmental Covenant as part of the CAP which will include the AUL language. Any proposed Environmental Covenant must be consistent with UECA. Each proposed Environmental Covenant is reviewed and evaluated on a site specific basis.

Following is a non-exclusive list of the types of activity and use limitations that may be included in an Environmental Covenant:

    - Restrictions on Water Use:

      - Prohibit use

      - Limit use

      - Monitor use

      - Report use

      - Operation and maintenance procedures for physical controls and devices

      - Prohibit well installation and operation

      - Abandon an existing well.

    - Restrictions on Land Use:

      - Prohibit, or require DEQ approval for, disturbance of soil, cap, or vegetation

      - Limit activities and land use

      - Limit structures and buildings.

    - Actions Regarding Access:

      - Right of Access necessary to implement the activity and use limitation

      - Limit or deny public access.

    - Obligations to perform certain actions in order to maintain the effectiveness of an approved corrective action. For example, this may involve the need to do periodic inspections and maintenance in order to ensure that a cover or barrier retains its primary

function. In this case there might be a specific reference in the Environmental Covenant to an Operations and Maintenance Plan, developed as part of the corrective action.

DEQ has an Environmental Covenant template on its website which list example restrictions that owners/operators can use when proposing activity and use limitations as part of a CAP. The owner/operator can propose an environmental covenant consistent with UECA for consideration and approval by DEQ.

#### Specific Application of Activity and Use Limitations at Petroleum Release Sites

Section 600.01 of the Rule outlines the circumstances under which AULs will be included as part of a CAP. These include:

    - Where the assumed on-site land use for the purpose of the RE is not residential and either the maximum media concentrations exceed the screening levels or the calculated risk for a residential receptor is unacceptable

    - Where off-site groundwater concentrations exceed screening levels or risk-based concentrations (AUL would be required on off-site properties), and

    - Where DEQ determines, based on the proposed CAP, that such AULs are required to assure the continued protection of human health and the environment or the integrity of the cleanup action.

#### Use of Activity and Use Limitations at Impacted Neighboring Properties

Implementability and enforceability issues become more complicated when an owner/operator proposes use of AULs on impacted neighboring properties. It is the owner/operators’s obligation to negotiate language and reach agreement with both the impacted property owner and DEQ.

# Section 7. References

#### References Cited

ASTM, 1990, Practice for Decontamination of Field Equipment Used At Nonradioactive Waste Sites, Designation D-5088-90, American Society for Testing and Materials, West Conshohocken, Pennsylvania.

ASTM, 1995, Standard Practice for Design and Installation of Ground Water Monitoring Wells in Aquifers, Designation: D 5092-95, American Society for Testing and Materials, West Conshohocken, Pennsylvania.

ASTM, 1996, Standard Guide for Selection of Aquifer Test Method in Determining Hydraulic Properties by Well Techniques, Designation: D 4043-96, American Society for Testing and Materials, West Conshohocken, Pennsylvania.

ASTM, 1998, Standard Test Method for Laboratory Determination of Water (Moisture) Content of Soil and Rock by Mass, Method D2216-98, American Society for Testing and Materials, West Conshohocken, Pennsylvania.

ASTM, 1999, Standard Guide for Decommissioning Ground Water Wells, Vadose Zone Monitoring Devices, Boreholes, and Other Devices for Environmental Activities, Standard Guide D5299-99, American Society for Testing and Materials, West Conshohocken, Pennsylvania.

ASTM, 2000a, Standard Test Method for Density of Soil in Place by the Drive-Cylinder Method, Method D2937-00, American Society for Testing and Materials, West Conshohocken, Pennsylvania.

ASTM, 2000b. Standard Test Methods for Moisture, Ash, and Organic Matter of Peat and Other Organic Soils, Method D2974-00, American Society for Testing and Materials, West Conshohocken, Pennsylvania.

ASTM, 2002, Standard Test Method for Specific Gravity of Soil Solids by Water Pycnometer, Method D854-00, American Society for Testing and Materials, West Conshohocken, Pennsylvania.

Cowherd, C., Muleski, G., Englehart, P., and Gillete, D. 1985. Rapid Assessment of Exposure to Particulate Emissions from Surface Contamination. Prepared for EPA Office of Health and Environmental Assessment. EPA/600/8-85/002.

Domenico, P. A. and V.V. Palciauskas, 1982, Alternative Boundaries in Solid Waste Management, in Ground Water, volume 20, number 3, pages 303 - 311.

Domenico, P. A., 1990, Physical and Chemical Hydrogeology, John Wiley and Sons, New York, New York.

EPA, 1986, RCRA Ground-Water Monitoring Technical Enforcement Guidance Document Draft, OSWER-9950.1, Office of Solid Waste and Emergency Response, Washington, D.C.

EPA, 1992, RCRA Groundwater Monitoring: Draft Technical Guidance, Office of Solid Waste, Washington D.C., November 1992.

EPA, 1994, Air Emissions Models for Waste and Wastewater, EPA-453/R-94-080A, November, 1994, Office of Air Quality Planning and Standards, Research Triangle Park, North Carolina.

EPA, 1995, Guidance for Data Quality Assessment, External Working Draft, EPA QA/G-9, Quality Assurance Management Staff, Washington D.C.

EPA, 1996, Soil Screening Guidance: Technical Background Document, EPA/540/R-95-128, Office of Solid Waste and Emergency Response, Washington, D.C.

EPA, 1999, USEPA Contract Laboratory Program National Functional Guidelines for Organic Data Review, EPA/540/R-99/008, Office of Emergency and Remedial Response, Washington, D.C.

EPA, 2002a, Guidance on Environmental Data Verification and Data Validation, U.S. Environmental Protection Agency, Office of Environmental Information, Washington, D.C., EPA/240/R-02/004, November 2002.

EPA, 2002b, Ground-Water Sampling Guidelines for Superfund and RCRA Project Managers. Ground Water Forum Issue Paper. Technology Innovation Office. OSWER.EPA 542-S- 02-001.

EPA, 2003, User’s Guide for Evaluating Subsurface Vapor Intrusion into Buildings, Office of Emergency Response and Management, Washington, D.C. June 2003.

EPA, 2005a. Guidelines for Carcinogen Risk Assessment. Risk Assessment Forum. U.S. Environmental Protection Agency, Washington, DC. EPA/630/P-03/001F. March 2005.

EPA. 2005b. Supplemental Guidance for Assessing Susceptibility from Early-Life Exposure to Carcinogens. Risk Assessment Forum. U.S. Environmental Protection Agency, Washington, DC. EPA/630/R-03/003F. March 2005.

EPA, 2009b. Statistical Analysis of Groundwater Monitoring Data at RCRA Facilities Unified Guidance. U.S. Environmental Protection Agency. Office of Resource Conservation and Recovery. Program Implementation and Information Division. March 2009.

EPA, 2012. United States Environmental Protection Agency Regions 3, 6, and 9. April 2012.

Regional Screening Levels for Chemical Contaminants at Superfund Sites.

http://www.epa.gov/reg3hwmd/risk/human/rb-concentration_table/index.htm

Gilbert, 1987, Statistical Methods for Environmental Pollution Monitoring, Van Nostrand Reinhold Company, NY.

Johnson, P. C. and R. A. Ettinger, 1991, Heuristic Model for Predicting the Intrusion Rate of Contaminant Vapors into Buildings, Environmental Science and Technology, Volume 25(8), Pages 1445-52.

Jury, W.A., W.F. Spencer, and W.J. Farmer, 1983, Behavior Assessment Model for Trace Organics in Soil- 1. Model Description, Journal of Environmental Quality, Volume 12, Pages 558-564.

Page et al., 1982, Method of Soil Analysis, Part 2. Chemical and Microbiological Properties, pp.

570-571, Second Edition.

Wilson, L.G., Lorne G. Everett, and Stephen J. Cullen, 2000, Editors, Handbook of Vadose Zone Characterization and Monitoring, Lewis Publishers, Boca Raton, Florida.

#### Additional Resources for Data Collection

EPA, 1988, Guidance for Conducting Remedial Investigations and Feasibility Studies under CERCLA, OSWER-9335.3-01, Office of Solid Waste and Emergency Response, Washington, D.C.

EPA, 1993, Data Quality Objectives Process for Superfund, Interim Final Guidance, EPA/540- R-93-071, Office of Solid Waste and Emergency Response, Washington, D.C.

EPA, 1994, Guidance for the Data Quality Objectives Process, Office of Research and Development, EPA/600/R-96/055, Washington, D.C.

EPA, 1997, Expedited Site Assessment Tools for Underground Storage Tank Sites, EPA/510 B- 97-001, Office of Solid Waste and Emergency Response, Washington, D.C.

EPA, 1998, Guidance for Data Quality Assessment: Practical Methods for Data Analysis, Office of Research and Development, EPA/600/R-96/084, Washington, D.C.

#### Additional Resources for Risk Evaluation

EPA, 1988, Superfund Exposure Assessment Manual, EPA/540/1-88/001, Office of Remedial Response, Washington, D. C.

EPA, 1989a, Risk Assessment Guidance for Superfund, Volume II Environmental Evaluation Manual, Interim Final, EPA/540 1-89/001, Office of Emergency and Remedial Response, Washington, D.C.

EPA, 1989b, Risk Assessment Guidance for Superfund, Volume I Human Health Evaluation Manual (Part A) Interim Final, EPA/540/1-89/002, Superintendent of Documents, Number EP1.8:Su7/3/989, Office of Emergency and Remedial Response, Washington, D.C.

EPA, 1997a, Exposure Factors Handbook Volume 1 General Factors, EPA/600/P-95/002Fa, Office of Research and Development, National Center for Environmental Assessment, Washington, D.C.

EPA, 1997b, Exposure Factors Handbook Volume 2 - Food Ingestion Factors, EPA/600/P- 95/002Fb, Office of Research and Development, National Center for Environmental Assessment, Washington, D.C.

EPA, 1997c, Exposure Factors Handbook Volume 3 Activity Factors, EPA/600/P-95/002Fc, Office of Research and Development, National Center for Environmental Assessment, Washington, D. C.

EPA, 2002a, IRIS, Integrated Risk Information System, Office of Research and Development, National Center for Environmental Assessment, Cincinnati, Ohio.

EPA, 2004, Risk Assessment Guidance for Superfund, Volume I: Human Health Evaluation Manual (Part E, Supplemental Guidance for Dermal Risk Assessment). EPA-540-R-099-

005. July 2004.

EPA, 2009a, Risk Assessment Guidance for Superfund, Volume I: Human Health Evaluation Manual (Part F, Supplemental Guidance for Inhalation Risk Assessment). EPA-540-R- 070-002. January 2009.

EPA, 2011, Exposure Factors Handbook: 2011 Edition, EPA/600/R-090/052F, Office of Research and Development, National Center for Environmental Assessment, Washington D.C.

### Appendix A: References to Technical Documents

This page intentionally left blank for correct double-sided printing.

### Technical Guide for Addressing Petroleum Vapor Intrusion at Leaking Underground Storage Tank Sites. EPA June 2015. https://www.epa.gov/ust/technical-guide-addressing-petroleum-vapor-intrusion-leaking-underground-storage-tank-sites

- OSWER Technical Guide for Assessing and Mitigating the Vapor Intrusion Pathway from Subsurface Vapor Sources to Indoor Air (USEPA, 2015. https://www.epa.gov/sites/default/files/2015-09/documents/oswer-vapor-intrusion-technical-guide-final.pdf

### Quality Assurance/Quality Control

### DEQ Quality Management: https://www.deq.idaho.gov/public-information/assistance-and-resources/quality-management/

### DEQ QAPP Template: https://www.deq.idaho.gov/public-information/assistance-and-resources/quality-management/

### Waste Division Sample Collection (QA/QC) Procedures:

### How DEQ Evaluated Site Assessment and Corrective Action Data (Fact Sheet)

### Data Review, Verification, and Validation of External Party Petroleum Data Quality Assurance Project Plan and Standard Operating Procedure (January 2023)

### How DEQ Evaluates Sample Collection and Data Analysis For Used Oil UST Closures and Releases (Fact Sheet)

### Used Oil UST Closure and Release Sampling Standard Operating Procedures (May 2017)

### Standard Operating Procedure for Landfarming Petroleum-Contaminated Soils (July 2017)

### Standard Operating Procedures for Management and Disposal of Petroleum-Contaminated Soil Following a Release from a Non-Underground Storage Petroleum Storage Tank (February 2014)

### https://www.deq.idaho.gov/waste-management-and-remediation/sampling-investigation-and-cleanup/sample-collection/

### EPA Quality Management: https://www.epa.gov/quality

### EPA Quality Assurance: https://www.epa.gov/fedfac/assuring-quality-federal-cleanups

### EPA Data Quality: https://www.epa.gov/fedfac/guidance-systematic-planning-using-data-quality-objectives-process

### EPA Data Useability: https://www.epa.gov/risk/guidance-data-useability-risk-assessment-part-1-final-april-1992

### Table 1 Sources:

- ATSDR (Agency for Toxic Substances and Disease Registry). 2024. Toxicological Profiles.  Division of Toxicology and Human Health Sciences Atlanta, GA 30329-4027. Available at: https://www.atsdr.cdc.gov/toxprofiledocs/index.html

- National Center for Biotechnology Information (NCBI). 2024. PubChem Compound Summary for CID 6734, Acenaphthene. Available at: https://pubchem.ncbi.nlm.nih.gov/compound/Acenaphthene. Retrieved August 19, 2024.

- National Center for Biotechnology Information (NCBI). 2024. PubChem Compound Summary for CID 8418, Anthracene. Available at: https://pubchem.ncbi.nlm.nih.gov/co. Retrieved August 19, 2024

- EPA (U.S. Environmental Protection Agency). 1998. Toxicological Review of Naphthalene. In Support of Summary Information on the Integrated Risk Information System (IRIS). Washington, D.C. Available at: https://iris.epa.gov/static/pdfs/0436tr.pdf. Retrieved August 19, 2024.

- EPA (U.S. Environmental Protection Agency). 1990. Integrated Risk Information System (IRIS). Chemical Assessment Summary for Pyrene. National Center for Environmental AssessmentWashington, D.C. Available at: https://iris.epa.gov/static/pdfs/0445_summary.pdf. Retrieved August 19, 2024.

5. Low-flow Purging. Low Stress (low flow) Purging and Sampling Procedure for the Collection of Groundwater Samples From Monitoring Wells (January 2017). https://www.epa.gov/sites/default/files/2017-10/documents/eqasop-gw4.pdf

This page intentionally left blank for correct double-sided printing.

s

d	





HB	Enclosed Space

ER

Ground surface

Foundation Cracks

Vadose zone

LTS

Diffusing vapors

Subsurface impacted soils

| RBTLsi | = | Risk-based target level for indoor inhalation of vapors from |
| --- | --- | --- |
|  |  | subsurface soils [mg/kg-dry soil] |
| RBTLai | = | Risk-based target level for indoor inhalation of air [mg/m3-air] |
| α | = | Chemical-specific attenuation factor from subsurface soil to indoor |
| EF | = | (enclosed space) air derived from the Johnson-Ettinger model [-] Equilibrium factor to convert vapor target level to total soil target |
|  |  | level |

Water table

d









o	d









HB	Enclosed Space

ER

Ground surface

Foundation Cracks

Vadose zone

LTS

Diffusing vapors

Subsurface impacted soils

Water table

SW

| K |  | wsKd s Has s |
| --- | --- | --- |
| Where: |  |  |
| LFSW | = | Soil to ground water leaching factor [mg/L-H20)/mg/kg-soil)] |
| DAFSummers | = | Dilution attenuation factor in the mixing zone [-] |
| KWater-soil | = | Water-soil partitioning coefficient [(mg/kg-soil)/(mg/L-H2O)] |
| s | = | Dry soil bulk density [g-soil/cm3-soil] |
| ws | = | Volumetric water content in vadose zone soils [cm3-H2O/cm3- soil] |
| Kd H | = = = | foc  Koc Chemical-specific soil-water sorption coefficient [cm3-H2O/g-soil] Chemical-specific Henry's Law constant [(L-H2O)/(L-air)] |
| as | = | Volumetric air content in the vadose zone soils [cm3-air/cm3-soil] |
| Ugw | = | Ground water Darcy velocity [cm/yr] |
| gw | = | Ground water mixing zone thickness [cm] |
| I | = | Infiltration rate of water through soil [cm/year] |
| Wgw | = | Ground water mixing zone length [cm] |



s

unsat





PEF  Q C 

3600

0.036  1 V  U

m	t

U



3

 F (x)

Where:

PEF	=	Particulate emission factor [(m3-air)/(kg-soil)]

Q/C	=	Inverse of the mean concentration at the center of square source [(g/m2-s)/(kg/m3)]

V	=	Fraction of vegetative cover [-]

Um	=	Mean annual wind speed [m/s]

Ut	=	Equivalent threshold value of wind speed at 7 m [m/s]

F(x)	=	Function dependent on Um/Ut derived using Cowherd et al. 1985 [-]

0.036	=	Empirical constant [m2-hr/g]

Source: EPA 1996

Ds

eff	a

= D  	+ D 



3.33

as

w



2.0

T

1

H

   ws

 3.33



2.0

T

Where:

D eff

s

Da Dw

as

ws

T

H

=  Effective diffusion coefficient in soil based on vapor-phase concentration [cm2/s]

=	Chemical-specific diffusion coefficient in air [cm2/s]

=	Chemical-specific diffusion coefficient in water [cm2/s]

=	Volumetric air content in capillary fringe soils [cm3-air/cm3-soil]

=	Volumetric water content in capillary fringe soils [cm3-H2O/cm3-soil]

=	Total soil porosity in the impacted zone [cm3/cm3-soil]

=	Chemical-specific Henry's Law constant [(L-H2O)/(L-air)]

Deff = L

ws

TGW



 h



 Dcap

cap +	TGW	cap

L

 h

eff

Ds

eff

-1





Where:

Dwseff

hcap

Dca eff

p

D eff

s

LTGW

=  Effective diffusion coefficient between ground water and surface soil [cm2/s]

=	Thickness of capillary fringe [cm]

=	Effective diffusion coefficient through capillary fringe [cm2/s]

=	Effective diffusion coefficient in soil based on vapor-phase concentration [cm2/s]

=	Source-building separation [cm]

D	D

eff	a

 3.33

cap

=	



acap + Dw 

2.0

T

1

H





3.33

wcap



2.0

T

Where:

Dca eff

Da

p

Dw

acap

wcap

T

H

=  Effective diffusion coefficient for the capillary fringe [cm2/s]

=	Chemical-specific diffusion coefficient in air [cm2/s]

=	Chemical-specific diffusion coefficient in water [cm2/s]

=	Volumetric air content in capillary fringe soils [cm3-air/cm3-soil]

=	Volumetric water content in capillary fringe soils [cm3-H2O/cm3-soil]

=	Total soil porosity [cm3/cm3-soil]

=	Chemical-specific Henry's Law constant [(L-H2O)/(L-air)]

Source: ASTM 1995

D

eff crack

= D 

a

 3.33

acrack



2.0

T

+

D

w



1

H

   wcrack

 3.33



2.0

T

Where:

Dcrackeff Da

Dw

acrack

wcrack

T

H

=  Effective diffusion coefficient through foundation cracks [cm2/s]

=	Chemical-specific diffusion coefficient in air [cm2/s]

=	Chemical-specific diffusion coefficient in water [cm2/s]

=	Volumetric air content in foundation/wall cracks [cm3-air/cm3-total volume]

=	Volumetric water content in foundation/wall cracks [cm3-H2O/cm3-total volume]

=	Total soil porosity [cm3/cm3-soil]

=	Chemical-specific Henry's Law constant [(L-H2O)/(L-air)]



wcap	r

    

1  hN 

   

Tcap	r

M

Where:

wcap	=	Water content in the capillary fringe zone soil [cm3-water/cm3-soil]

r	=	Residual soil water content [cm3-water/cm3-soil]

Tcap	=	Total porosity of soil in the capillary fringe zone [cm3-voids/cm3-soil]

α	=	Point of inflection in the water retention curve where dw/dh is maximal [1/cm]

h	=	Air-entry pressure head [cm]

=	1/ and assumed to be positive

N	=	Van Genuchten curve shape parameter [-]

M	=	1 − (1/N)

i, j

HQ	

i, j

HQ

HQ

C	

C

C

i, j

i, j

i, j

HQ

HQ

C	

C

C

Page 35 of 123

### Appendix C: Estimation of Exposure Point Concentrations in Soil and Groundwater

This page intentionally left blank for correct double-sided printing.

- Background	1

- Statistical Considerations for Estimating Exposure Point Concentrations	2

  - Maximum Detected Concentration	2

  - Upper Confidence Limit	3

  - Arithmetic Average	4

  - Geometric Average	4

  - Area-Weighted Average	4

  - Depth-Weighted Average	5

  - Volume-Weighted Average	5

- General Considerations for Estimating Exposure Point Concentrations	5

  - Surface and Subsurface Soil Concentrations	5

  - Groundwater Concentrations	6

- Calculation of Exposure Point Concentrations	8

  - Surficial Soil (0–1 Foot below Ground Surface)	8

    - Exposure Point Surficial Soil Concentration for the Groundwater Protection Pathway	8

    - Exposure Point Surficial Soil Concentrations for the Direct Contact Pathway	9

  - Subsurface Soil (Greater Than 1 Foot below Ground Surface)	10

    - Exposure Point Subsurface Soil Concentration for Groundwater Protection

Pathway	10

    - Exposure Point Subsurface Soil Concentration for Vapor Intrusion Pathway	10

  - Exposure Point Concentration for Protection of a Construction Worker	11

  - Groundwater	12

    - Exposure Point Compliance Well Concentration for Groundwater Protection Pathway	12

    - Exposure Point Groundwater Concentration for Protection of Indoor Inhalation Pathway	13

4.5. Soil Vapor	14

- Estimating Area-Weighted Exposure Point Concentrations	14

  - Step 1: Identify the Exposure Unit	14

  - Step 2: Subdivide the Exposure Unit	15

  - Step 3: Estimate Exposure Point Concentrations for Each Thiessen Polygon	15

  - Step 4: Estimate Area-Weighted Average Concentration for the Exposure Unit	15

- Recommendations	17

References	19

Other Useful References	20

Figure 1. Schematic of soil leaching to groundwater	9

Figure 2. Thiessen polygon method	16

### Background

Exposure point concentrations (EPC) are the average chemical concentrations to which receptors are exposed over a specified duration within a specified geographical area. The geographical area about which a receptor moves and contacts contaminated media during the specified exposure duration is termed an exposure unit (EU). EPCs are also often referred to as representative concentrations.

This appendix describes the concept and methodology that should be used to estimate EPCs. This guidance provides general recommendations on procedures to accurately determine EPCs. These recommendations may not apply to all sites, particularly large, complex sites. Site-specific circumstances may require consultation with the Idaho Department of Environmental Quality (DEQ), the application of professional judgment, and reference to the more detailed literature cited herein. Other useful references are also included at the conclusion of this appendix.

In the site-specific application of the risk evaluation process, EPCs are used as input parameters in estimating the risk to a specified receptor for each complete route of exposure identified in the conceptual site exposure model (CSM) and each petroleum chemical of interest (COI). Risk management decisions are then made based on a comparison of the estimated risk with the regulatory specified target risk. This comparison is termed a forward-mode calculation. EPCs can also be used in what are termed backward-mode calculations when they are compared with calculated target levels for each complete route of exposure and each petroleum COI. In either mode, the calculation of EPCs is critical in the outcome of the risk evaluation.

Complications in the calculation of the EPCs may arise because the concept of EPC is often mistakenly associated with a site as opposed to an exposure pathway or the source. Since there may be several complete pathways at a site, several EPCs, one for each complete pathway, should be estimated. For example, estimation of the soil EPC for the pathway involving soil leaching and protection of groundwater ingestion typically considers data over the entire soil profile from the source area. At the same site, soil EPC estimates for the direct contact route of exposure involving surficial soil would typically consider only the upper several feet of the soil profile and would focus on the portions of the site where receptors may travel. The accurate estimation of EPCs is complicated by several factors, including the following:

- Spatial variability in the concentrations

- Temporal variability in the concentrations

- Lack of sufficient, appropriately located, site-specific concentration data

- Poor definition or uncertainty in the location, size, or other characteristics of the EU

The uncertainty in the EPC introduced by some of these factors can be reduced by considering them during the development of the CSM and the sampling and analysis plan.

After samples have been collected and analyzed, the results need to be appropriately evaluated to produce sound estimates of EPCs. This evaluation includes assessing the following:

- Data quantity

- Data quality (which data are acceptable for use)

- Statistical methods (which methods to use)

- Which usable data apply to a specific EU

This guidance focuses primarily on the latter two issues.

### Statistical Considerations for Estimating Exposure Point Concentrations

Numerous statistical methods can be employed to develop estimates of the exposure point (average) chemical concentration to which a receptor will be exposed. These methods include examining the maximum detected concentration, upper confidence limit (UCL) of the mean (for example the 95% UCL), arithmetic average, geometric average, area- weighted average, depth-weighted average, and volume-weighted average concentration (very rarely used). Associated with each of these methods are certain advantages, disadvantages, and constraints on their appropriate use. There is no uniformly accepted statistical methodology to estimate the EPC. Implicit in the decision to use any particular method is the need to acknowledge and account for the representativeness of the samples collected, the homogeneity of the defined EU, the statistical distribution of the data, the minimum data requirements of the statistical method used, and the uncertainty in the resultant data used to derive the estimated concentration.

From a practical perspective, several factors common to petroleum release sites may make the use of statistical methods to develop EPCs at a given site unrealistic, particularly for ground water. These factors include the following:

- Small release areas

- Dominantly subsurface releases, often at great depth

- Focus on source areas and not exposure pathways

- Limited sample numbers due to temporal/spatial constraints

##### Maximum Detected Concentration

The use of the maximum detected chemical concentration is required for comparison to screening levels during the screening level evaluation (see Section 3, Screening Level Evaluation, in the main body of the document). Depending on the quality of the site characterization data available, the maximum detected concentration can be used as EPCs in site-specific risk evaluations and will typically represent a conservative estimate of the average concentrations, particularly when sample numbers are low (i.e., four or less).

The effort necessary to calculate EPCs for certain complete pathways can be avoided by using the maximum media-specific concentrations when they do not exceed the target

levels or when the cumulative risk, calculated using maximum concentrations, does not exceed the target risk.

##### Upper Confidence Limit

The true population mean is a unique value that can be calculated only if the entire population has been sampled (i.e., the entire contaminated media analyzed). Since the entire population is almost never sampled, the true mean is never known. Thus, at best, only an estimate of the true population mean concentration is possible.

To account for the uncertainty associated with the estimated mean concentration, a confidence interval about the true but unknown mean is often constructed. The interval estimate includes a range and an associated degree of confidence that the true unknown mean lies within this range. In risk assessment applications, instead of calculating a two- sided confidence interval, a one-sided confidence interval is most often estimated.

For the calculation of EPCs, the U.S. Environmental Protection Agency (EPA) has recommended, in most cases, using the 95% upper confidence limit (UCL) of the mean. The upper limit for a one-sided 95% confidence interval of the mean is defined as a value that, when calculated repeatedly for randomly drawn subsets of site data, equals or exceeds the true mean 95% of the time. Alternatively, the true mean exceeds the UCL only 5% of the time. Ideally, a minimum of 8 to 10 samples is desirable to generate a 95% UCL. Although the UCL can be determined with fewer samples, doing so tends to results in the 95% UCL estimate being larger (large variances) than the arithmetic average and will sometimes exceed the maximum measured concentration. In these instances, the maximum measured concentration can be substituted for the 95% UCL or the user can decide to collect more samples to reduce the sampling variance.

Several issues commonly arise with environmental data sets that complicate the development of the confidence limit and should be evaluated through the use of an exploratory analysis prior to performing the calculation. These issues include the following:

- Determination of the distribution of the data chosen to represent the EU (normal, lognormal, or nonparametric [i.e., not conforming to a specific distribution]). This determination includes decisions as to what analysis and statistical tests to perform, how to interpret the test results, and how to treat the data once a determination is made.

- How to handle censored data and/or skewed data (typically due to non-detect [ND] values).

Depending on the nature of the underlying distribution of the data (or lack thereof), the one-sided 95% UCL can be calculated using a number of methodologies. The details regarding these methodologies are beyond the scope of this guidance.

EPA has developed technical guidance, a user’s guide, and computer software (ProUCL, current version) for calculating the UCL based on parametric (normal and lognormal distributions) and nonparametric distributions and when data sets have ND observations

. The ProUCL technical guidance describes the minimum data set conditions under which the ProUCL software will perform UCL calculations. These guidelines generally involve data set sizes greater than or equal to five with more than one detected value.

For samples collected using a multi-increment sampling approach, data set sizes as small as three may be sufficient to calculate a UCL. The Interstate Technology Regulatory Council (ITRC) has developed a guide for incremental sampling methodology

.

##### Arithmetic Average

Using the arithmetic average (or mean) is similar to the UCL with the disadvantage that no estimate of the uncertainty, variability of the average, or information regarding the underlying distribution of the population is incorporated into the value generated. For a population that is normally distributed, the estimates generated using the 95% UCL versus the mean tend to converge as the number of samples approaches 20 to 30

.

##### Geometric Average

The geometric average is sometimes used as an estimate of the mean of lognormally distributed variables. It represents the value in the original scale of the mean of the transformed variable. It is, however, a biased estimate of the mean, and statistical texts caution against its use in environmental applications. Statistical texts or the ProUCL guide should be consulted when attempting to develop an unbiased estimate of the mean of a lognormally distributed variable. Exploratory data analysis should be conducted to confirm an assumption that the variable is lognormally distributed prior to calculating an EPC. As with the arithmetic average, no estimate of the uncertainty or variability is incorporated into the value generated by the geometric average and it may not be conservative.

##### Area-Weighted Average

Where sampling locations are unevenly spaced, area-weighted averaging methods can be employed to generate an estimate of the average concentration across an area. This calculation is done by generating an area (or Thiessen polygon) associated with each sampling location, assigning a “weight” to each data point based on the area of the polygon associated with that point, and summing the weighted data point values. One advantage of the method is that it is useful in characterizing areas where the number of sample points is low.

The method also has several disadvantages. First, the shape, and consequently the contribution of an area associated with a sample location to the “average,” is totally dependent on the spatial distribution of all data points and may not be reflective of the true distribution of contaminants at a site. Second, since each polygon is associated with one sample point, no estimate of the accuracy or error in the average value is possible. Finally, the method, unlike several other interpolation schemes, does not assume that points that are closer together are more similar than points that are farther apart. The method for calculating an area-weighted average using Thiessen polygons is discussed in section 5.

##### Depth-Weighted Average

Depth-weighted averaging methods are used to develop an average or EPC for a specific borehole where multiple samples at varying depth intervals have been obtained. In characterizing a source area with multiple boreholes, the depth-weighted averages from each borehole can be used in a 95% UCL calculation. The methodology for depth- weighted averaging is described in EPA’s Soil Screening Guidance: User’s Guide (EPA 1996, section 2.3).

##### Volume-Weighted Average

Volume-weighted averaging methods are a combination of area- and depth-weighted averaging techniques. They are rarely used in risk assessment evaluations.

### General Considerations for Estimating Exposure Point Concentrations

Estimating EPCs requires considering several issues. Prior to performing the computations, consider the information in the following sections.

##### Surface and Subsurface Soil Concentrations

When evaluating exposure point soil concentrations consider the following:

- The spatial resolution of the data must be sufficient. While the exact number of necessary samples cannot be specified, data should be available from areas of known or likely sources, and the EU should be defined for a given pathway and receptor.

- Historical data should be examined closely prior to inclusion in the EPC estimate, particularly if the petroleum COIs are susceptible to biodegradation (such as petroleum hydrocarbons), volatilization, or leaching. If sufficient new data are collected, they may be used for risk evaluation and the old data may be disregarded. A new release requires the collection of additional data.

- If there is a “high” density of soil samples (>30) for a homogeneous EU and sample locations are approximately equally spaced, the arithmetic average may be

used instead of the 95% UCL or the area-weighted average. This method is acceptable because the area-weighted average, arithmetic average, and UCL concentrations will tend to converge if the underlying population can be assumed to be normally distributed. The >30 value mentioned above should also be considered in relation to the size of the EU and may not be appropriate for very large EUs.

- ND soil samples located at the periphery of the EU (e.g., the footprint of a building) should not be used.

- ND samples (also referred to as left-censored data) located within the EU require special treatment. The EPA has recently discouraged using replacement methods (such as half the detection limit). The proportion of ND samples in the subject data set and whether multiple detection limits exist will affect the specific methods used to generate summary statistics and the UCL. EPA ProUCL

software has several options for handling left-censored data. Consult the technical guidance accompanying the software to determine which method is most appropriate for the specific data set and how to treat the censored data.

- If multiple surficial soil samples and/or multiple subsurface soil samples are available from the same borehole within the EU, the depth-weighted average or arithmetic average concentration of these samples may be used. If the samples are equally spaced, the depth-weighted average concentration will be the same as the arithmetic average concentration.

- An alternative to using discrete soil samples to calculate the UCL is to use incremental sampling methodologies. Incremental sampling is a sampling approach to obtain a pooled sample for analysis that has an analyte concentration representative of the EU. While it is similar to composite sampling, the incremental approach takes into account sample size to reduce variability and also allows for the calculation of a UCL while only requiring a few samples for laboratory analysis. It typically requires greater up-front planning with respect to sampling design, methodology, and defining of data quality objectives than traditional composite sampling.

- If vapor intrusion is a potentially complete pathway, collecting soil vapor data, in lieu of soil samples, for characterizing the potential risk should be considered. Consult EPA’s Technical Guide for Addressing Petroleum Vapor Intrusion at Leaking Underground Storage Tank Sites.

##### Ground Water Concentrations

Follow the guidelines listed below to account for the temporal and spatial variation in ground water concentrations. These recommendations apply primarily to evaluation of ground water data for the purpose of estimating EPCs for risk evaluation.

They are not intended for cases where other monitoring objectives, such as determining compliance with cleanup criteria or site closure, are the primary goals.

- For wells with a clear increasing or a decreasing trend, data from the most recent 12 months or the 4 most recent measurements should be averaged (use whichever covers the longer duration). Note that for wells with increasing trends, continued monitoring may be needed until the trend stabilizes or decreases. The need for additional monitoring is a site-specific determination that is dependent on a number of factors, including the magnitude of measured concentrations in relation to risk-based criteria, the degree to which samples are representative and address site factors such as seasonality, and the estimated travel time of contamination from the release area to ground water and to potential receptors. For example, four measurements, taken at appropriate intervals over the course of a year, show an increasing trend but are all below applicable risk criteria. Further monitoring to develop an EPC may not be necessary.

- For wells with stable or fluctuating concentrations with no apparent trend, data from either the most recent 24 months or the 8 most recent measurements should be used to develop a 95% UCL.

- If site groundwater analytical data are limited (number of wells or number of sampling events) and prevent adequate statistical analysis, using the maximum concentrations is preferred. Using the average concentration is discouraged, particularly if the plume stability status (increasing, decreasing, or stable) with respect to the release is unknown or the representativeness of the available data is in question.

- While developing the EPC for wells described by the first two bullets above, the treatment of left-censored data (concentrations below detection limits) deserves special consideration and may require specialized analysis techniques. Replacement techniques (such as using half the detection limit) are currently discouraged by EPA. If the percentage of ND values is a significant percentage of the total number of data points (>25%), the representativeness of the data set for the EU in question should be evaluated.

- Wells with concentrations consistently below detection limits in the periphery of the EU should not be used.

- For wells that contain free product where it is not possible to obtain a representative dissolved phase sample, the use of the effective solubility of petroleum COIs should be considered as a conservative surrogate for their chemical concentration in estimating EPCs.

Two useful documents that provide guidance on the statistical evaluation of ground water monitoring data are the EPA’s Statistical Analysis of Groundwater Monitoring Data at RCRA Sites Unified Guidance and DEQ’s Statistical Guidance for Determining Background Ground Water Quality and Degradation

.

### Calculation of Exposure Point Concentrations

An EPC is necessary for each complete exposure pathway at a site. Based on the pathways typically considered in the risk evaluation process, the EPCs listed below are typically needed for each media.

##### Surficial Soil (0–1 Foot below Ground Surface)

The risk evaluation process requires consideration of two pathways of exposure associated with the surficial soil: 1) the ingestion of chemicals in groundwater due to leaching of residual chemicals present in the surficial soil and 2) the accidental ingestion of soil, outdoor inhalation of vapors and particulates from surficial soil emissions, and dermal contact with surficial soil. These two pathways are referred to as the groundwater protection and direct soil contact pathways, respectively. Thus, at most, two different surficial soil EPCs are required, though typically one EPC estimate can be used for both pathways.

    - Exposure Point Surficial Soil Concentration for the Groundwater Protection Pathway

Figure 1 shows a schematic of the soil leaching to groundwater pathway. The evaluation of this pathway for surficial soil assumes that the contamination does not extend greater than 1 foot below ground surface. If this is not the case, then samples taken in the surficial soil zone should be included with those from greater depth and evaluated together.

The conceptual model for this pathway also assumes that the leachate from the surficial (or subsurface) soil source travels vertically downwards to the water table without any lateral or transverse spreading. Thus, the horizontal dimensions of the surficial (or subsurface) soil source and the groundwater source are identical. Estimating the source dimensions requires considerable professional judgment. Factors typically considered in making these estimates include historical knowledge of where a spill or source was located, visual clues such as soil discoloration, and laboratory data (taking care not to include samples with ND results.

Irrespective of the manner in which the source area is identified, it is important to indicate the dimensions of the source on a map. The exposure point surficial soil source concentration should be estimated using only the surficial soil data collected within the delineated source zone. The method used to derive the EPC should typically be either the UCL of the mean (when the quantity and characteristics of the data available allow a statistical analysis) or the values derived from an incremental sampling approach.

Figure 1. Schematic of soil leaching to groundwater

    - Exposure Point Surficial Soil Concentrations for the Direct Contact Pathway

For this pathway, the exposure point surficial soil concentration has to be based on the receptor’s EU (i.e., the area over which the receptor may be exposed to the surficial soil). The exact location of this area is often difficult to estimate since it must be representative of a period of time equal to the receptor’s exposure duration (for example, up to 30 years for residential land use and about 7 years for nonresidential land use). In most cases, it is reasonable to assume that the receptor will access all portions of the EU in a random manner. Under current conditions, in the absence of specific information about the receptor’s activities, the unpaved portion of the site may be considered as the receptor’s approximate EU. Where volatile organic compounds (VOCs) are included in the list of potential petroleum COIs and/or the paved portion is in poor condition or consists of a material that does not limit VOC migration, this guide would not apply. For potential future exposures, if current condition cannot be assumed to continue, the assumption should be made that the pavement is removed, exposure to surficial soil is possible, and the entire site should be considered as the EU.

If the possibility exists that, for a future use residential scenario, contaminated subsurface soil existing under a current nonresidential scenario may be brought to the surface and result in direct contact residential exposures at the surface, the appropriate subsurface data should be included in the estimate of the EPC.

To estimate the EPC for this pathway, it would be necessary to estimate the receptor’s area of exposure and determine the number of soil samples available within this EU. This information should be used to estimate the 95% UCL or an area-weighted average concentration using the procedures discussed in section 5. The use of an incremental sampling approach for characterizing EPCs for this route of exposure may be particularly well-suited and efficient.

##### Subsurface Soil (Greater Than 1 Foot below Ground Surface)

The risk evaluation process includes two pathways of exposure associated with subsurface soil: 1) the ingestion of chemicals in groundwater or exposure to surface water impacted by discharge of contaminated groundwater resulting from leaching of chemicals in subsurface soil and 2) indoor inhalation of vapor emissions. These two pathways are referred to as the groundwater protection and vapor intrusion pathways, respectively. Thus, two different subsurface soil EPCs are required (one for each complete pathway). Additional EPCs may be required if the receptor’s EU for current and future conditions is different.

    - Exposure Point Subsurface Soil Concentration for Ground Water Protection Pathway

The leachate from the subsurface soil source is assumed to travel vertically downwards without any lateral or horizontal spreading (Figure 1). Thus, the EPC for this pathway should be based on the subsurface soil concentrations measured within the source area.

As in the case of the surficial soil concentration protective of groundwater

(section 4.1.1), the exposure point subsurface soil source concentration would be the average concentration calculated using the data within the soil source area. If the source area is small (several tens of feet across), typically smaller numbers of soil samples are available. In this case, the 95% UCL should first be calculated if the sample numbers for applicable data are adequate. If the 95% UCL exceeds the maximum concentration, it may be appropriate to use the maximum concentration.

    - Exposure Point Subsurface Soil Concentration for Vapor Intrusion Pathway

The use of soil concentration data is not allowable, soil vapor data must be collected, see section 4.5.

##### Exposure Point Concentration for Protection of a Construction Worker

The risk evaluation process includes the evaluation of one pathway of exposure for construction workers: accidental ingestion, dermal contact, and outdoor inhalation of vapors and particulates from soil.

For the construction worker, no distinction is made between the surficial and subsurface soil because subsurface soils may be exposed during construction. To estimate the EPC for the construction worker, it is necessary to identify the depth, areal extent, and number of samples taken within the zone of construction. The potential future depth of construction can be estimated by contacting local construction firms and identifying the typical depth of utilities on and adjacent to the site, contacting local utility companies, or

using as-built plans for proposed facilities if they exist. If the areal extent of the construction area is not known, it may be conservatively estimated as the source area. If sample results are available for several depth intervals, these should be averaged using depth weighting. The depth-weighted averaged values from multiple boreholes could then be combined to calculate a 95% UCL or area-weighted average, as appropriate. The EPC would be the depth-weighted average concentration within this zone of construction.

##### Groundwater

The risk evaluation process requires evaluating two routes of exposure associated with shallow groundwater: 1) ingestion of groundwater and 2) indoor inhalation of vapor emissions from groundwater. Where multiple aquifers are present, the shallowest aquifer is typically the aquifer first considered for both pathways. Site-specific circumstances— such as the disappearance of the shallow aquifer or transport of contamination to the deeper aquifer—may require evaluation of both aquifers. Characterization of petroleum COIs in both aquifers should still be performed. At a minimum, two different groundwater EPCs, one for each complete pathway, are required.

    - Exposure Point Compliance Well Concentration for Ground Water Protection Pathway

For the ingestion of groundwater pathway, maximum contaminant levels (MCLs) or equivalent concentrations have to be met at the point of exposure location. For any petroleum COI that does not have an MCL, the risk-based equivalent level is calculated as residential tap water (see section 3.1). Often, the point of exposure well location is hypothetical and data may not be available.

Alternatively, if a point of exposure well is present, an upgradient sentry or compliance well is often used as an early warning monitoring location. In both of these cases, ground water transport models are used to calculate the predicted concentrations at the point of exposure based on source area concentrations, the allowable concentrations in the source area, or an alternate point of compliance location concentration that will result in acceptable concentration at the point of exposure. In any case, EPCs will need to be estimated for the source area or for the alternate point of compliance location. In addition, one or more compliance wells have to be identified, and target compliance well concentrations (typically higher than the exposure well concentration) have to be calculated at these wells.

To evaluate this pathway, the EPCs in the source area, point of compliance, or point of exposure locations should be calculated based on the measured concentrations as discussed below. In most cases, a rule of thumb is that measurements taken more frequently than quarterly would not be considered statistically independent samples, would not be accepted to meet the specified number of measurements, and should not be used for statistical analysis and hypothesis testing. Methods to evaluate temporal independence are discussed in EPA’s Statistical Analysis of Groundwater Monitoring Data at RCRA Sites Unified Guidance and DEQ’s Statistical Guidance for Determining Background Ground Water Quality and Degradation.

.

      - For wells with no clear trend, the EPC should be estimated as the 95% UCL if the data permit. Where the calculated 95% UCL exceeds the maximum concentration or if the data are not suitable for statistical analysis, the maximum concentration should be used as the EPC. For compliance wells with stable, clearly decreasing,

or clearly increasing trends, the EPC is estimated as the 95% UCL of the most recent 1 year of data or most recent 4 measurements, whichever represents the longer time duration. For wells with an increasing concentration trend, continued monitoring may be needed until the trend stabilizes. Quarterly data for at least

1 year is recommended. The data should be pooled for source area characterization where multiple wells are available. Wells clearly not part of the source area of contamination should not be included in the calculations.

    - Exposure Point Groundwater Concentration for Protection of Indoor Inhalation Pathway

Groundwater concentrations protective of indoor inhalation are estimated using EPA’s Vapor Intrusion Screening Level (VISL) CalculatorThe EPCs for this pathway should be based on the groundwater concentration measured within and adjacent to the footprint of the building.

For the indoor inhalation of vapor emissions from groundwater, multiple EPCs may be required if the plume has migrated below several current or potential future buildings. For example, if a plume has migrated or is likely to migrate below two different buildings

(for example on-site and off-site buildings), separate on-site and off-site EPCs should be estimated.

If the plume has migrated below several buildings with similar receptors (residential or commercial), it may be sufficient to evaluate this pathway only for the building below which the concentrations are the highest and/or the depth to groundwater is the lowest. If this building is protective of indoor inhalation exposures, it may not be necessary to evaluate other buildings.

The plume may not be in the proximity of current structures. If assumed future land use includes the possibility of building structures in the vicinity of the plume, a similar calculation of EPCs and risk should be made using appropriate assumptions about the location and type of future structures.

While the target groundwater concentrations are based on the assumption of no lateral or transverse spreading of the vapors as they diffuse upwards to the building, site characteristics such as site lithology and preferential pathways may result in lateral migration towards a structure (see EPA’s Technical Guide for Addressing Petroleum Vapor Intrusion at Leaking Underground Storage Tank Sites). For this reason, the EPCs should be conservatively based on data gathered adjacent to the structure. After identifying the locations of the building footprints and the appropriate groundwater monitoring data within and adjacent to each footprint, the 95% UCL or area-weighted average concentration within each footprint may be estimated, as discussed in section 5.

##### 4.5. Soil Vapor

Soil vapor is used exclusively to evaluate the risk posed via the vapor intrusion pathway. In most cases, because of the variability commonly observed with vapor measurements and the uncertainty of vapor flow paths from the subsurface into indoor spaces, it is preferable to use the maximum concentrations measured within the defined EU. Where multidepth sampling of soil vapor has demonstrated attenuation of concentrations and the presence of a sufficient thickness of oxygenated soil conditions, the use of lower concentrations may be acceptable. Sufficient sample numbers are more often available where source areas or the buildings under consideration are large. In these situations, the use of area- weighted averaging or other statistical methods may be appropriate.

### Estimating Area-Weighted Exposure Point Concentrations

Area-weighted EPCs can be estimated using the Thiessen polygon method. If the available data are located on a uniform grid, the area-weighted average would be the same as the arithmetic average. If the dimensions of the source and the receptor’s EU are relatively small (several tens of feet across) and very few (1 to 6) soil samples are available within the soil source, the arithmetic average concentration may be used as an approximation of the area-weighted average concentration.

##### Step 1: Identify the Exposure Unit

The first and most critical step is to identify the size and location of the EU over which the area-weighted EPC has to be estimated. The location and size of this EU will often vary depending on the pathway being evaluated. Specific guidance on the location of the receptor’s area of exposure has been discussed in sections 3 and 4. Area-weighted concentrations can only be estimated if multiple samples have been collected within the unit. If several samples are available just outside the EU, it may be reasonable to extend the size of the unit to include this data. If the values of these samples are ND, they should not be included. This exclusion is technically justified since at most sites the location of the EU is approximate at best.

As part of Step 1, the various EUs for which area-weighted concentrations are desired should be drawn on a site map. The location of data points (soil borings, monitoring wells, etc.) should also be clearly labeled on the map.

If the borings or monitoring wells within the EU are located in a regular grid, the calculation of an area-weighted average is unnecessary and the user should proceed with an EPC calculation using a different method, such as the 95% UCL.

##### Step 2: Subdivide the Exposure Unit

The EU, identified in Step 1, is discretized into polygonal elements by connecting the sampling points within each EU (also identified in Step 1) and drawing perpendicular bisectors to these lines to form polygons. The area of each Thiessen polygon is then estimated.

##### Step 3: Estimate Exposure Point Concentrations for Each Thiessen Polygon

The concentration measured at the sampling location within each polygon is considered representative of the area of each polygon. As discussed in section 3, if multiple data are available from a location (either multiple depths or dates), compute the arithmetic average concentration of each petroleum COI measured at that location. The arithmetic concentration is then considered representative of the polygon.

##### Step 4: Estimate Area-Weighted Average Concentration for the Exposure Unit

The area-weighted average concentration for the EU is estimated using Equation 1:

| i  n ( Ai * Cavg,i) Carea  i 1 ATotal | (Equation 1) |
| --- | --- |

Where,

Carea = area-weighted average concentration over the EU [mg/kg]

Ai	= area of each Thiessen polygon [m2]

ATotal = total area of the polygons (i.e., area of the EU) [m2]

Cavg,i = mean of soil or groundwater concentrations measured within the polygonal element i [mg/kg]

An example application of the Thiessen polygon method is schematically shown in Figure 2.

I

MW-4

MW-1

IV

II

III

MW-3

MW-2

Street "X"

Highway "N"

N

| Polygonal Element | Mean Soil Concentration Cavg,i mg/kg | Area of the Element Ai m2 |  |  | Ai * Cavg,i m2 - mg/kg | Area Weighted Average Concentration mg/kg |
| --- | --- | --- | --- | --- | --- | --- |
| I | 2 | 604 |  |  | 1208 |  |
| II | 1.8 | 398 |  |  | 716.4 |  |
| III | 1.2 | 578 |  |  | 693.6 |  |
| IV | 1 | 234 |  |  | 234 |  |
| TOTAL |  | 1814 |  |  | 2852 |  |
| Area Weighted Average Concentration = |  |  | Carea | i n ( Ai * Cavg ,i)  i 1 ATotal |  | 1.57 |

Figure 2. Thiessen polygon method

### Recommendations

- For most pathways, 95% UCL techniques are recommended for development of EPC estimates of average concentrations. For cases where the number of samples is low (four or fewer) and/or the number of ND values is high, the use of the maximum concentration may be most appropriate. The use of the maximum may also be appropriate when the calculated 95% UCL exceeds the maximum concentration measured at the site and the collected data are considered representative of site and EU conditions.

- The use of area-weighted average methods is not recommended for most pathways, but may be justified in developing EPCs for the surficial soil pathways. The inclusion of methods that evaluate and incorporate spatial uncertainty into the area-weighted average calculation increases the validity of the estimate derived.

This page intentionally left blank for correct double-sided printing.

### References

API (American Petroleum Institute). 2010. BioVapor Indoor Vapor Intrusion Model. http://www.api.org/ehs/groundwater/vapor/bio-vapor-intrusion.cfm

Alaska DEC (Alaska Department of Environmental Conservation). 2009. Draft Guidance on Multi-Increment Sampling. Juneau, AK: Alaska DEC, Division of Spill Prevention and Response.

Burmaster, D. E., and K. M. Thompson. 1997. Estimating Exposure Point Concentrations for Surface Soils for Use in Deterministic and Probabilistic Risk Assessments. Human and Ecological Risk Assessment 3(3):363–384.

DEQ (U.S. Department of Environmental Quality). 2009. Statistical Guidance for Determining Background Ground Water Quality and Degradation. Boise, ID: DEQ. Version 2008-1.

EPA (U.S. Environmental Protection Agency). 1992a. Supplemental Guidance to RAGS: Calculating the Concentration Term. Washington, DC: EPA, Office of Emergency and Remedial Response, Hazardous Site Evaluation Division.

OSWER Directive 9285.7.08.

EPA (U.S. Environmental Protection Agency). 1996. Soil Screening Guidance: User’s Guide. Second edition. Washington, DC: EPA, Office of Solid Waste and Emergency Response. Publication 9355.4-23.

EPA (U.S. Environmental Protection Agency). 2009. Statistical Analysis of Groundwater Monitoring Data at RCRA Facilities Unified Guidance. Washington, DC: EPA, Office of Resource Conservation and Recovery, Program Implementation and Information Division.

EPA (U.S. Environmental Protection Agency). 2011a. ProUCL Version 4.1 Technical Guide (Draft). Washington, DC: EPA. EPA/600/R-07/041. Available at: http://www.epa.gov/osp/hstl/tsc/softwaredocs.htm .

EPA (U.S. Environmental Protection Agency). 2011b. ProUCL Version 4.1 Users Guide (Draft). United States Environmental Protection Agency. EPA/600/R-07/038.

May 2011. Available at: http://www.epa.gov/osp/hstl/tsc/softwaredocs.htm.

Fetters, C. W. 1993. Contaminant Hydrogeology. New York: MacMillan.

Gilbert, R. O. 1987. Statistical Methods for Environmental Pollution Monitoring. Van Nostrand Reinhold Company Inc.

Hawaii DOH (Hawaii Department of Health). 2008. Technical Guidance Manual for the Implementation of the Hawaii State Contingency Plan. Interim Final. Honolulu, HI: Hawaii DOH. Section 4 available at http://www.hawaiidoh.org/tgm- pdfs/HTGM%20Section%2004-02.pdf.

ITRC (Interstate Technology Regulatory Council). 2012. Incremental Sampling Methodology. Technical and Regulatory Guidance. February 2012. Available at: http://www.itrcweb.org/ism-1/.

Johnson, P. C., and R. A. Ettinger. 1991. Heuristic Model for Predicting the Intrusion Rate of Contaminant Vapors into Buildings. Environmental Science and Technology 25(8):1445–1452.

Linsley, R. K., M. A. Kohler, and J. L. H. Paulhus. 1975. Hydrology for Engineers.

Second Edition. Columbus, OH: McGraw-Hill Publishing Company.

USACE (U.S. Army Corps of Engineers). 2009. Implementation of Incremental Sampling (IS) of Soil for the Military Munitions Response Program. Huntsville, AL: USACE, Environmental and Munitions Center of Expertise. Interim Guidance 09-

02. July 20 2009.

##### Other Useful References

EPA (U.S. Environmental Protection Agency). 1988. Superfund Exposure Assessment Manual. Washington, DC: EPA. OSWER Directive 9285.5-1. EPA/540/1-88/001.

EPA (U.S. Environmental Protection Agency). 1992b. Guidance for Data Usability in Risk Assessment, Part A. Washington, DC: EPA, Office of Solid Waste and Emergency Response. Publication 9285.7-09A.See especially chapters 5 and 6.

EPA (U.S. Environmental Protection Agency). 1997. The Lognormal Distribution in Environmental Applications. Washington, DC: EPA, Office of Solid Waste and Emergency Response. EPA/600/R-97/006. EPA (U.S. Environmental Protection Agency). 2001. Risk Assessment Guidance for Superfund: Volume III—Part A, Process for Conducting Probabilistic Risk Assessment. Washington, DC: EPA, Office of Emergency and Remedial Response. EPA 540-R-02-002.

OSWER 9285.7-45.

EPA (U.S. Environmental Protection Agency). 2002a. Estimation of the Exposure Point Concentration Term Using a Gamma Distribution. Washington, DC: EPA, Office of Solid Waste and Emergency Response. EPA/600/R-02/084.

EPA (U.S. Environmental Protection Agency). 2002b. Calculating Upper Confidence Limits for Exposure Point Concentrations at Hazardous Waste Sites. Washington, DC: Office of Emergency and Remedial Response. OSWER 9285.6-10.

EPA (U.S. Environmental Protection Agency). 2006a. On the Computation of a 95% Upper Confidence Limit on the Unknown Population Mean Based Upon Data Sets with Below Detection Limit Observations. Washington, DC: EPA, Office of Research and Development. EPA/600/R-06/022.

EPA (U.S. Environmental Protection Agency). 2006b. Data Quality Assessment: Statistical Methods for Practitioners, EPA QA-G-9S. Washington, DC: EPA, Office of Environmental Information. EPA/240/B-06/003. Available at http://www.epa.gov/quality/qs-docs/g9s-final.pdf.

Remington, R. 2003. Robust Confidence Interval Estimation on the Lognormal Mean.

M.S. Thesis. Moscow, ID: University of Idaho.

This page intentionally left blank for correct double-sided printing.

### Appendix B: Application of Natural Attenuation

This page intentionally left blank for correct double-sided printing.

###### Table of Contents

Introduction	1

PART I: The Science of Natural Attenuation	1

- The Concept of Natural Attenuation	1

- Overview of Natural Attenuation Processes	2

  - Advection	3

  - Molecular Diffusion	4

  - Mechanical Dispersion	4

  - Hydrodynamic Dispersion	5

  - Sorption	5

  - Volatilization	5

  - Biodegradation	5

  - Overall Effect of Natural Attenuation Processes	6

    - Expanding Plume	7

    - Stable Plume	7

    - Shrinking Plume	7

    - Lifecycle of a Plume	7

- Biodegradation of Petroleum Hydrocarbons	8

  - Composition and Amount of Hydrocarbons	9

  - Available Electron Acceptors	9

  - Quantity and Quality of Nutrients	10

  - Characteristics of Microorganisms	11

  - Indicators of Natural Attenuation of Hydrocarbon Plumes	11

    - Primary Line of Evidence	11

    - Secondary Line of Evidence	11

    - Tertiary (Optional) Line of Evidence	12

PART II: Process for the Implementation of Remediation by Natural Attenuation	14

- Risk Management Plan for Remediation by Natural Attenuation	15

- Source Characterization and Control	15

- Plume Characterization	16

- Reasonable Time Frame Determination	17

- Monitoring to Demonstrate the Effectiveness of Remediation by Natural Attenuation	18

- Contingency Measures	19

PART III: Techniques Available to Demonstrate Natural Attenuation	20

- Techniques to Demonstrate the Occurrence of Natural Attenuation	20

  - Graphical Techniques	20

  - Statistical Evaluations	22

- Techniques to Estimate the Site-specific Rate of Natural Attenuation	22

  - Mass Balance Analysis	23

  - Well Concentration-versus-Time Plot	23

  - Well Concentration-versus-Distance Plot	24

  - Evaluation of Plume Behavior with a Recalcitrant Tracer	25

- Techniques to Quantify the Future Behavior of the Plume	25

References	27

List of Tables

Table 1. Expected pattern of natural attenuation indicators (secondary line of

evidence)	12

List of Figures

Figure 1. Schematic of a hydrocarbon release	3

Figure 2. Concentration profile as a function of distance (ideal behavior and data from

an actual site)	6

Figure 3. Expected pattern of natural attenuation indicators (secondary line of

evidence)	10

Figure 4. Concentration-versus-time plot of benzene	21

### Introduction

This appendix presents general guidance on the applicability and implementation of remediation by natural attenuation (RNA) at petroleum release sites in Idaho. Its primary focus is on attenuation of groundwater contamination. Attenuation of petroleum hydrocarbons has also been documented in other media such as soil and soil vapor.

Attenuation in soil vapor is addressed in EPA’s Technical Guide for Addressing Petroleum Vapor Intrusion at Leaking Underground Storage Tank Sites (Appendix A), which discusses evaluation of the vapor intrusion pathway. This appendix is divided into three parts:

- Part I presents a brief overview of the science of natural attenuation (NA)

- Part II presents the regulatory requirements for implementing NA as a remedial option

- Part III discusses the techniques available to support, quantify, and implement NA as a remedial option

This document should be used in conjunction with the Idaho Risk Evaluation Manual for Petroleum Releases (REM) and publicly available literature when implementing RNA. Additional information is available in the references provided at the end of this document.

### PART I: The Science of Natural Attenuation

Part I presents an overview of the science of NA, including the following topics:

- A brief overview of the processes that constitute NA (section 2)

- A description of how biodegradation operates in attenuating petroleum hydrocarbons (section 3)

- A description of the indicators of NA of hydrocarbon plumes (section 3.5)

### The Concept of Natural Attenuation

The term natural attenuation refers to the reduction in concentration or mass of chemicals in a media due to the effects of various environmental fate and transport processes. NA occurs without human intervention. Although NA is applicable to a reduction in concentration in any medium (e.g., air, surface water, groundwater), in this appendix it refers to the reduction in concentration of a dissolved groundwater contaminant plume. For a groundwater plume, environmental fate and transport processes include physical processes (advection, dispersion, dilution, sorption, and volatilization); chemical processes (hydrolysis, oxidation, and reduction); and biodegradation processes (aerobic and anaerobic). Physical processes are nondestructive in that they redistribute the chemicals in the environment without affecting the total mass of the individual chemical. Chemical transformation processes and biodegradation result in the transformation of the parent chemical into other chemicals referred to as daughter products.

The only human action involved in RNA is monitoring to demonstrate that concentration reduction is occurring at a reasonable rate. Depending on site-specific conditions, RNA may be used as a stand-alone corrective action strategy or in combination with other engineered remediation alternatives.

This guidance focuses on using NA on sites contaminated with petroleum products. NA has also been used at sites with other classes of contaminants, such as metals, explosives, and pesticides. The same concepts for evaluation, requirements for site characterization, and demonstrations as to effectiveness are required regardless of the class of contaminant. A general approach to the evaluation of the bioremediation aspects of NA for a range of biotreatable compounds is described in ITRC’s A Systematic Approach to In Situ Bioremediation in Groundwater Including Decision Trees on In Situ Bioremediation for Nitrates, Carbon Tetrachloride, and Perchlorate.

### Overview of Natural Attenuation Processes

Although each contaminated site is unique, for this discussion it is useful to consider a generic site where a petroleum spill has occurred. The soil immediately below and around the point of release becomes contaminated, and the contaminant front moves downward. The downward movement of the contaminant front continues until one of the following three conditions occurs:

- The soil sorption capacity exceeds the amount of contaminant available.

- An impermeable barrier is encountered and the contaminant spreads horizontally.

- The contaminant reaches the water table and, if it is a light non-aqueous phase liquid (LNAPL), spreads horizontally.

The term LNAPL relates to the density of the released contaminant. LNAPLs are chemicals that are liquid at ambient temperature but lighter than water. For example, the density of toluene is 0.866 grams per cubic centimeter (g/cc) compared to a density of

1 g/cc for water.

After the contaminant movement has stopped, the soil through which it traveled is contaminated with residual chemicals that act as a contaminant source for the groundwater plume. Figure 1 shows a schematic of a LNAPL moving through the soil to the groundwater.

When rainwater infiltrates through the residual contaminated soils, it dissolves and leaches out the soluble components. For petroleum hydrocarbons, the toxic soluble chemicals include, but are not limited to, benzene, toluene, ethylbenzene, and xylene (BTEX); the soluble additives methyl tertiary butyl ether (MTBE) and ethylene dibromide; and smaller fractions of other less-soluble constituents. After undergoing NA processes in the unsaturated zone, these dissolved chemicals reach the water table, mix with groundwater, and move with the groundwater to form a dissolved plume. At sites where the spill is of a sufficient quantity to reach the water table and form an LNAPL layer on the water table, soluble constituents from the LNAPL gradually dissolve in groundwater and form a dissolved groundwater plume.

Additionally, vapors of volatile hydrocarbons (chemicals with a Henry’s Law constant greater than 1×10-5 atm-m3/mole) will spread outwards from the contaminated soil and dissolve in soil moisture and groundwater.

As the dissolved plume moves through the unsaturated and the saturated zones, its concentration decreases due to the combined influence of several NA processes. Each of these processes is briefly discussed below.

Volatilization

Aerobic Unsaturated Zone

Source

ground water flow

Aerobic Margin

Clean GW

Aerobic Margin

Aerobic - Uncontaminated Groundwater

Profile

Aerobic Margin

Source

Anaerobic Core

Aerobic Margin

Plan

Figure 1. Schematic of a hydrocarbon release

##### Advection

Advection refers to the bulk movement or flow of water caused by differences in density, temperature, or pressure. In most aquifers, flow occurs predominantly due to hydraulic gradients, which may be natural—resulting in regional groundwater movement—or man-made, caused by pumping or artificial recharge. In saturated zones, hydraulic gradients may exist in horizontal and vertical directions, resulting in complex three- dimensional flow patterns. However, at most sites there exists a predominant flow direction that results in the migration of most mass.

As water moves in the saturated zone, it carries with it the dissolved constituents. The process results in the migration of dissolved constituents in the saturated zone without a change in concentration and is termed advection.

Parameters required to estimate the volume of water moving through a saturated zone include the Darcy velocity and the cross-sectional flow area. Darcy velocity is estimated using hydraulic conductivity and hydraulic gradient.

Hydraulic conductivity may be estimated using a variety of methods, including slug tests, pump tests, grain size distribution, and literature values corresponding to the site stratigraphy. For most saturated zones, hydraulic conductivity varies in the horizontal and vertical directions.

Site-specific values of hydraulic gradients are estimated based on water-level measurements in monitoring wells, or piezometers. When estimating horizontal hydraulic gradients, it is important to compare data from wells screened in the same saturated zone. Wells screened in different zones or at different depths (e.g., cluster wells) can be used to estimate the vertical hydraulic gradient between the zones. Site-specific hydraulic gradients may exhibit seasonal and spatial variations (in magnitude and direction) due to a variety of factors such as pumping, seasonal flow in surface water bodies, and seasonal variations in rainfall.

Another related parameter used to estimate the travel time of a chemical due to advection is the seepage velocity, which requires an estimate of the porosity of the media.

Typically, literature values corresponding to the aquifer type (e.g., sand or silt) are used to estimate porosity.

##### Molecular Diffusion

Molecular diffusion refers to the transport of chemical mass from a zone of higher concentration to a zone of lower concentration due to the movement of molecules. The effect of diffusion is to spread the chemical mass over a large area and hence reduce the overall concentration. Thus, diffusion results in a reduction in concentration by dilution. In most groundwater systems where there is advection, the attenuation in concentration or the chemical mass transport due to diffusion is small and often negligible. However, in no-flow or low-flow situations, diffusion can be an important attenuation mechanism.

Molecular diffusion is quantified using Fick’s Law, and the parameter required to quantify diffusion is the effective diffusion coefficient of the chemical.

##### Mechanical Dispersion

Mechanical dispersion refers to the spreading of the contaminant plume that occurs due to variations in the flow velocity. In the saturated zone, velocity variations occur due to a variety of factors, including inter-pore and intra-pore velocity variations and the tortuosity of the porous media. The net effect of mechanical dispersion is that the chemical spreads both horizontally and vertically, thus reducing the overall concentration in the plume. Thus, like diffusion, dispersion is a dilution process.

The transport of mass due to mechanical dispersion is quantified using Fick’s Law. The key parameters are the dispersivity coefficients in the three cardinal (x, y, and z) directions. Dispersivity values have been measured using tracers at several research sites, based on which empirical dispersivity relationships have been developed. For most site-specific applications of fate and transport models, these empirical relationships are used. Higher dispersivity values result in larger plumes, which result in increased dilution and lower concentrations.

##### Hydrodynamic Dispersion

The term hydrodynamic dispersion refers to the sum of molecular diffusion and mechanical dispersion. In most groundwater systems where there is advection, hydrodynamic dispersion is approximately equal to mechanical dispersion, since the effect of molecular diffusion is negligible.

##### Sorption

Sorption refers to the transfer or distribution of mass between the liquid and solid phases. Sorption has the overall effect of reducing the mobility of the chemical, which increases its residence time in the subsurface and hence the amount of microbial degradation. This distribution is quantified by the soil water partition coefficient. For organic chemicals, the partition coefficient depends on the chemical-specific normalized organic carbon partition coefficient and the organic carbon content in the soil. The partition coefficient is used to estimate the retardation factor for the chemical, which is a measure of the “stickiness” of the chemical to the formation. Parameters required to estimate the retardation factor include porosity, organic carbon content of soil, bulk density of soil, and chemical-specific normalized organic partition coefficient.

##### Volatilization

Volatilization refers to the transfer of chemical from the LNAPL or dissolved phase to the vapor phase. Vapors can migrate by diffusion alone (in the absence of a pressure gradient) and by diffusion and advection if a pressure gradient exists in the formation. Chemicals volatilizing from the dissolved plume occupy the pores in the unsaturated zone and migrate outwards due to the combined influences of molecular diffusion and vapor- phase advection if pressure gradients exist. Thus, volatilization can result in a net loss of chemical from the dissolved phase or from the LNAPL.

The chemical-specific factors controlling volatilization from a dilute solution and LNAPL are the Henry’s Law constant and saturated vapor pressure, respectively. These factors are sensitive to the temperature of the media where volatilization occurs.

##### Biodegradation

Biodegradation refers to the destruction of chemicals by indigenous microorganisms present in the aquifer. For petroleum hydrocarbons, biodegradation is the primary

destruction mechanism. Due to the significance of biodegradation within the overall process of NA, biodegradation is discussed at length in section 3. In certain situations, additional compounds (e.g., ozone, oxygen, peroxide); nutrients; or microorganisms may be added to the contaminant plume to enhance the rate of biodegradation.

##### Overall Effect of Natural Attenuation Processes

As explained above, the process of NA refers to a combination of advection, molecular diffusion, dispersion, sorption, volatilization, chemical reactions, and biodegradation.

The combined effect of these processes is to spread or reduce the chemical mass in the dissolved plume. Thus, the concentration of the chemical decreases as distance from the source increases. Figure 2 shows the concentration profiles in actual monitoring wells down-gradient of a source and the expected concentrations under ideal conditions.

Although such patterns have been observed at several sites, fluctuations in water levels, flow direction, and climatic conditions often result in nonideal behavior. Additional complications occur due to errors in data collection, analytical measurements, and the existence of multiple sources (in space and time) at a site.

Upgradient

Inside the Plume

Downgradient

-100

0.00

-50

0

50	100	150	200	250	300	350

-1.00

Log  -2.00

Concen- tration -3.00 (mg/L)

-4.00

-5.00

-6.00

Distance (feet)

Legend

Ideal

Actual field data

September '99

X	April 2000 Monitoring well

Figure 2. Concentration profile as a function of distance (ideal behavior and data from an actual site)

Depending on the overall rate of NA and the on-going contribution of chemicals to the groundwater plume from the residual soil source, groundwater plumes may be classified as expanding, stable, or shrinking. These concepts are discussed below.

    - Expanding Plume

An expanding plume is characterized by increasing concentrations within the plume and/or an outward movement of the plume that increases the size of the plume. An expanding plume occurs when the chemical mass loading to the plume from leaching or dissolution of residual LNAPL exceeds the mass loss due to NA processes.

In an expanding plume, NA processes continue to occur but at a rate too slow to prevent the increase in concentrations or the size of the plume.

    - Stable Plume

A stable plume is characterized by stable concentrations within the plume and at the periphery of the plume (i.e., the concentrations do not show a decreasing or increasing trend). For a plume to be stable in size or concentration, the rate of chemical mass addition to the plume from the source is equal to the rate of chemical mass loss by NA processes. The source may be the chemicals leaching from adsorbed and residual concentrations in the capillary/water table fluctuation zone or in the vadose zone or the dissolution of chemicals from the LNAPL source.

    - Shrinking Plume

A shrinking plume is characterized by decreasing groundwater concentrations within the plume, decreasing concentrations at the periphery of the plume, or a decrease in the area of the plume. For a shrinking plume, the addition of chemical mass from the source is less than the mass lost due to NA processes. Thus, unless site-specific conditions change that result in an increase in chemical mass addition to the plume or a decrease in the rate of NA, the plume will eventually disappear.

    - Lifecycle of a Plume

Initially, when a chemical reaches the groundwater and a plume forms, it is an expanding plume. The plume continues to expand as long as the addition of chemical mass from the source exceeds mass loss by NA processes. As the plume expands and occupies an ever- increasing portion of the aquifer, the loss of mass by NA processes increases. Thus, a point is reached when the mass addition equals mass lost, and the plume becomes stable. Because of the phenomenon of NA, most BTEX plumes resulting from service station spills become stable at lengths less than 500 feet, as demonstrated by numerous plume studies. At most sites, the chemical source is finite; in time, the mass loading from the source to the dissolved plume decreases.

Meanwhile, the rate of mass loss due to NA processes increases or stays the same, so eventually the plume begins to shrink.

Under ideal conditions, all dissolved chemical plumes would follow the above lifecycle; however, real plumes may not follow this pattern due to a number of confounding factors, including the following:

      - Variations in groundwater velocities that affect the rate of NA

      - Fluctuations in the water table that affect the source mass loading due to the release of chemicals trapped in the capillary fringe

      - Variations in climatic conditions that affect the rate of NA

      - Variations in rainfall and infiltration that affect mass loading to groundwater

      - Additions to the plume from new releases and spills from operating facilities

Because of the above complications, it is important to collect and evaluate all the site data and look at multiple lines of evidence to understand the plume behavior and predict its future behavior (see section 3.5).

### Biodegradation of Petroleum Hydrocarbons

This section discusses the biodegradation of dissolved petroleum hydrocarbons and identifies the various indicators that should be measured to confirm biodegradation.

The biodegradation of hydrocarbons is the process by which naturally occurring subsurface microorganisms biodegrade contaminants. The process of biodegradation, as facilitated by microorganisms, can be represented by the general reaction:

Hydrocarbon + Electron Acceptors + Nutrients YIELDS Carbon Dioxide + Water + Microorganisms + Waste Products

The above biologically mediated reaction produces energy for cell growth and reproduction of the microbial population. The process of electron transfer results in the oxidation of the donor (hydrocarbon), reduction of the electron acceptor, and the production of usable energy for the organisms. Based on the principles of thermodynamics, biologically mediated reactions that yield the most energy are favored over reactions that yield less usable energy for the organisms. For hydrocarbons, biologically mediated reactions ideally occur in the following order:

- Dissolved oxygen in the groundwater is used as the electron acceptor.

- After all oxygen has been depleted and anaerobic conditions exist within the dissolved plume, dissolved nitrate is used as the electron acceptor.

- After the depletion of dissolved oxygen and nitrate, ferric iron is used as an electron acceptor and is reduced to ferrous iron.

- After the depletion of oxygen, nitrate, and iron, sulfate is used by sulfate- reducing bacteria to degrade the hydrocarbons.

- Finally, methanogenesis degrades the hydrocarbons.

While all petroleum hydrocarbons are biodegradable, the rate of biodegradation can differ significantly from site to site depending on the composition and amount of hydrocarbons; type and amount of available electron acceptors; type, number, and characteristics of the microorganisms; and quantity and quality of nutrients.

Site-specific measurements of these factors, comparison of these factors within and outside the plume, and temporal variations in these factors can be used to demonstrate the occurrence of biodegradation. Each of these factors is discussed below.

##### Composition and Amount of Hydrocarbons

Almost all petroleum hydrocarbons are biodegradable under aerobic conditions. The ease of biodegradation depends on the type of hydrocarbon. Low molecular weight hydrocarbons (e.g., C10 to C24 alkenes) and single-ring aromatics are the most easily biodegradable. As molecular weight increases, resistance to biodegradation also increases. Individual compounds degrade at different rates under aerobic and anaerobic conditions.

For most hydrocarbon-impacted sites, the primary chemicals of interest (from a risk perspective) are BTEX, MTBE, and naphthalene. Naphthalene and BTEX are readily degradable under aerobic conditions. Benzene typically degrades slower than other BTEX hydrocarbons under anaerobic conditions.  MTBE degrades under both aerobic and anaerobic conditions. However, the rates of MTBE degradation and controlling factors have not yet been firmly established.

##### Available Electron Acceptors

The biodegradation of hydrocarbons is essentially an oxidation-reduction reaction. In this reaction, the hydrocarbon donates the electron (i.e., the hydrocarbon is oxidized) and a second compound (the electron acceptor) is reduced. Of the several electron acceptors available in the subsurface environment, oxygen, because of its high energy yield, is typically used first by microorganisms. Anaerobic bacteria can use other electron acceptors, including the following:

- Nitrate (NO3-) that is reduced to nitrogen (N2)

- Manganese (Mn4+) that is reduced to water soluble Mn2+

- Ferric iron (Fe3+) that is reduced to water soluble Fe2+

- Sulfate (SO4--) that is reduced to sulfide (S-)

- Carbon dioxide (CO2) that may be used by methanogens to yield methane (CH4)

As the biodegradation of hydrocarbons occurs, the concentration of the electron acceptors decreases and the concentration of the products formed increases. This concept can be used to demonstrate the occurrence of biodegradation (i.e., secondary line of evidence, see section 3.5.2). Table 1 shows the expected relationship between the BTEX concentration and the electron acceptors and the products of the oxidation reduction reaction.

Oxygen

Upgradient

Downgradient

Inside the plume

Sulfate

Upgradient	Inside the plume	Downgradient

Nitrate

Upgradient

Downgradient

Inside the plume

Methane

Upgradient

Downgradient

Inside the plume

Manganese (+2)

Upgradient

Downgradient

Inside the plume

Carbon dioxide

Upgradient

Downgradient

Inside the plume

Ferrous Iron (+2)

Upgradient

Downgradient

Inside the plume

Figure 3. Expected pattern of natural attenuation indicators (secondary line of evidence)

##### Quantity and Quality of Nutrients

In addition to the electron acceptors, microorganisms also require nutrients. These nutrients are incorporated into the biomass and are necessary to form cells. The nutrients nitrogen and phosphorous are required in relatively large amounts. Small amounts of micro-nutrients—such as sulfur, manganese, and magnesium—are also required.

Certain compounds, such as nitrates and sulfates, can serve either as nutrients or electron acceptors. Nutrients are rarely a limiting factor in the subsurface biodegradation of petroleum hydrocarbons. A decrease in nutrient levels within the zone of degradation can indicate biodegradation.

##### Characteristics of Microorganisms

The ability of microorganisms to degrade a wide variety of petroleum hydrocarbons is well documented. Hydrocarbon-degrading microorganisms are widespread in the environment, as they occur in fresh water, salt water, soil, and groundwater. The number of cells per milliliter of water can also be used as an indicator of biodegradation. As hydrocarbons degrade, the cell count for hydrocarbon-degrading bacteria increases.

.

##### Indicators of Natural Attenuation of Hydrocarbon Plumes

Based on the information presented above, concentrations of several substances (hydrocarbons, electron acceptors, microorganisms, nutrients, and carbon dioxide) can be measured to demonstrate the occurrence of NA.

These measurements are typically divided into three tiers, or “lines of evidence” (i.e., primary, secondary, and tertiary lines of evidence), to demonstrate NA. Data collected under each line of evidence can be evaluated qualitatively or quantitatively.

    - Primary Line of Evidence

The purpose of the primary line of evidence of NA is to demonstrate the loss of chemical mass by evaluating measured petroleum hydrocarbon concentrations. Of all the methods available to demonstrate the occurrence of NA, this is perhaps the simplest and most useful to demonstrate site-specific reductions in risk. Site-specific application of the primary line of evidence requires an adequate number of correctly installed sampling points (monitoring wells), an adequate amount of chemical data from these points, and a thorough evaluation of these data. These issues are discussed in Parts II and III of this document.

Although the primary line of evidence can show whether the concentration of a plume is attenuating, it does not show whether the decrease is due to destructive mechanisms or merely dilution. Secondary lines of evidence are necessary to determine whether the decrease is due to biodegradation.

    - Secondary Line of Evidence

Secondary evidence of NA refers to measurements of electron acceptors and products of metabolism within the plume and their comparison with concentrations in the unimpacted area of the aquifer, where no biodegradation activity would be expected to occur.

Parameters that are typically measured in the field include dissolved oxygen, REDOX potential, dissolved nitrates, manganese, ferrous iron, sulfate, and methane. These parameters should be measured at up-gradient locations, inside the plume near the source, and at down-gradient locations. Expected patterns of these parameters that are indicative of biodegradation are shown in Table 1 and Figure 3.

As microorganisms consume chemicals, there is a corresponding consumption of the compounds that serve as electron acceptors. Thus, the concentration of these compounds would decrease in the portion of the plume where biodegradation is occurring. For example, under aerobic biodegradation, the concentration of oxygen would decrease, assuming oxygen is not being added to the plume. Similarly, under anaerobic conditions, a depletion of nitrate, ferric (III) iron, and sulfate can be expected.

Biodegradation also results in an increase in the concentration of metabolic byproducts. For example, increased concentrations of nitrite and ferrous (II) iron within the plume would be indicative of biodegradation.

Table 1. Expected pattern of natural attenuation indicators (secondary line of evidence)

| Upgradient | Inside the plume | Downgradient |
| --- | --- | --- |

| BTEX | ND | High | Low | ND |
| --- | --- | --- | --- | --- |

| Type of Reaction | Geochemical Indicator | Geochemical Indicator Concentration |  |  |
| --- | --- | --- | --- | --- |
| Aerobic | Oxygen | High | Low | High |
| Anaerobic | Nitrate | High | Low | High |
|  | Manganese | Low | High | Low |
|  | Ferrous Iron | Low | High | Low |
|  | Sulfate | High | Low | High |
|  | Methane | Low | High | Low |
|  | Carbon dioxide | Low | High | Low |

The secondary line of evidence demonstrates the occurrence of biodegradation only. It does not provide any data on the occurrence of other NA processes.

    - Tertiary (Optional) Line of Evidence

The tertiary (optional) line of evidence involves performing microbiological studies, such as identifying the microorganisms present in the formation and counting their cells, in an effort to demonstrate the occurrence of NA. Thus, the objectives of secondary and tertiary lines of evidence are similar. In the portion of the plume where NA is occurring, the ratio of petroleum degraders to the total number of bacteria should be higher than in

the uncontaminated portion of the plume due to the readily available petroleum energy source. Tertiary lines of evidence are seldom required at petroleum hydrocarbon impacted sites and are not discussed further.

### PART II: Process for the Implementation of Remediation by Natural Attenuation

Remediation by natural attenuation refers to the achievement of site-specific cleanup goals using NA. As discussed in Part I, NA consists of several processes that occur at all contaminated sites at varying rates. As with any other remedial option, the site-specific applicability of RNA must be carefully evaluated before it is selected as the remedial alternative of choice. RNA should not be considered a presumptive remedy; rather, it is one of several available strategies that should be evaluated to ensure its applicability based on site-specific conditions.

The first step in any evaluation of NA is the collection of appropriate site characterization data. In general, the data collected need to confirm the occurrence of NA, the effectiveness of RNA, and that the receptors are not exposed to unacceptable risk throughout the period when the site is being remediated by NA. The data collected should help answer the following questions and support the activities described in the questions.

      - Has the full horizontal and vertical extent of the source and the plume been delineated?

      - Is the plume at steady state and is it stable or shrinking in size?

      - Can contaminant degradation be demonstrated and quantified?

      - Can the long-term behavior of the source and the plume be modeled?

      - Can the time frame to achieve remedial cleanup goals be estimated?

      - What is the impact of source removal/reduction on the remedial time frame?

Once the site has been characterized to document the feasibility of NA, an evaluation as to the site-specific appropriateness of NA as the preferred remedy should be performed. RNA is applicable only at the following types of sites:

      - Sites where immediate threats to human health, safety, and the environment do not exist or have been mitigated

      - Sites where “active” sources, such as leaking tanks, drums, etc., have been removed

      - Sites where the projected time frame to achieve remedial objectives is “reasonable,” as defined in section 7

      - Sites where the plume is stable or shrinking and is not likely to impact current receptors or sensitive habitat

      - Sites where active remediation has removed the bulk of the contaminants and the role of RNA is to perform the “final touchup”

      - Sites where any necessary institutional controls can be reliably implemented

Upon selection of RNA as part or all of the preferred remedy, a corrective action plan will be developed to document the selection process and provide details about implementation of the remedy. The Idaho Department of Environmental Quality (DEQ) must approve this corrective action plan. At all sites, RNA will be accompanied by long- term monitoring to demonstrate that RNA is occurring at the rate anticipated when RNA

was selected as the remedial option. Monitoring will continue until the specified cleanup goals have been achieved. If the monitoring data indicate that RNA is not occurring at an acceptable rate, the remedial plan will need to be modified.

### Risk Management Plan for Remediation by Natural Attenuation

At sites where RNA is part of the preferred remedial option, the responsible party should include a discussion in the corrective action plan that addresses the following elements:

- A summary of site characterization activities that demonstrates a thorough understanding of the nature and extent of the source and the nature and extent of the impacts, provides data supporting the likely effectiveness of RNA, and discusses any source removal/control activities performed at the site

- A discussion of remedial endpoints and points of compliance, and the manner in which these were determined

- A discussion of the time frame over which the endpoints are expected to be achieved

- A demonstration that during this time frame, risks to human health and the environment are acceptable

- A discussion of monitoring locations, the rationale for monitoring location selection, and the type and frequency of data that will be collected to monitor the performance of RNA and the achievement of cleanup criteria

- A description of the tools to be used to evaluate/analyze the monitoring data

- A description of the institutional controls that may be required and documentation of their implementation

- A description of a contingency plan that will be implemented if the monitoring results indicate that RNA is not sufficiently effective or is not proceeding at the expected rate

- A description of the frequency and form of the reports to be submitted to DEQ during the course of implementing RNA

A brief discussion of critical components of the RNA portion of the corrective action plan is presented below.

### Source Characterization and Control

Characterizing and controlling the source are important aspects of RNA. Sufficient data should be collected during the site investigation phase to delineate the nature and extent of the source and estimate the source lifetime. The term “nature” refers to an evaluation of the chemicals of interest and a determination that the chemicals are amenable to NA. The extent refers to the physical dimensions of the source as well as a determination of the mass of residual chemicals present in the source. An estimate of the lifetime of the source can be made by calculating the mass flux out of the source area (using groundwater flow characteristics and contaminant concentrations in the source area) and

comparing this to the total contaminant mass in the source area (including free phase product, soils, and groundwater).

DEQ requires that all active sources (e.g., leaking pipes, tanks, spills) be stopped and any free phase product in groundwater be removed, to the maximum extent practicable. It is best to reduce the source and remediate soils containing residual product that are significant sources of contamination to groundwater. Such control measures will reduce the time required to achieve remedial objectives. At a minimum, sufficient source material should be removed to ensure a declining plume. The amount of active remedial activity necessary should be determined on a case-by-cases basis and clearly presented in the risk management plan.

### Plume Characterization

The goals of collecting data to characterize the plume are to demonstrrate the magnitude and direction of contaminant transport and the stability status of the plume, to confirm degradation is occurring, and to estimate the contaminant degradation rate or the ability of the aquifer to assimilate the contamination.

To achieve these goals, the responsible party must construct wells to account for aquifer heterogeneity and dominant zones of contaminant transport, select appropriate monitoring locations, collect samples at an adequate frequency, and collect and evaluate the appropriate analytical data. How to determine the appropriate monitoring locations, sampling frequency, and data is discussed below.

The location and number of groundwater samples collected and analyzed must be determined based on site-specific conditions.

At a minimum, sampling points should be located so as to achieve the following:

- Locate the distribution of contaminants within the plume

- Locate the plume boundaries

- Track plume movement and migration

While they do help define the extent of the plume, sampling points at nondetect locations provide little useful information for documenting plume characteristics for an NA evaluation. Most sampling points need to be located within the plume boundaries for NA evaluation purposes.

Monitoring wells should be located up-gradient of the source, within or immediately down-gradient of the source area, and within the plume aligned along the plume axis. Data should be collected from an unimpacted down-gradient well also. A down-gradient clean well can be used to help characterize the extent of the plume and may be used to protect down-gradient receptors by providing early detection of plume movement. This function is described in section 8. Wells delineating the sides of the plume assist in determining if there are significant seasonal changes in groundwater flow direction.

As discussed in sections 3.5.1 and 3.5.2, data related to the primary and secondary lines of evidence should be collected. In rare cases, it may be necessary to collect data to demonstrate tertiary lines of evidence as well. Based on the data collected, a determination should be made whether the plume is expanding, stable, or shrinking. The specific analytical tools that may be used to conduct this characterization are discussed in Part III of this document.

Expanding plumes require continued monitoring. Depending on site conditions, such as the risk to current or potential future receptors and the rate at which the plume is expanding, additional assessment of the plume and/or source removal and reduction may be necessary. An expanding plume typically requires active remediation.

Plumes documented to be stable or shrinking are candidates for the use of NA. These plumes will also require continued monitoring, though perhaps at a lower frequency than expanding plumes. Depending on site conditions and the time frame of remediation, a stable plume may require residual source characterization or removal.

The frequency and duration of monitoring during the plume characterization phase should be determined on a site-specific basis and in consultation with DEQ. However, in most cases, 1–2 years of quarterly monitoring data are necessary to evaluate the degree of seasonal variations in water levels and flow directions present at a site. This information is necessary to characterize a plume as expanding, stable, or shrinking and to estimate degradation rates. As clear trends emerge in the data, the monitoring frequency may be modified. The actual duration of sampling will depend on the time it takes to demonstrate a clear trend in the concentrations.

### Reasonable Time Frame Determination

The determination as to what constitutes a reasonable time frame for RNA is a complex, site-specific determination not amenable to simple rules of thumb or quantification. In making this determination, DEQ will take into account factors that include, but are not limited to, the following:

- The time frame for RNA compared to that for other remedies being evaluated

- The time frame in which affected portions of the aquifer might be needed for various uses

- The likelihood that impacted groundwater will be utilized

- The degree of uncertainty in site characterization and NA estimates

- The reliability of institutional controls over the time frames for which they may be required to function

- The ability of the responsible party to maintain the required monitoring and plume evaluation required when NA is used as a remedy

### Monitoring to Demonstrate the Effectiveness of Remediation by Natural Attenuation

Due to the uncertainties associated with the site-specific implementation of RNA, long- term monitoring is necessary to demonstrate its effectiveness. Long-term monitoring is used to ensure that the behavior of the plume does not change and that predictions of plume behavior are accurate. The objectives of long-term monitoring are to demonstrate NA is continuing to occur, human health and the environment are being protected, and the plume is not expanding.

The specifics of the monitoring plan (e.g., the location, frequency, and type of samples to be collected and the analytical procedures to be used) should be determined on a site- specific basis. The primary factors that should be considered when designing a long-term monitoring program include the following:

- Distance to potential receptor exposure points

- Groundwater seepage velocity and direction

- Types of contaminants

- Aquifer heterogeneity

- Three-dimensional distribution of chemicals of interest

- Areas of unique geochemical conditions

- Surface water impacts

- Effects of active remediation systems

The secondary factors that should be considered include the following:

- Access issues

- Property lines

- Contaminant contributions from off-site sources

Two types of wells are required for any long-term monitoring program: performance monitoring wells and contingency monitoring wells. Performance monitoring wells are used to demonstrate that NA is proceeding according to expectations; document that geochemical conditions continue to be adequate to support NA processes; identify any toxic products resulting from NA processes; determine if plume conditions remain stable or are shrinking; identify changes in groundwater conditions, such as change in flow direction, recharge, etc.; and document that cleanup criteria have been met.

Contingency monitoring wells are placed beyond the predicted down-gradient boundary of the plume and up gradient from known or potential receptor exposure points. Their purpose is to provide an “early-warning” if unexpected plume expansion occurs and allow implementation of a contingent remedy if needed. Multiple contingency wells may be needed, particularly if seasonal variations in groundwater flow direction are known to occur.

Sampling frequency should be based on the following factors:

- The natural variability observed in contaminant concentrations

- The distance and travel time from the source to the point of compliance

- The reduction in concentrations needed to meet target levels

### Contingency Measures

The RNA corrective action plan should include a contingency plan to be implemented if the site data indicate that RNA is not occurring at the expected rate or the exposure conditions at the site have changed, resulting in an unacceptable risk to human health or the environment.

Contingency measures will prevent delays in site remediation and provide a clear roadmap for site remediation. Triggers should be established and included in the plan that would cause the responsible party to implement alternative active strategies to enhance RNA. The triggers should be as objective and quantitative as possible. Examples of triggers that would cause the responsible party to implement alternative strategies include, but are not limited to, the following:

- A consistent increase in concentrations in one or more wells

- A failure of any of the institutional conditions necessary to protect human health and the environment during the period of RNA

- A change in the exposure conditions (e.g., removal of pavement)

- Continued expansion of the plume

- Unacceptably low rates of RNA

### PART III: Techniques Available to Demonstrate Natural Attenuation

Several techniques are available to evaluate the data collected to demonstrate NA or to design and implement an RNA program. These techniques can be divided into three categories: techniques to demonstrate the occurrence of NA, techniques to estimate the site-specific rate of NA, and techniques to quantify the future behavior of the plume. The available techniques for each of these evaluations are presented below.

### Techniques to Demonstrate the Occurrence of Natural Attenuation

The occurrence of NA may be demonstrated by using graphical and/or statistical techniques. The specific techniques used will vary depending on site conditions and the specifics of the data. To the extent possible, multiple techniques should be used to provide added insight into the NA process.

##### Graphical Techniques

Chemicals-of-interest data collected from strategically located monitoring wells can be used to draw site-wide contour maps of individual constituent concentrations for each monitoring event, create concentration-versus-time plots for each well with detectable constituent levels and at least four rounds of data, and create concentration-versus- distance plots along the flow direction for several monitoring events. Depending on the variability in the concentrations, it may be better to plot the natural logarithm of concentration versus time and distance. An example plot is shown in Figure 4. When creating these plots, care should be taken to ensure that the selected scale clearly demonstrates the trend. Since the concentrations are affected by water level fluctuations, it is important to also plot water levels as a function of time.

These plots can indicate whether the concentration trend is decreasing, stable (no significant trend), increasing, or mixed. The latter refers to a situation where different wells (source wells versus periphery wells) exhibit different trends. Increasing concentrations in the source well and decreasing concentrations in the down-gradient wells may occur due to a variety of reasons. For example, the mass loading to the source may cause an increase in the source wells due to an increase in the infiltration rate, or the rate of biodegradation near the source may be reduced due to a depletion of oxygen.

BENZENE CONCENTRATION

9.6

7.2

4.8

2.4

0.0

Sampling Date

NATURAL LOG OF BENZENE CONCENTRATION

2.4

1.8

1.2

0.6

0.0

Sampling Date

Concentration (mg/L)

Concentration (mg/L)

3/93

3/93

10/94

10/94

3/97

3/97

6/97

6/97

9/97

9/97

12/97

12/97

3/98

3/98

6/98

6/98

9/98

9/98

11/98

11/98

1/99

1/99

4/99

4/99

7/99

7/99

10/99

10/99

DEPTH TO GROUND WATER

20.0

17.5

15.0

12.5

= Average groundwater level

10.0

Sampling Date

Depth (feet)

3/93

10/94

3/97

6/97

9/97

12/97

3/98

6/98

9/98

11/98

1/99

4/99

7/99

10/99

Figure 4. Concentration-versus-time plot of benzene

Conversely, increasing concentrations in down-gradient wells and decreasing concentrations near the source may indicate that the plume is moving but the source has depleted. Conclusions related to concentration trends based on visual observations of the data must be supported by statistical analysis.

In addition to the chemical-of-interest concentrations, indicator chemical concentrations should be plotted along the plume axis (in the concentration-versus-distance plots) along with the hydrocarbon concentrations. The observed pattern of concentrations can be used

to demonstrate NA. For example, low dissolved oxygen concentrations within the plume and higher concentrations up-gradient and down-gradient of the source are indicative of biodegradation within the plume.

##### Statistical Evaluations

Statistical tools may be used to determine and quantify the concentration trend. These tests can be used to test a null hypothesis and an alternative hypothesis. A null hypothesis might be that there is no time trend in the concentrations-versus-distance or time, and the alternative hypothesis might be that there is a downward trend. Application of the statistical test would then result in accepting or rejecting the null hypothesis at a specified level of significance.

A nonparametric test, called the Mann-Kendall test, is often used to determine whether a trend exists in the data. This test is particularly useful for environmental data for several reasons:

- The test is nonparametric, which implies that the test is applicable irrespective of the underlying distribution of the data.

- The test is simple to implement.

- Concentration values below the detection limit can be used.

- The results are not affected by missing data.

The Mann-Kendall test is applicable only when there is no seasonality in the data. This is the case if the data do not show any seasonal variations or the data were collected from one season. When data indicate seasonality, the seasonal Kendall test may be used. For details on both of these tests, refer to  books on statistical analysis.

DEQ recommends the use of the Mann-Kendall test to determine the trend in nonseasonal data except in situations where a visual plot of data indicates without any ambiguity that a trend exists in the data.

If the concentration-versus-time or concentration-versus-distance data indicate a decreasing trend, a regression analysis may be used to estimate the slope of the best-fit line.

As explained in section 11.2 and 11.3, the slope of the line can be used to estimate the NA or the biodegradation rate. For additional information on regression analysis, refer to any statistics textbook.

### Techniques to Estimate the Site-specific Rate of Natural Attenuation

The data collected to demonstrate the primary line of evidence of NA can be used to estimate site-specific NA rates and biodegradation rates. Since NA includes multiple processes in addition to biodegradation, the NA rate would be higher than the

biodegradation rate. The latter can be used as an input into a groundwater fate and transport model, such as Domenico’s model, to estimate the future migration of the plume, and to estimate a site-specific dilution attenuation factor. Three methods are available to estimate the NA rate: 1) mass balance analysis for expanding, stable, or shrinking plumes; 2) plume concentration-versus-time plots; and 3) plume concentration-versus-distance plots. Descriptions of each of these methods are presented below. An excellent discussion of the calculation and use of rate constants for use in NA evaluations is presented in EPA’s Calculation and Use of First-Order Rate Constants for Monitored Natural Attenuation Studies. This section also includes a discussion of calculating biodegradation rates using recalcitrant tracers.

##### Mass Balance Analysis

This method of estimating the NA rate is based on the concept of mass balance. For a stable plume, the mass entering the groundwater plume must equal the mass lost by NA. Thus, if the mass entering the plume can be estimated, it should be possible to estimate the NA rate. A detailed description of the application of mass balance analysis to estimate the NA rate of a petroleum release is described in Section X6.1 of the Standard Guide for Remediation of Groundwater by Natural Attenuation at Petroleum Release Sites

.

##### Well Concentration-versus-Time Plot

The estimate of the overall attenuation rate for a shrinking plume can be calculated assuming a first-order decay rate and is represented mathematically by Equation 1:

| dC  exp( kt ) dt | (Equation 1) |
| --- | --- |

Where:

C = concentration

t = time [years]

k = NA rate [1/yr]

The solution to the above differential equation results in the familiar exponential decay equation (Equation 2):

| C(t)  C(t  0) exp(kt) | (Equation 2) |
| --- | --- |

Where:

C(t) = Concentration at any time t (mg/L)

Taking the natural logarithms of both sides of Equation 2 results in Equation 3:

| ln C(t)  ln C(t  0)   kt | (Equation 3) |
| --- | --- |

Thus, a plot of natural log of concentration versus time would be a straight line with a slope equal to k (the NA rate).

##### Well Concentration-versus-Distance Plot

Advective travel time can be expressed as below (Equation 4):

| t  x v | (Equation 4) |
| --- | --- |

Where:

x = Advective travel distance (cm)

v = Seepage velocity (cm/yr)

Substituting Equation 4 in Equation 2 results in Equation 5:

| C(t)  C(t  0) exp(k x ) v | (Equation 5) |
| --- | --- |

Taking the natural log of both sides of the equation results in Equation 6:

| ln[C(t  0)]  ln[C(t)]   k x v | (Equation 6) |
| --- | --- |

For a shrinking plume that follows the first-order attenuation rate presented above, a plot of log concentration versus distance would be a straight line with a slope of k/v. By multiplying the slope with the seepage velocity, the NA rate (k) can be estimated.

Buscheck and Alcantar used the solution of the one-dimensional transport equation with biodegradation to estimate the biodegradation rate based on the slope of the log concentration-versus-distance plot. Specifically, they derived the following expression (Equation 7) for the biodegradation rate:

|   v  k 2   4 α  1 2 αx v 1 x   | (Equation 7) |
| --- | --- |

Where:

 = Biodegradation rate, assumed to occur at equal rates in the dissolved and sorbed phases

x = Longitudinal dispersivity (0.1x)

Zhang and Heathcote (2003) modified this method to account for situations with finite source size and lateral dispersion to improve the estimates of the biodegradation rate.

The following step-by-step description of the biodegradation rate estimation process can be implemented on a site-specific basis.

Step 1:	Determine the groundwater flow direction based on the water level measurements for each monitoring event.

Step 2: For each monitoring event, identify the wells located along the direction of flow (i.e., along the plume center line). Since the flow direction may vary seasonally, different wells may be used for different monitoring events.

Step 3: Tabulate the concentrations of the chemicals of interest and calculate the natural log concentrations.

Step 4:	Plot the natural log concentrations on the y-axis and the distance along the x- axis.

Step 5:	Calculate the slope of the best-fit line and the confidence in this estimate by examining the 95% confidence limits of the slope.

Step 6:	Estimate the groundwater seepage velocity.

Step 7:	Multiply the slope of the best-fit line calculated in Step 5 with the seepage velocity. The result will represent the overall NA rate. This NA rate represents the reduction in concentration due to the combined influence of the various NA processes mentioned in section 2. This decay rate should not be confused with the biodegradation rate (λ) that is an input to groundwater models.

Step 8:	Estimate the biodegradation rate from Equation 7.

Steps 1–8 should be completed for each time period for which data are available and the results presented as a range of NA and biodegradation rates. The latter can be used as an input to the Domenico model to estimate the dilution attenuation factor. Due to confounding factors such as seasonal variations in groundwater velocity, fluctuations in water levels, and errors in sampling and analysis methods, the NA and biodegradation rates may vary significantly (by as much as a factor of 10). Therefore, it is best to present the range as well as the average rates.

##### Evaluation of Plume Behavior with a Recalcitrant Tracer

Physical, chemical, and biological processes act together to decrease contaminant concentrations away from a source. However, monitored NA requires that accurate biodegradation rates for a site be known. To estimate accurate biodegradation rates, conservative tracers can be employed. The concentrations of these tracers are not affected by biodegradation processes and so can be used to delineate between the effects of biodegradation and other NA processes such as dilution and sorption. Generally, tracers are biologically recalcitrant and have chemical properties similar to the contaminant of interest. Examples of common tracers include MTBE for petroleum hydrocarbon plumes.

### Techniques to Quantify the Future Behavior of the Plume

The future behavior of the plume can be estimated by using fate and transport models. A number of analytical and numerical models are available. Commonly used analytical

models include the Domenico model and BIOSCREEN.

These models can be used to estimate plume length, concentrations at down-gradient receptor locations, and dilution attenuation factors and can be calibrated to site-specific groundwater data. The correct application of fate and transport models requires experience and specialized knowledge that is beyond the scope of this document.

### References

API (American Petroleum Institute). 2007. Technical Protocol for Evaluating the Natural Attenuation of MtBE. Washington DC: API, Regulatory and Scientific Affairs Department. API Publication 4761.

ASTM (American Society for Testing and Materials). 2004. Standard Guide for Remediation of Ground Water by Natural Attenuation at Petroleum Release Sites. Conshohocken, PA: ASTM. Standard E1943-98(04).

Buscheck, T.E., and C.M. Alcantar. 1995. Regression Techniques and Analytical Solutions to Demonstrate Intrinsic Bioremediation. In R.E. Hinchee, J.T. Wilson, and

D.C. Downey, eds. Intrinsic Bioremediation. Columbia, OH: Batelle Press. pp. 109–116.

Buscheck, T.E., D.C. Wickland, and D.L. Kuehne. 1996. Multiple Lines of Evidence to Demonstrate Natural Attenuation of Petroleum Hydrocarbons. Proceedings of the Conference on Petroleum Hydrocarbons and Organic Chemicals in Groundwater. NGWA/API, Houston, TX.

Domenico, P.A. 1987. An Analytical Model for Multidimensional Transport of a Decaying Contaminant Species. Journal of Hydrology 91:49–58.

EPA (U.S. Environmental Protection Agency). 1999. Use of Monitored Natural Attenuation at Superfund, RCRA Corrective Action and Underground Storage Tank Sites. Washington DC: EPA, Office of Solid Waste and Emergency Response.

Directive 9200.4-17P.

EPA (U.S. Environmental Protection Agency). 2002. Calculation and Use of First-Order Rate Constants for Monitored Natural Attenuation Studies. Cincinnati, OH: EPA, National Risk Management Research Laboratory. Ground Water Issue Paper, EPA/540/S-02/500.

EPA (U.S. Environmental Protection Agency). 2005. Monitored Natural Attenuation of MTBE as a Risk Management Option at Leaking Underground Storage Tank Sites. Cincinnati, OH: EPA, National Risk Management Research Laboratory.

EPA/600/R014/179.

EPRI (Electric Power Research Institute). 1985. A Review of Field-Scale Physical Solute Transport Processes in Saturated and Unsaturated Porous Media. Palo Alto, CA: EPRI. Report EA-4190.

Freeze, A.R., and J. A. Cherry. 1979. Groundwater. Englewood Cliffs, NJ: Prentice-Hall, Inc.

Gibbons, R.D. 1994. Statistical Methods for Groundwater Monitoring. New York: John Wiley & Sons, Inc.

Gilbert, R.O. 1987. Statistical Methods for Environmental Pollution Monitoring.

New York: Van Nostrand Reinhold Company.

ITRC (Interstate Technology and Regulatory Council). 2002. A Systematic Approach to In Situ Bioremediation in Groundwater Including Decision Trees on In Situ Bioremediation for Nitrates, Carbon Tetrachloride, and Perchlorate. ITRC, In Situ Bioremediation Team.

McKee, J.E., F.B. Laverty, and R.M. Hertel. 1972. Gasoline in Ground Water. J. Water Pollution Control Federation 44(2):293–302.

Moyer, E.E., and P.T. Kostecki. 2003. MTBE Remediation Handbook. Amherst, MA: Amherst Scientific Publishers.

Rice, D.W., R.D. Grose, J.C. Michaelson, B.P. Dooher, D.H. Macqueen, S.J. Cullen,

W.E. Kastenberg, L.G. Everett, and M.A. Marino. 1995. California Leaking Underground Fuel Tank (LUFT) Historical Case Analysis. Livermore, CA: Lawrence Livermore National Laboratory, Environmental Protection Department, Environmental Restoration Division.

SNL (Sandia National Laboratories). 1997. Natural Attenuation of Metals and Radionuclides: Report from a Workshop held by Sandia National Laboratories. SNL. SAND97-2727.UC-800.

USACE (U.S. Army Corps of Engineers). 1999. Draft Protocol for Evaluating, Selecting, and Implementing Monitored Natural Attenuation at Explosives-Contaminated Sites. Vicksburg, MS: USACE, Engineer Research and Development Center.

Technical Report EL-99-10.

Wiedemeier, T.H., M.A. Lucas, and P.E. Haas. 2000. Designing Monitoring Programs to Effectively Evaluate the Performance of Natural Attenuation. Brooks Air Force Base, TX: U.S. Air Force, Air Force Center for Environmental Excellence.

Xu, M., and Y. Eckstein. 1995. Use of the Least-Squares Method in Evaluation of the Relationship Between Dispersivity and Field Scale. Ground Water 33(6):905–908.

Zhang, Y., and R.C. Heathcote. 2003. An Improved Method for Estimation of Biodegradation Rate with Field Data. Ground Water Monitoring and Remediation 23(3):112–116.

##### Other Useful References

Alaska DEC (Alaska Department of Environmental Conservation). 2000. Guidance on the Selection of Natural Attenuation as a Cleanup Alternative for the Restoration of Soil and Groundwater at Contaminated Sites. Juneau, AK: Alaska DEC, Division of Spill Prevention and Response. Guidance No. SPAR 2000-1.

Bauer, S., C. Beyer, and O. Kolditz. 2006. Assessing Measurement Uncertainty of First- Order Degradation Rates in Heterogeneous Aquifers. Water Resources Research Vol. 42, W01420, 14 p.

Wiedemeier, T.H., J.T. Wilson, and D.H. Kampbell. 1996. Natural Attenuation of Chlorinated Aliphatic Hydrocarbons at Plattsburg Air Force Base, New York. Symposium on Natural Attenuation of Chlorinated Organics in Ground Water. Dallas, TX, September 1996.

Wiedemeier, T.H., J.T. Wilson, R.N. Miller, and J.E. Hanson. 1999. Technical Protocol for Implementing Intrinsic Remediation for Long-Term Monitoring for Natural Remediation of Fuel Contamination Dissolved in Groundwater, Volume I. Brooks Air Force Base, TX: U.S. Air Force, Air Force Center for Environmental Excellence.

Wisconsin DNR (Wisconsin Department of Natural Resources). 2003. Guidance On Natural Attenuation For Petroleum Releases. Madison, WI: Wisconsin DNR, Bureau for Remediation and Redevelopment. PUB-RR-614.

This page intentionally left blank for correct double-sided printing.

### Appendix D: Sample Table of Contents for a Risk Evaluation Report

This page intentionally left blank for correct double-sided printing.

## TABLE OF CONTENTS

LIST OF TABLES LIST OF FIGURES

EXECUTIVE SUMMARY

  - INTRODUCTION

  - SCOPE

  - OBJECTIVES

  - REVIEW AND ANALYSIS OF AVAILABLE DATA

  - INTRODUCTION

  - SITE DESCRIPTION AND LAND USE

    - Location and Vicinity

    - Topography

    - Current Land Use

    - Future Land Use

  - CHRONOLOGY OF EVENTS

  - SITE GEOLOGY AND SOIL STRATIGRAPHY

    - Regional Geology

    - Stratigraphy

    - Groundwater Flow Direction and Magnitude (horizontal and vertical)

  - WATER USE

  - RELEASE SCENARIO AND SOURCE

    - Chemicals of Concern

    - Distribution of Chemicals

      - Chemicals in Soil

      - Chemicals in Groundwater

      - Chemicals in Surface Water

      - Chemicals in Soil Vapors

  - SCREENING LEVEL EVALUATION

  - COMPARISON WITH SCREENING LEVELS

  - RECOMMENDATIONS

  - EXPOSURE ASSESSMENT

  - INTRODUCTION

    - Source

    - Release Mechanism

    - Transport Media

    - Receptors

    - Route of Exposure

  - CONCEPTUAL SITE EXPOSURE MODELS FOR EACH EXPOSURE UNIT

  - GROUNDWATER PROTECTION

  - INPUT PARAMETERS FOR RISK EVALUATION, ESTIMATING RISK, AND DEVELOPING REMEDIATION STANDARDS

  - CALCULATING SITE-SPECIFIC SCREENING LEVELS AND ESTIMATING RISK

    - JUSTIFICATION OF SITE-SPECIFIC FATE AND TRANSPORT PARAMETERS

    - JUSTIFICATION OF SITE-SPECIFIC EXPOSURE FACTORS

    - JUSTIFICATION OF ALTERNATIVE TOXICITY FACTORS AND PHYSICAL AND CHEMICAL PROPERTIES

    - REPRESENTATIVE CONCENTRATIONS FOR EACH RECEPTOR, ROUTE OF EXPOSURE, AND EXPOSURE UNIT

    - JUSTIFICATION OF MODELS AND EQUATIONS

  - COMPARISON OF CALCULATED RISK WITH TARGET RISK CRITERIA FOR EACH EXPOSURE UNIT

  - CALCULATION OF REMEDIATION STANDARDS

- CONCLUSIONS AND RECOMMENDATIONS

- REFERENCES

### Appendix E: Practical Quantitation Limits

This page intentionally left blank for correct double-sided printing.

### Introduction

When collecting media-specific chemical concentration data and establishing remediation standards, situations may arise where practical quantitation limits (PQLs) for specific chemicals and samples may exceed screening levels or site-specific, risk-based concentrations. Section 500 of IDAPA 58.01.24, Application of Risk Based Corrective Action at Petroleum Release Sites, delineates the following factors that may be used to allow the use of PQLs as remediation standards:

- Analytical method

- Method detection limit

- Sampling procedures

- Estimated risk levels

- Other

This appendix provides background on the concepts of PQLs, MDLs and Remediation Standards and provides guidance on using the above factors when a REM user is proposing PQLs as remediation standards.

### Background— Practical Quantitation Limits, Method Detection Limits and Remediation Standards

What are PQLs and MDLs?

A PQL is defined in part in 58.01.24.010 as “The lowest concentration of a chemical that can be reliably quantified among laboratories within specified limits of precision and accuracy for a specific laboratory analytical method during routine laboratory operating conditions.” The method detection limit (MDL) is defined as “The minimum concentration of a substance that can be reported with 99% confidence is greater than zero.”

How are PQLs and MDLs Derived?

PQL values are derived in a number of ways, and there is no consensus as to the process used to develop them. In some cases, they are derived as a fixed multiplier applied to the MDL value or the standard deviation of samples used to develop the MDL. The multiplier commonly ranges from 3 to 5 and may be as great as 10. In other cases, PQLs may be derived by analysis of data from actual laboratory performance studies. In these cases, PQL values are a function of analytical methods, a given laboratory’s equipment, operating protocols, operating conditions, quality assurance/quality control procedures, and sampling and media considerations. Values are sometimes reported by laboratories at levels between the MDL and the PQL and are commonly qualified as estimated quantities. The closer the estimated value is to the PQL, the more confidence in the estimated value.

For the petroleum chemicals of interest (COI) listed in the rule, the common analytical methods at petroleum release sites are established by the U.S. Environmental Protection Agency (EPA). These analytical methods include 8011, 8260, 8270 (SIM), TO-17, and TO-15 SIM. In their method descriptions, EPA lists examples of lower limits of quantitation for individual analytes but indicates that they are instrument-dependent and influenced by sample preparation/introduction technique or highly matrix-dependent and are provided for guidance only.

For a given media, sampling considerations may have an impact on achievable detection limits. For example, when sampling soil vapor or ambient air with certain methods, the achievable quantitation limit can be a function of the amount of vapor or air sampled.

Quantitation limits are commonly reduced as the amount of air sampled increases.

Specialized analytical techniques can sometimes provide lower MDL and PQL values. In a comparison of values achieved with the use of standard method 8270 for semi-volatile compounds versus employing 8270 using selective ion monitoring (SIM). PQL values are 6 times lower for soils and 20 times lower for water samples.

Depending on the petroleum product released, levels of contamination, chemicals present, and media in question (soil, water, and vapor), interferences to analytical detection and quantitation may be present. Procedures such as sample dilution may allow quantification of petroleum COI concentrations but will also result in elevated detection and quantitation limits.

.

### Remediation Standard

A remediation standard is defined in IDAPA 58.01.24 as a “media specific concentration which, when attained, is considered to provide adequate protection of human health and the environment.”

Remediation standards are established at petroleum release sites where corrective action has been determined to be required. The need for corrective action is commonly determined after completion of a risk evaluation. Remediation standards are commonly established as part of a risk evaluation or an approved corrective action plan (CAP).

The magnitude of a remediation standard that is established for a given chemical and the potential that it may be exceeded by a PQL is dependent on a number of factors, including, but not limited to, the following:

- Assumptions regarding current and future site land use, both on and off site

- What petroleum products were released and the relative distribution of chemicals detected

- The magnitude and distribution of contamination (which media are impacted)

- The routes of exposure determined to pose the greatest threats

The two risk evaluation scenarios where elevated PQL values may require the re-evaluation of initially selected remediation standards are as follows:

- Screening Level Evaluation

The screening level evaluation described in IDAPA 58.01.24 employs the simplest and most straightforward use of remediation standards in the form of screening levels. The screening levels are conservative, calculated, risk-based concentrations determined to be protective of unrestricted use exposures.

Because laboratory analytical technology is continually changing, these media-specific chemical concentrations are established without consideration of the ability of standard laboratory methods to detect and quantify these values. The compounds benz(a)anthracene (BAA) and benzo(b)fluoranthene (BBF) in water and ethylene dibromide (EDB) in soil pose the greatest potential challenges with respect to PQL values meeting screening level concentrations.

For BAA and BBF, common practical quantitation limits in water using EPA Method 8270 with SIM slightly exceed the groundwater screening levels while the method detection limits are slightly below the screening values. At petroleum release sites where these chemicals are COIs and groundwater analytical results for these chemicals are reported either as estimated values (between the quantitation and detection limits) or below the detection limit, the screening levels will be assumed to have been achieved. Additional support for this determination is provided if all other polycyclic aromatic hydrocarbon chemicals are also reported at concentrations below screening levels.

The most commonly used analytical method for EDB in soils, EPA Method 8260, has typical quantitation and detection limits that exceed the soil screening level concentration . The soil screening level for EDB is based on protection of groundwater. The groundwater quantitation limit for EDB is below groundwater screening levelF and the soil quantitation limits are low enough to evaluate the other relevant pathways and routes of exposure. At petroleum release sites where EDB is a COI and groundwater is potentially impacted and representative groundwater samples report EDB below the screening level but soil quantitation limits are above the soil screening level, the groundwater results will take precedence.

In the screening level evaluation, the user must compare maximum media-specific concentrations measured during site assessment activities to the screening levels. If the measured concentrations do not exceed the screening levels, the site may be eligible for site closure. If the measured concentrations do exceed screening levels, the user may select screening levels as the remediation standards upon which corrective action is based and develop a CAP. The user may also choose to conduct additional investigation or evaluation and perform a site-specific risk evaluation. Samples may also have results reported as undetected at a concentration that is less than the PQL but higher than the screening level. In this case, a potentially unacceptable risk would be indicated.

- Site-Specific Risk Evaluation

During the site-specific risk evaluation process, media-specific chemical concentration data are collected and used, along with assumptions regarding exposure pathways and receptors, to calculate risk to expected receptors. If the calculated risk is unacceptable, media-specific, risk-based chemical concentrations are established. These risk-based concentrations become the remediation standards for site corrective action. Samples are then taken during and after corrective action to demonstrate the standards have been attained.

PQLs for selected analytes in a given media sample may exceed one or more of the calculated risk-based remediation standards, making the demonstration that the standard has been attained problematic. It is also problematic when samples with results below a PQL are included in the calculation of exposure point concentrations and risk.

Remediation standards are oftentimes the lowest at sites where:

  - screening levels have been selected as the standards,

  - unrestricted use is the desired corrective action land use goal,

  - contamination has migrated off-site in groundwater,

  - on-site vapor intrusion risk is determined to exist, or

  - diesel fuel contamination (and polynuclear aromatic hydrocarbon chemicals) is present in surface soil or in close proximity to surface water.

Accordingly, PQL-related issues often arise at such sites.

Risk Evaluation Manual for Petroleum Releases—August 2012

### Checklist for Proposals to Use PQLs as Remediation Standards

Proposals to DEQ for the use of PQLs as remediation standards at petroleum release sites should include:

- the results of any risk evaluation,

- the remediation standards developed from that process,

- the proposed PQL, and

- the site-specific issues, with supporting information, that justify the use of the proposed PQL(s) as a remediation standard(s).

The supporting information that should be provided in the proposal is listed below along with a discussion of how DEQ will evaluate that information along with potential solutions to various issues, as they may be relevant to the site being evaluated. Given the number and variety of factors which may arise regarding PQLs in relation to remediation standards, both the list and the discussion are not considered all-inclusive.

##### Risk Evaluation

  - Describe the conceptual site model.

  - Identify the level of risk evaluation, the chemicals, media, and pathway or route of exposure that are at issue.

  - State the magnitude of the proposed PQL and MDL vs. the remediation standard.

Screening Level Evaluation

For sites where a screening level evaluation has taken place and the proposal is to use PQLs in place of screening levels as remediation standards for specific COI and media, the information provided will be reviewed in a number of ways. The proposed PQLs will first be reviewed relative to the conceptual site model (CSM) and the pathway/routes of exposure, which are the basis of the screening levels for the chemicals under consideration.

    - If the PQL for a chemical exceeds a soil screening level based on groundwater protection but the PQL achieves the groundwater screening level (also based on groundwater protection) for the same chemical, the soil PQL may be used as the soil remediation standard for that chemical.

    - If the PQL for a chemical exceeds a soil or groundwater screening level for other routes of exposure (groundwater ingestion, vapor inhalation, or direct contact), DEQ will review the magnitude of the difference between the PQL/MDL and the screening level (whether greater than or less than the screening level) for all COI with a common route of exposure to estimate the potential impact on the cumulative risk from that pathway.

If only a screening level evaluation has been done, completion of a site-specific evaluation may revise the remediation standards and still allow unrestricted use of the site, if that is the desired/expected future land use condition.

Site-Specific Evaluation

PQL issues may be related to cleanup criteria from a site-specific risk evaluation. Site-specific evaluations depend on the calculation of cumulative risk to a receptor from all pathways/routes of exposure and chemicals, based on estimated exposure point concentrations. The calculation of exposure point concentrations can be influenced by the number of chemicals with PQL issues, proportion of samples with <PQL values, the magnitude of the PQL, and how these censored data are treated in the calculation procedure. If the calculations result in unacceptable risks, remediation standards for individual COI are calculated, some of which may be low enough to be exceeded by PQLs. DEQ will evaluate whether and to what extent < PQL data were used appropriately in these calculations, if the resultant calculated risks are over or under estimated, the degree to which the < PQL data contribute to overall risk, and whether the calculated cleanup criteria can be modified to avoid the PQL issue or if use of a PQL will not materially affect the protectiveness of the cleanup.

The assumptions of the CSM and risk evaluation will be reviewed to determine if the land use scenarios, receptors, and input parameter values are appropriate. Exclusion of an inappropriate pathway may eliminate the specific PQL concern. Alternatively, the pathways/routes of exposure may be appropriate, but the receptor/land use scenario may not be.

If the PQL for a COI is above a remediation standard but the MDL is at or below the standard, there may be instances where sample results falling between these limits can be considered to have met the remediation standard. If a sample result is reported as an estimated value and that concentration is at or below the remediation standard, the standard will have been met. Sample results reported as undetected (less than the detection limit, provided the limit is at or below the standard) will have met the standard.

Additional guidance on the treatment of PQL/MDL data and issues in site-specific risk evaluation are available in the EPA Guidance for Data Useability in Risk Assessment (Part A), the Risk Assessment Guidance for Superfund,

Volume I, Part A, and the EPA ProUCL statistical software technical guide.

##### Analytical Issues

  - Identify the analytical methods used that are the basis of the PQL

  - Provide all relevant quality assurance/quality control (QA/QC) information from the laboratory sheets for the analyses in question, including data quality assessment and summaries provided by the laboratory.

When reviewing the proposal, DEQ will evaluate whether the data quality assessment and QA/QC accompanying the analytical results adequately explain and support the PQLs proposed. Generally, PQL proposals should be based on PQLs derived from undiluted samples. Elevated PQLs resulting from sample dilution are typically the result of an attempt to quantitate all the COIs when the concentrations of one or more chemicals (some of which may not be COI) are highly elevated, often referred to as matrix interference.

Matrix interference can also occur due to the nature of the media (soils in particular) which can impair the quantitation of a COI. The matrix spike and matrix spike duplicate portions of the laboratory QA procedures can sometimes clarify whether there are matrix interferences and whether they are causing over or underestimation of actual concentrations of a COI.

##### Sampling

  - Provide information on the number of sampling events completed to support the PQL proposal.

  - Provide information on the sampling procedures and QA/QC measures implemented during each of the sampling programs undertaken at the release site. Copies of quality assurance project plans and evaluations of adherence to and departure from those plans for the site in question for each event should be included.

The owner/operator may select alternative sampling procedures, which may provide for a lower PQL. The PQLs for analysis of air and vapor samples can be dramatically affected by the volume of air which is sampled.

DEQ will review the proposal for use of a specific PQL as a remediation standard as part of the overall CAP and approve or deny the request along with specifying any conditions of approval or reasons for denial.

### How to Evaluate Cancer Potency of of Polycyclic Aromatic HydrocarbonsPAHs in Soil

Benzo(a)pyrene can be used as a  surrogate, or index chemical, to  estimate the toxicity of other congener PAHs, see https://www.epa.gov/risk/regional-screening-levels-rsls-frequent-questions#FQ47.  They have similar toxicological properties and can be evaluated as a mixture. In 1993, EPA released an interim guidance for the quantitative risk assessment of polycyclic aromatic hydrocarbons; however, EPA is conducting a hazard assessment of PAH mixtures that will be part of the Integrated Risk Information System (IRIS) database. A common approach to screen PAHs is to use potency equivalency factors (PEFs) to express the carcinogenicity of several PAHs as a single value (i.e., benzo(a)pyrene equivalent).

To calculate a benzo(a) equivalent for screening purposes, follow these steps:

- For each PAH (benzo(a)pyrene, benzo(a)anthracene, benzo(b)fluoranthene, benzo(k)fluoranthene, and chrysene) listed in Table 1, identify the concentration sampled. For PAHs that have a PQL above the screening level, three options are possible:

  - If one or more PAHs analyzed have not been detected in any sample at site then a value of zero may be assigned for the analytes for the calculation of the toxicity equivalent concentration.

  - If the analyte has not been detected in the sample, but has been detected at the site, it is recommended to calculate toxic equivalent concentrations using one-half of the MDL.

  - If the analyte has been detected in the sample and has been detected at the site, it is recommended to calculate the toxic equivalent concentration using the full method MDL.

- Multiply the measured concentration of each PAH by the relative potency factor in the table below to obtain a toxic equivalent concentration for each PAH.

- Sum the congener-specific toxic equivalent concentration to calculate the total toxicity equivalent factor for each sample.

- Compare the maximum total benzo(a)pyrene equivalent across samples to the screening level for benzo(a)pyrene in soil.

Example: TEC Calculation for Determining Compliance for Residential Soil

| cPAH | Measured Soil Concentration | Toxicity Equivalent factor (unitless) | Toxicity Equivalent Concentration mg/kg |
| --- | --- | --- | --- |
| Benzo(a)pyrene | 0.06 | 1.0 | 0.06 |
| Benzo(b)fluoranthene | 0.03 | 0.10 | 0.003 |
| Benzo(k)fluoranthene | 0.15 | 0.10 | 0.015 |
| Benzo(a)anthracene | 0.09 | 0.1 | 0.009 |
| Chrysene | 0.05 | 0.01 | 0.0005 |
| Sum |  |  | 0.0875 |
| Benzo(a)pyrene RSL for residential soil | 0.1 mg/kg |  |  |

The sum of the toxicity equivalent concentrations (0.0875 mg/kg) is lower than the benzo(a)pyrene screening level (0.11 mg/kg). Therefore, no further action is needed. If levels are at or above the screening level, further evaluation would be necessary.

### References

EPA (U.S. Environmental Protection Agency). 1989. Risk Assessment Guidance for Superfund, Volume 1, Human Health Manual (Part A). United States Environmental Protection Agency. EPA/540/1-89/002.

EPA (U.S. Environmental Protection Agency). 1992. Guidance for Data Useability in Risk Assessment (Part A). United States Environmental Protection Agency.

9285.7-09A.

EPA (U.S. Environmental Protection Agency). 1999. Compendium of Methods for the Determination of Toxic Organic Compounds in Ambient Air, Second Edition. United States Environmental Protection Agency. EPA/625/R-96/010b.

EPA (U.S. Environmental Protection Agency). 2001. The Relationship Between SW-846, PBMS, and Innovative Analytical Techniques. United States Environmental Protection Agency. EPA 542-R-01-015.

EPA (U.S. Environmental Protection Agency). 2006. Method 8260C. Volatile Organic Compounds by Gas Chromatography/Mass Spectrometry (GC/MS), Revision 3, August 2006 in Test Methods for Evaluating Solid Waste, Physical/Chemical Methods. United States Environmental Protection Agency. SW-846.

EPA (U.S. Environmental Protection Agency). 2007. Method 8270D. Semivolatile Organic Compounds by Gas Chromatography/Mass Spectrometry (GC/MS). ), Revision 4, February 2007 in Test Methods for Evaluating Solid Waste, Physical/Chemical Methods. United States Environmental Protection Agency. SW-846.

EPA (U.S. Environmental Protection Agency). 2010. ProUCL Version 4.1.00 Technical Guide (Draft), Statistical Software for Environmental Applications for Data Sets with and without Nondetect Observations. United States Environmental Protection Agency. EPA/600/R-07/041.

EPA (U.S. Environmental Protection Agency). 2011. Test Methods for Evaluating Solid Waste, Physical/Chemical Methods. United States Environmental Protection Agency. SW-846. Update IV of the Third Edition. Available at: http://www.epa.gov/epawaste/hazard/testmethods/index.htm

EPA (USA Environmental Protection Agency). 1993. Provisional Guidance for Quantitative Risk Assessment of Polycyclic Aromatic Hydrocarbons. Office of Research and Development. Washington, DC, EPA/600/R-93/089.

EPA (U.S. Environmental Protection Agency). 2010. Development of a Relative Potency Factor (Rpf) Approach for Polycyclic Aromatic Hydrocarbon (PAH) Mixtures (External Review Draft, Suspended). U.S. Environmental Protection Agency, Washington, DC, EPA/635/R-08/012A.

California Office of Environmental Health Hazard Assessment (OEHHA). 2015. Guidance manual for preparation of health risk assessments; Appendix G, PAH potency factors and selection of potency equivalency factors (PEFs) for PAHs based on benzo(a)pyrene potency. Sacramento, CA: Air Toxics Hot Spots Program. Available at: https://oehha.ca.gov/media/downloads/crnr/2015gmappendicesgj.pdf.

This page intentionally left blank for correct double-sided printing.

Project Quality Assurance Officer

Enter Staff Name

DEQ Quality Assurance Director

Mark Clough

Project Manager

Enter Staff Name

Program Manager

Enter Staff Name

Laboratory Contact

Enter Contact Name

Project Technical Lead

Enter Staff Name